| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0078-v2 |
| --- | --- |

| Title: | EE2-2.10: IBC-LIC extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the simulation results of IBC-LIC extension, which is studied in EE2-2.10. In the current design, IBC-LIC derives the linear model from a template that consists of both the top and left adjacent areas. Two new IBC-LIC modes are proposed to allow using only the top or only the left area, respectively. In addition, it is proposed to allow IBC-LIC to have two linear models in one CU, which is similar to MMLM. Two tests are conducted in this EE, one is testing the proposed method, and the other one is testing the removal of larger block-size constraint for IBC-LIC.

On top of ECM-9.0, the coding performance is shown as below:

Test2.10a: the proposed IBC-LIC extension with removal of large block-size constraint

AI: 	{-0.01% (Y), 0.00% (U), -0.02% (V),  100.4% (EncT), 99.5% (DecT)} for Overall

{-0.49% (Y), -0.30% (U), -0.30% (V),  99.7% (EncT), 98.8% (DecT)} for class F

{-0.10% (Y), -0.15% (U), -0.13% (V),  99.6% (EncT), 99.9% (DecT)} for class TGM

RA:   { 0.00% (Y), -0.03% (U), -0.06% (V),  100.3% (EncT), 100.1% (DecT)} for Overall

{-0.34% (Y), -0.52% (U), -0.38% (V),  101.3% (EncT), 100.8% (DecT)} for class F

{-0.08% (Y), -0.06% (U), -0.09% (V),  101.2% (EncT), 100.4% (DecT)} for class TGM

Test2.10b: ECM IBC-LIC with removal of large block-size constraint

AI: 	{-0.02% (Y), -0.02% (U), 0.01% (V),  100.1% (EncT), 99.6% (DecT)} for Overall

{-0.04% (Y), -0.02% (U), 0.21% (V),  100.3% (EncT), 100.3% (DecT)} for class F

{-0.02% (Y), -0.01% (U), -0.01% (V),  100.6% (EncT), 100.4% (DecT)} for class TGM

RA:   { 0.00% (Y), -0.10% (U), 0.03% (V),  100.2% (EncT), 99.7% (DecT)} for Overall

{-0.06% (Y), 0.13% (U), -0.28% (V),  101.2% (EncT), 99.9% (DecT)} for class F

{0.02% (Y), -0.03% (U), -0.06% (V),  100.7% (EncT), 99.8% (DecT)} for class TGM

Test2.10c: the proposed IBC-LIC extension

AI: 	{ 0.01% (Y), 0.01% (U), 0.03% (V),  100.4% (EncT), 99.7% (DecT)} for Overall

{-0.38% (Y), -0.29% (U), 0.28% (V),  100.9% (EncT), 99.8% (DecT)} for class F

{-0.11% (Y), -0.11% (U), -0.11% (V),  101.0% (EncT), 101.3% (DecT)} for class TGM

RA:   { x.xx% (Y), x.xx% (U), x.xx% (V),  100.0% (EncT), 100.0% (DecT)} for Overall

{-0.28% (Y), -0.24% (U), -0.23% (V),  100.9% (EncT), 99.8% (DecT)} for class F

{-0.06% (Y), -0.04% (U), -0.15% (V),  101.0% (EncT), 100.3% (DecT)} for class TGM

Introduction

In the current ECM design, intra block copy with local illumination compensation (IBC-LIC)[1] is a coding tool which compensates the local illumination variation within a picture between the CU coded with IBC and its prediction block with a linear model. The parameters of the linear model are derived by the same method as LIC for inter prediction except that the reference template is generated using a block vector in IBC-LIC. IBC-LIC only applies to the CU whose block size is larger than or equal to 32 and smaller than or equal to 256.

IBC-LIC can be applied to the IBC AMVP mode and the IBC merge mode. For the IBC AMVP mode, an IBC-LIC flag is signalled to indicate the usage of IBC-LIC. For the IBC merge mode, the IBC-LIC flag is inferred from the merge candidate.

Proposed method

In JVET-AD0083[2], it is proposed to add three more modes to IBC-LIC to further improve the coding performance. The first two modes are related to the template shape selection. It is proposed to allow IBC-LIC to use top-only, left-only, or L-shape templates for deriving the single model parameters. The third mode is proposed to extend the MMLM to IBC-LIC, which allows IBC-LIC to have two linear models in one CU. And the template has L-shape in this mode.

In addition, it is proposed to remove the large block-size constraint for IBC-LIC. In EE2-2.10, IBC-LIC can be applied to the CU whose block size is larger than or equal to 32.

In the IBC AMVP mode, the signalling of the proposed method is summarized in Table.1.

Table.1 IBC-LIC signalling in the proposed method

| Value | Mode |
| --- | --- |
| 0 | Not IBC-LIC |
| 100 | Default IBC-LIC |
| 101 | Multi-models IBC-LIC |
| 110 | Top-only IBC-LIC |
| 111 | Left-only IBC-LIC |

Experimental results

The proposed method is implemented on top of ECM-9.0[3] and tested under the common test condition[4]. As discussed in the last meeting, the results of camera captured sequences are also provided for reference.

The proposed IBC-LIC extension with removal of large block-size constraint

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | -0.01% | 100.2% | 99.3% |
| Class A2 | -0.02% | -0.01% | 0.02% | 100.4% | 99.3% |
| Class B | -0.04% | -0.02% | -0.09% | 100.3% | 99.2% |
| Class C | 0.00% | -0.06% | 0.03% | 100.4% | 100.2% |
| Class E | 0.02% | 0.09% | 0.00% | 100.5% | 99.4% |
| Overall | -0.01% | 0.00% | -0.02% | 100.4% | 99.5% |
| Class D | 0.02% | 0.03% | -0.08% | 100.6% | 99.9% |
| Class F | -0.49% | -0.30% | -0.30% | 99.7% | 98.8% |
| Class TGM | -0.10% | -0.15% | -0.13% | 99.6% | 99.9% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | 0.01% | 100.6% | 99.8% |
| Class A2 | -0.01% | -0.07% | -0.12% | 100.3% | 99.9% |
| Class B | 0.02% | 0.12% | -0.14% | 100.0% | 99.8% |
| Class C | 0.00% | -0.16% | 0.02% | 100.3% | 100.6% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.06% | 100.3% | 100.1% |
| Class D | 0.05% | -0.35% | -0.02% | 100.2% | 100.1% |
| Class F | -0.34% | -0.52% | -0.38% | 101.3% | 100.8% |
| Class TGM | -0.08% | -0.06% | -0.09% | 101.2% | 100.4% |

ECM IBC-LIC with removal of large block-size constraint

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.06% | 0.01% | 99.9% | 99.9% |
| Class A2 | -0.02% | -0.01% | -0.01% | 99.9% | 99.3% |
| Class B | -0.05% | -0.05% | 0.02% | 100.3% | 99.8% |
| Class C | 0.00% | -0.05% | -0.06% | 100.1% | 98.7% |
| Class E | -0.01% | 0.06% | 0.08% | 100.3% | 100.7% |
| Overall | -0.02% | -0.02% | 0.01% | 100.1% | 99.6% |
| Class D | 0.00% | -0.02% | -0.06% | 100.4% | 100.2% |
| Class F | -0.04% | -0.02% | 0.21% | 100.3% | 100.3% |
| Class TGM | -0.02% | -0.01% | -0.01% | 100.6% | 100.4% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | -0.02% | 100.6% | 99.4% |
| Class A2 | -0.01% | -0.25% | -0.02% | 100.3% | 99.7% |
| Class B | -0.01% | -0.03% | 0.08% | 99.6% | 99.2% |
| Class C | 0.02% | -0.19% | 0.06% | 100.4% | 100.4% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.10% | 0.03% | 100.2% | 99.7% |
| Class D | 0.02% | -0.03% | -0.07% | 100.0% | 99.7% |
| Class F | -0.06% | 0.13% | -0.28% | 101.2% | 99.9% |
| Class TGM | 0.02% | -0.03% | -0.06% | 100.7% | 99.8% |

The proposed IBC-LIC extension

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.01% | 101.1% | 100.1% |
| Class A2 | 0.02% | 0.07% | 0.05% | 100.3% | 99.0% |
| Class B | 0.01% | -0.01% | 0.07% | 100.2% | 100.0% |
| Class C | -0.01% | -0.01% | -0.01% | 100.3% | 99.7% |
| Class E | 0.01% | 0.04% | 0.03% | 100.4% | 99.4% |
| Overall | 0.01% | 0.01% | 0.03% | 100.4% | 99.7% |
| Class D | 0.01% | 0.04% | 0.03% | 101.3% | 98.4% |
| Class F | -0.38% | -0.29% | -0.28% | 100.9% | 99.8% |
| Class TGM | -0.11% | -0.11% | -0.11% | 101.0% | 101.3% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.00% | -0.13% | -0.07% | 100.4% | 100.5% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.04% | -0.31% | 0.00% | 100.2% | 99.8% |
| Class F | -0.28% | -0.24% | -0.23% | 100.9% | 99.8% |
| Class TGM | -0.06% | -0.04% | -0.15% | 101.0% | 100.3% |

Conclusion

This contribution reports the simulation results of EE2-2.10. It is reported that the proposed method in EE2-2.10 provides attractive coding performance on top of ECM-9.0 for class F and class TGM with negligible complexity change. It is suggested to adopt this method into next version of ECM.

Reference

[1] Y. Wang, K. Zhang, L. Zhang, N. Zhang, C. Ma, X. Xiu, W. Chen, H. Jhu, C. Kuo, N. Yan, X. Wang. EE2-3.6: IBC-CIIP, IBC-GPM, and IBC-LIC. JVET-AC0112, 29th Meeting, by teleconference, 11-20 Jan 2023.

[2] Z. Xie, Y. Yu, H. Yu, D. Wang. Improvements over IBC-LIC. JVET-AD0083, 30th Meeting, Antalya, 21-28 April 2023.

[3] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-9.0.

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AD2017, April. 2023.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).