| Joint Video Experts Team (JVET) of ITU-T SG16 WP3/16 and ISO/IEC JTC 1/SC 29/WG 11 31st Meeting: Geneva, CH, 11–19 July 2023 | Document: JVET-AE0082 |
| --- | --- |

| Title: | AHG12: Non-square quadtree partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjo Ahn Junghak Nam Naeri Park Jaehyun Lim 19 Yangjaedaero-11gil, Soecho-gu, Seoul 06772, South Korea Seunghwan Kim 10680 Treena St, San Diego, CA 92131, USA | Tel: Email: | +82-2-6912-6561 yongjo.ahn@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the current VVC and ECM, quadtree partitioning is applied to only square blocks while binary and ternary tree partitioning are allowed to be applied to both square and non-square blocks. This contribution allows quadtree partitioning is also applied to non-square blocks of which shapes are 2N×N and N×2N. The proposed partitioning method was implemented on top of ECM-9.0 and the summarized test results are shown as follows:

AI:   -0.17%/-0.29%/-0.26% (Y/U/V), 137% (EncT), 100% (DecT)

RA:  -0.67%/-0.92%/-0.82% (Y/U/V), 167% (EncT), 100% (DecT)

LDB: -0.31%/-0.90%/-0.54% (Y/U/V), 144% (EncT), 99% (DecT)

Introduction

In the current VVC and ECM, three different kinds of block tree structures and six block partitions are applied. As shown in Figure 1, three block tree structures are quadtree, binary tree, and ternary tree, and six block partitions include NO_SPLIT, QT_SPLIT, BT_HOR, BT_VER, TT_HOR, and TT_VER.

Figure 1. Block partitioning of the current VVC and ECM

Figure 2. Block partitioning syntaxes of the current VVC and ECM

In order for signalling various shapes of block partitioning, related syntaxes shown in Figure 2 are engaged and signalled. If a coding unit (CU) is split with one of the block tree structures, split_cu_flag is signalled as true. Otherwise, the CU is decided as a leaf node of the block tree structure (NO_SPLIT). When split_cu_flag is signalled as true, split_qt_flag, mtt_split_cu_vertical_flag, and mtt_split_cu_binary_flag can be signalled depending on the specific conditions.

As an example of the specific condition, quadtree partitioning is applied to only square blocks (2N×2N) which are not split with binary or ternary tree while binary and ternary tree partitioning can be applied to both square and non-square blocks. Hence, in order to apply quadtree partitioning for non-square blocks, we need to apply binary tree multiple times as shown in Figure 3. It can be signalling overhead compared with signalling method using a single flag for the quadtree partitioning.

Figure 3. An example of the same cases of non-square quadtree by applying binary tree multiple times

Proposed method

In this contribution, quadtree partitioning for non-square blocks is proposed. Technical details of the proposed method are described in the following subsections:

Block shapes and sizes

The current VVC and ECM generates and deals with various shapes of non-square blocks (e.g., 2N×N, N×2N, 4N×N, N×4N, etc.). The proposed partitioning method additionally allows quadtree partitioning for non-square blocks of which block shapes are 2N×N and N×2N.

Specifically, it is applied for block sizes which are greater than or equal to minimum block size (i.e. 16×8 or 8×16). The minimum block size depends on picture size and configuration. The proposed method doesn’t set the specific maximum block size for the non-square quadtree partitioning. The maximum block sizes can be decided according to the maximum block sizes for binary or ternary tree partitioning. Under the common test conditions, the largest block sizes for the non-square quadtree partitioning are 128×64 or 64×128 for Class A/B and 64×32 or 32×64 for other classes.

Figure 4. Block partitioning syntaxes of VVC and ECM with the proposed non-square quadtree partitions

Syntax signalling

In Figure 4, a basic idea and syntax structure of block partitioning for the proposed method are described. As shown in Figure 4, the proposed method has no additional syntax for non-square quadtree partitioning compared to the current VVC and ECM described in Figure 2. The proposed partitioning method uses split_qt_flag, a conventional syntax for quadtree partitioning for square blocks, to signal whether the non-square quadtree partitioning is applied or not. There is no additional syntax for the proposed partitioning, but, this proposal adds a CABAC context for the cases when the non-square quadtree partition can be applied.

Constraint of the conventional quadtree partitioning for square blocks

In the current VVC and ECM, quadtree partitioning for square blocks cannot be applied to a CU if any of its ancestor CUs is split with binary or ternary tree partitions. The proposed partitioning method also keeps this constraint. Therefore, if any of its ancestor CUs is split with binary, ternary, or non-square quadtree, the CU cannot split with the quadtree for square blocks.

Examples of multi-type tree (MTT) and MTT with the proposed non-square quadtree partition are shown in Figure 5. In Figure 5, the black lines boundaries of coding tree units (CTUs), the white lines mean square quadtree split, the green lines are binary tree split, the blue lines are ternary tree split, and the red lines are non-square quadtree split. Figure 5 (a) represents an example of MTT with the current VVC and ECM and Figure 5 (b) represents an example of MTT with the proposed non-square quadtree partitioning. As shown in Figure 5 (b), non-square quadtree partitioning is working at the boundaries of moving object and complex regions.

(a) An example of MTT in the current VVC and ECM

(b) An example of MTT with the proposed non-square quadtree partition

Figure 5. Examples of block partitions

Experimental results

The proposed partitioning method has been implemented in ECM-9.0 [1] and evaluated under all intra (AI), random access (RA), and low-delay B (LDB) common test configurations [2].

As shown in the below tables, the proposed partitioning method provides 0.17%, 0.67%, and 0.31% BD-rate savings over ECM-9.0 under the AI, RA, and LDB configurations, respectively. Encoding time increments are 137%, 167%, and 144% and no meaningful decoding time is observed.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.09% | -0.13% | 120% | 99% |
| Class A2 | -0.08% | -0.29% | -0.32% | 129% | 99% |
| Class B | -0.16% | -0.27% | -0.19% | 138% | 99% |
| Class C | -0.26% | -0.39% | -0.41% | 152% | 101% |
| Class E | -0.29% | -0.37% | -0.25% | 139% | 101% |
| Overall | -0.17% | -0.29% | -0.26% | 137% | 100% |
| Class D | -0.23% | -0.41% | -0.35% | 158% | 101% |
| Class F | -0.53% | -0.67% | -0.76% | 144% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.69% | -0.97% | -1.01% | 162% | 100% |
| Class A2 | -0.85% | -1.06% | -0.97% | 166% | 100% |
| Class B | -0.67% | -0.87% | -0.88% | 172% | 100% |
| Class C | -0.52% | -0.83% | -0.49% | 166% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.67% | -0.92% | -0.82% | 167% | 100% |
| Class D | -0.52% | -1.15% | -0.60% | 166% | 102% |
| Class F | -0.83% | -0.97% | -1.09% | 144% | 98% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.29% | -0.59% | -0.29% | 143% | 100% |
| Class C | -0.37% | -1.08% | -0.59% | 150% | 99% |
| Class E | -0.24% | -1.19% | -0.91% | 137% | 99% |
| Overall | -0.31% | -0.90% | -0.54% | 144% | 99% |
| Class D | -0.44% | 0.48% | -1.35% | 145% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusion

This contribution proposes to allow quadtree partitioning to be applied for non-square blocks. The proposed partitioning method can reduce the syntax overhead for the same cases by applying binary tree multiple times and can provide the capability of faster localization for small objects. It is recommended to include the proposed partitioning method in the next round of EE2.

Reference(s)

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-9.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AD2017, 30th Meeting, Antalya, TR, 21–28 April 2023.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).