| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0086-v1 |
| --- | --- |

| Title: | EE2-4.2: Large NSPT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Jie Zhao Seung-Hwan Kim 10680 Treena St. St# 550 San Diego, CA 92131, USA | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com jaehyun.lim@lge.com jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In this contribution, test results of EE2 Test 4.2 (Large NSPT) are reported. In this test, the large NSPT (Non-Separable Primary Transform) is applied to 4x32/32x4 and 8x32/32x8 blocks instead of LFNST, of which kernel matrix dimensions are 20x128 and 24x256, respectively. The kernel mapping from intra prediction mode to kernel set and the number of kernels per set for the large NSPT are the same as those in ECM-9.0. The existing NSPT kernels for other block sizes are untouched, and the same encoding algorithm as in ECM-9.0 are utilized for the large NSPT. The BD-rate and complexity changes relative to ECM-9.0 for AI configuration are -0.13%/-0.12%/-0.07% for Y/U/V with 101% EncT and 100% DecT, and those for RA are -0.06%/-0.10%/-0.07% for Y/U/V with 101% EncT and 100% DecT.

Proposed architecture

In the 30th JVET meeting, large NSPT was proposed in [1] and decided to be tested through EE2 (Test 4.2). The large NSPT is targeted for 4x32/32x4 and 8x32/32x8 blocks, of which kernel matrix dimensions are 20x128 and 24x256. Therefore, 20 and 24 transform coefficients are generated by applying the two types of kernel matrices, respectively, which are placed from DC position following scan order. The remaining 108 and 232 positions in each transform block are zeroed-out, respectively. When non-zero coefficient(s) are found during residual parsing, an NSPT index is not parsed. Basically, the way of applying large NSPT is the same as in other block sizes, i.e. 4x4, 4x8/8x4, 4x16/16x4, 8x8, and 8x16/16x8, of which NSPT kernels are untouched. Moreover, any specific encoding algorithm for the large NSPT are not employed.

Experimental results

The proposed architectures have been implemented based on ECM-9.0 reference software according to common test conditions defined in JVET-AD2017 [2].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.14% | -0.14% | 102% | 101% |
| Class A2 | -0.14% | -0.19% | -0.10% | 102% | 100% |
| Class B | -0.10% | -0.14% | -0.13% | 101% | 100% |
| Class C | -0.16% | -0.08% | -0.01% | 101% | 100% |
| Class E | -0.12% | -0.08% | 0.04% | 101% | 100% |
| Overall | -0.13% | -0.12% | -0.07% | 101% | 100% |
| Class D | -0.01% | -0.08% | -0.03% | 101% | 100% |
| Class F | -0.10% | -0.03% | 0.21% | 101% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.20% | -0.09% | 101% | 100% |
| Class A2 | -0.10% | -0.19% | -0.14% | 101% | 101% |
| Class B | -0.06% | 0.08% | 0.03% | 101% | 100% |
| Class C | -0.05% | -0.19% | -0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.10% | -0.07% | 101% | 100% |
| Class D | -0.04% | -0.15% | -0.15% | 100% | 100% |
| Class F | -0.09% | -0.12% | -0.02% | 100% | 100% |

Conclusion

The EE2 test (Test 4.2) results for the large NSPT kernels for Nx32/32xN (N = 4 and 8) are presented in this contribution, which are applied in the same manner as the existing NSPT in ECM-9.0. The benefit of these large NSPT kernels is observed by 0.13% and 0.06% coding gains for AI and RA configurations, respectively. It is recommended to incorporate the large NSPT for the Nx32/32xN blocks into next ECM.

References

[1] M. Koo, J. Zhao, J. Lim, S. Kim (LGE), “Non-EE2: On large NSPT”, JVET-AD0187, April 2023.

[2] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, April. 2023.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).