| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0092-v1 |
| --- | --- |

| Title: | [AHG4] Occupancy-only PSNR calculations for V3C V-PCC coding evaluation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sebastian Schwarz, Miska M. Hannuksela Nokia Technologies Munich, Germany | Email: | {sebastian.schwarz, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution discusses occupancy-only PSNR calculations to simplify V3C coding efficiency evaluation. Objective evaluations resulted in correlation coefficients of 0.9 and higher between the proposed evaluation metric and established end-to-end metrics. Therefore, it is proposed to recommend the use of occupancy-only PSNR in V3C V-PCC coding evaluations provided to JVET.

# Introduction

JVET-AD0044 [1] presented V3C content coding evaluation results for ECM-8.0 over VTM-19.0, showing an average improvement of -12.81% and -8.16% in Luma-BDBR for RA and AI respectively. As was pointed out during the discussion, these results were derived on the coded videos and were not an end-to-end evaluation of the full V3C encoding chain.

As discussed in JVET-AC0041 [2], the full V3C encoding chain can be rather cumbersome. Therefore, we investigated simplified evaluation approaches based on the V3C occupancy signal and report our finding in this contribution.

# Occupancy-only PSNR for V3C V-PCC encoding evaluation

In the V-PCC standard, a dynamic 3D point cloud model is segmented into coherent groups of 3D points, so called “patches” based on point surface normals in relation to a minimum of six projection planes. These patches are then packed into 2D video frames. Typically, there are three types of videos associated with a single point cloud sequence: the geometry video, the occupancy video, and at least one attribute video. An example of this process is shown in Fig. 1 below.

Figure 2: Example of the V-PCC patch generation.

The occupancy video is typically compressed using lossless video coding settings; thus, we investigated to calculate distortions for geometry and texture videos for occupied pixels only, to determine the reconstruction quality. To determine if such occupancy-only metrics correlate well with the established V-PCC metrics [3] and ran tests for the test content and test points presented in [1]. Tab. 1 below summarises our findings, showing high Pearson correlation coefficients (PCC) and Spearman’s rank correlation coefficients (SPCC) between the proposed metrics and their established V-PCC counterparts.

Table 1: Occupancy-only PSNR correlations to V-PCC metrics (against original point clouds)

|  | D1-PSNR (point-point) |  | D2-PSNR (point-plane) |  | D1 Y-PSNR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PCC | SPCC | PCC | SPCC | PCC | SPCC |
| Geometry Occ. Y-PSNR | 0,88 | 0,91 | 0,90 | 0,91 |  |  |
| Texture Occ. Y-PSNR |  |  |  |  | 0,94 | 0,95 |

Pearson correlation coefficient (PCC) and Spearman’s rank correlation coefficient (SPCC) are statistical measures of linear correlation between two sets of data. The Pearson correlation coefficient is a normalised measure of the covariance. The Spearman’s rank correlation coefficient is equivalent to the Pearson correlation but calculated over ranked data. Pearson's correlation assesses linear relationships, Spearman's correlation assesses monotonic relationships (whether linear or not) [4].

Correlation is a statistical measure that tells us about the association between the two variables. It describes how one variable behaves if there is some change in the other variable. Thus, the high correlation coefficients shown in Tab. 1 point at strong predictability of established V-PCC quality metrics from the proposed occupancy-only PSNR calculations.

Table 2: Occupancy-only PSNR correlations to V-PCC metrics (against uncompressed V3C reconstruction)

|  | D1-PSNR (point-point) |  | D1 Y-PSNR |  |
| --- | --- | --- | --- | --- |
|  | PCC | SPCC | PCC | SPCC |
| Geometry Occ. Y-PSNR | 0,89 | 0,93 |  |  |
| Texture Occ. Y-PSNR |  |  | 0,98 | 0,98 |

In a second study, D1-PSNR and D1 Y-PSNR were calculated between point clouds reconstructed from uncompressed V3C video and reconstructed point clouds from compressed V3C video. The result for this correlation analysis is presented in Tab. 2.

The results of these experiments show a good correlation between the established V3C V-PCC end-to-end metrics and the investigated occupancy-only PSNR metrics.

## Details on V-PCC occupancy-only PSNR calculations

The results of Tab. 1 and 2 were obtained as follows.

Geometry Occupancy Y-PSNR: MSE calculated for all occupied pixels (occupancy map value at pixel position x, y = 1), between uncompressed geometry video luma channel and compressed geometry video luma channel. PSNR calculated with a maximum value of 255 (8 Bit video).

Texture Occupancy Y-PSNR: MSE calculated for all occupied pixels (occupancy map value at pixel position x, y = 1), between uncompressed texture video luma channel and compressed texture video luma channel. PSNR calculated with a maximum value of 255 (8 Bit video).

D1-PSNR, D2-PSNR, and D1 Y-PSNR: Distortion between original and reconstructed point clouds following [3].

All the above metrics were derived for the test content and test points presented in [1].

Pearson and Spearman’s rank correlation coefficients were calculated per-frame, over a total of 2570 correlation points (1 sequence with 64 frames, 1 sequence with 150 frames, 2 sequences with 300 frames, 5 test points each).

# Conclusion

In this contribution, we introduced occupancy-only PSNR for V3C encoding evaluation. Objective evaluation results show a correlation to the established V-PCC metrics with correlation coefficients of 0.9 and above. Therefore, it is proposed to recommend the use of occupancy-only PSNR in V3C V-PCC coding evaluations provided to JVET.

Furthermore, we showed that the findings of the encoding performance evaluation presented at the last meeting are still valid if evaluated in V3C end-to-end conditions, i.e., ECM performance over VTM for V3C content is slightly worse than for other content categories.

The sequences used for this evaluation are available at [5]. A Python implementation for occupancy-only PSNR calculations is provided alongside this contribution. Full experiment results are also attached to this contribution.

# References

[1] JVET-AD0044, Visual volumetric video-based coding (V3C) test content

[2] JVET-AC0041, On visual volumetric video-based coding (V3C) testing conditions,
https://jvet-experts.org/doc_end_user/current_document.php?id=12226

[3] Common testing conditions for V-PCC, https://dms.mpeg.expert/doc_end_user/current_document.php?id=76918&id_meeting=184

[4] Comparison of Pearson and Spearman correlation coefficients [Link]

[5] https://mpegfs.int-evry.fr/mpegcontent/ws-mpegcontent/Explorations/V3C

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).