| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0093-v1 |
| --- | --- |

| Title: | AHG11: Content-adaptive neural network loop-filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruiying Yang Maria Santamaria Francesco Cricri Honglei Zhang Jani Lainema Ramin G. Youvalari Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | {ruiying.yang, maria.santamaria_gomez, francesco.cricri, honglei.1.zhang, jani.lainema, ramin.ghaznavi-youvalari, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution presents a content-adaptive neural network loop-filter. The content adaptation is achieved by means of overfitting a base loop-filter on the test video. The base loop-filter was described in JVET-AD0156 and it is the default (low-complexity) neural network loop-filter in NNVC 5.0. The generated weight-updates are coded with the MPEG-NNR standard and the bitrate overhead is considered into BD-rate calculations. The inference was done using SADL library with int16 precision and evaluated against NNVC 5.0 in RA configuration, with neural network intra prediction and low-complexity neural network loop-filter enabled. It is reported that, for class C, the coding gains are -1.06% (Y), -4.80% (Cb) and -3.17% (Cr) and for class D, the coding gains are -0.80% (Y), -7.86% (Cb) and -7.23% (Cr), with negligible impact to the decoding times. Thus, it is reported that the proposed content-adaptive loop-filter outperforms NNVC 5.0. It is proposed to include the method into the next EE1 round for a training and inference cross-check, and to eventually adopt it into the next version of NNVC anchor.

# Introduction

Performing content adaptation has been previously shown to be effective for neural network post-processing filters. In fact, in JVET-AA0066 [1], overfitting a low-complexity post-processing filter [2] for each input video sequence increased the overall BD-rate gains by -2.36% (Y), -7.77% (Cb), -4.66% (Cr) in RA configuration over using pretrained (base) models. This contribution studies the feasibility of applying content-adaptation to a neural network loop-filter.

The proposed content-adaptation is performed by overfitting multipliers applied after each convolutional block, shown as the following equation:

σW*x+b∙m,

where W is the kernel, * is the convolution operator, x is the input, b is the bias, m is the multiplier and σ is the activation function.

The network architecture is based on JVET-AD0156 [3] (Figure 1) and the difference is that the multiplier layers are added after each convolutional block.

3x3 conv 3x3x10xM

Leaky ReLu

3x3 conv 3x3xKxL

1x1 conv 1x1xMxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

BS info

Qstep

Yx4+U+V 72x72x10

NxN

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

Y’x4+U’+V‘ 64x64x6

N’xN’

n Hidden Layers

1x1 conv 1x1xKxR

3x1 Sep conv 3x1xRxR

3x1xRxR

1x1 conv 1x1xRxK

1x3 Sep conv 1x3xRxR

CP decomposition

Layers approximated with CP decomposition

Separable Convolution layers

1x1 conv 1x1xMxR

1x1 conv 1x1xRxM

Figure 1. NN architecture from JVET-AD0156 [3] .

The input to the NN is a CTU, 8 neighbouring samples at each of its sides and the frame-level QP step. The luma samples are interleaved into four blocks of size 72x72. The output is the filtered CTU with size 64x64x6, where the luma samples are interleaved as the input luma samples were.

The overfitting is done for each {test sequence, QP} pair by using, as base model, one of the pretrained models of the default low-complexity filter in NNVC 5.0. Afterwards, the resulting weight-update is coded with NNR standard [4, 5] and the overhead from weight-update bitstream is considered into calculation of overall bitrates.

In the main results, the inference was done by replacing one low-complexity filter model with the corresponding overfitted model and evaluated against NNVC 5.0 on the RA configuration.

# Overfitting

One of the models of the default low-complexity filter in NNVC 5.0 is selected to be overfitted for each {test sequence, QP} pair. The model selection is based on the highest PSNR gain on the whole sequence. The input data contains reconstruction, boundary strength and frame-level QP with size 72x72x10 and output is the filtered CTU with size 64x64x6, including luma with 4 channels and chroma with 2 channels. In training, only multipliers are overfitted and the resulting weight update is coded using the MPEG-NNR standard [4, 5].

Table 1 shows the NN information during the overfitting stage.

Table 1 Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Overfitting Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU NVIDIA A100-SXM-80GB |
|  | Framework: | TensorFlow 2.8.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 200 epochs for class C&D, 100 epochs for class A,B,F |
|  | Batch size: | 64 |
|  | Loss function: | Weighted-MSE |
|  | Training time (for 1 model): | A1 & A2: 11 hours B: 7 hours C: 2.5 hours D: 1 hour F: 4 hours |
|  | Training data information: | JVET CTC RA mandatory sequences |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 72x72 |
|  | Learning rate: | 1e-3 |
|  | Learning rate update strategy | Constant learning rate |
|  | Optimizer: | ADAM |
|  | Preprocessing: | Convert 144x144 YUV420 signal to 6 72x72 blocks. Normalize to 0~1 |
|  | Mini-batch selection process: |  |
|  | Training data update strategy: |  |
|  | Other information: |  |

# Inference

At inference, SADL [6] was used with int16 precision, which means that model’s parameters and intermediate activations are quantized to int16. The approach of quantization was presented in JVET-AD0156 [3] by means of normalization of weights and bias by scaling factors to reduce the weights with large values. After this process, the weights are quantized using naïve approach and the quantizers of bias, multipliers, input, and output are 11, 13, 11, 13 respectively. More information during inference stage is shown in Table 2.

Table 2 Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task |  |
|  | Number of Parameters (Each Model) | 53724 |
|  | Total Number of Parameters (All Models) | 107448 |
|  | Parameter Precision (Bits) | Int16 |
|  | Memory Parameter (MB) | 0.21 |
|  | Multiply Accumulate (kMAC/pixel) | 16.5 |
|  | Calculation Method | on a block basis |
| Optional |  |  |
|  | Total Conv. Layers | 46 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 72x72 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

# Experiment results

Table 3 shows the main results of our proposals, where the inferences are performed on the JVET-NNVC common test conditions in RA configurations. The anchor is NNVC 5.0, where both the default neural network low-complexity filter and the neural network intra filter are enabled. In the test, the neural network intra filter and the (overfitted) low-complexity loop filter are enabled. The overfitted model was obtained based on data coded by the NNVC 5.0 anchor. Table 3 shows that coding performance is improved by the overfitted models.

Table 3 BD-rate of using 4 models (3 base models + 1 overfitted model) in inference. Anchor: NNVC 5.0

|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class C | -1.06% | -4.80% | -3.17% | -0.22% | -5.02% | -3.56% | 99% | 106% |
| Class D | -0.80% | -7.86% | -7.23% | 0.71% | -7.66% | -7.59% | 98% | 107% |

For the experimental results showed in Table 3, one of the base models is replaced with an overfitted one. However, compared to base model, decreased performance for some blocks by overfitted filter was noticed during overfitting. Using 5 models (4 base models + 1 overfitted model) in the inference is studied and the corresponding results are showed in Table 4.

Table 4 BD-rate of using 5 models (4 base models + 1 overfitted model) in inference. Anchor: NNVC 5.0

|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class C | -1.12% | -5.14% | -3.30% | -0.29% | -5.11% | -3.59% | 102% | 106% |
| Class D | -0.84% | -8.08% | -7.35% | 0.68% | -7.76% | -7.61% | 102% | 106% |

Additionally, in order to remove the influence of the NN intra prediction tool on the overfitting approach, Table 5 presents results (classes C and D) for the case where the NN intra prediction tool is disabled during inference in both the anchor and the test. The overfitted model was obtained based on data coded with NN intra prediction tool disabled.

Table 5 BD-rate of disabling intra filters in inference. Anchor: NNVC 5.0 without intra filters

|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class C | -1.07% | -4.80% | -3.27% | -0.22% | -4.19% | -2.92% | 99% | 107% |
| Class D | -0.83% | -7.57% | -7.42% | 0.48% | -7.06% | -7.89% | 98% | 107% |

# Recommendation

It is proposed to include the method into the next EE1 round for a training and inference cross-check, and to eventually adopt it into the next version of NNVC anchor.

# Reference

- M. Santamaria, R. Yang, F. Cricri, H. Zhang, J. Lainema, R. G. Youvalari, M. M. Hannuksela. EE1-1.7: Content-adaptive post-filter based on SADL inference. JVET-AA0066. July 2022.

- H. Wang, J. Chen, K. Reuze, A. M. Kotra and M. Karczewicz. EE1-related: Neural-Network-based in-loop filter with constrained computational complexity. JVET-W0131. July 2021.

- J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya, T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy. EE1-1.1: Complexity Reduction on Neural-Network Loop Filter. JVET-AD0156. April 2023.

- ISO/IEC FDIS 15938-17. Information technology – Multimedia content description interface – Part 17: Compression of neural networks for multimedia content description and analysis.

- Test Model of Incremental Compression of Neural Networks for Multimedia Content Description and Analysis (INCTM), N0179. February 2022.

- F. Galpin, T. Dumas, P. Bordes, P. Nikitin, F. Le Leannec, E. Francois. AHG11: Small ad-hoc deep-learning library. JVET-W0181. July 2021.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).