| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0094-v2 |
| --- | --- |

| Title: | EE2-2.6: IBC with non-adjacent spatial candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Changyue Ma, Xiaoyu Xiu, Wei Chen, Hong-Jheng Jhu, Che-Wei Kuo, Ning Yan, Xianglin Wang Yang Wang, Kai Zhang, Li Zhang | Email: | machangyue@kwai.com wangyang.cs@bytedance.com |
| Source: | Kwai Inc., Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2 Test 2.6c on IBC with non-adjacent spatial candidates in JVET-AD0216 and JVET-AD0231. On top of ECM-9.0, simulation results of TEST 2.6c are reported as below:

Results on SCC content compared with ECM-9.0:

AI:

Class F : {-0.45%, -0.22%, -0.26%, 100.2%, 102.1%}, Class TGM : {-0.53%, -0.57 %, -0.52%, 100.1%, 102.7%};

RA:

Class F : {-0.25%, -0.23 %, -0.27%, 100.2%, 100.3%}, Class TGM : {-0.40%, -0.43%, -0.41%, 100.0%, 100.3%};

Results on natural content compared with ECM-9.0 :

AI :{ 0.00%, 0.00%, 0.02%, 100.2%, 100.1%}, RA :{ -0.01%, -0.13%, 0.02%, 100.3%, 100.2%}.

Introduction

IBC with non-adjacent spatial candidates was proposed in JVET-AD0216 [1] and JVET-AD0231 [2], and established as Test 2.6c in JVET-AD EE2 [3].

EE2-2.6c

In EE2-2.6c, it is proposed to insert several new candidates, which are obtained from the BVs of non-adjacent spatial neighboring blocks, into the candidate lists of IBC merge modes and IBC AMVP. As shown in Fig. 1, similar grid pattern as that being used for the non-adjacent spatial candidates of regular inter is used to obtain non-adjacent BVs. The obtained BVs are inserted between the adjacent spatial candidates and the HBVP candidates for both IBC merge and IBC AMVP. Additionally, the restriction that spatial candidates are not used to construct IBC merge candidate list for a 44 CU is removed.

Figure 1. Non-adjacent spatial candidates for IBC mode.

Simulation results

Test 2.6c was implemented on top of ECM-9.0 [4] and tests were performed following ECM CTC [5]. The simulation results of Test 2.6c are summarized in the following table.

Table 1: Performance of EE2-2.6c*

|  | AI |  |  |  |  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | 0.02% | 100.0% | 100.1% | 0.00% | -0.06% | -0.01% | 100.1% | 100.1% |
| Class A2 | -0.01% | -0.11% | -0.01% | 100.0% | 99.8% | -0.03% | -0.18% | -0.05% | 100.0% | 99.4% |
| Class B | 0.00% | 0.06% | 0.06% | 100.0% | 100.0% | -0.01% | -0.08% | 0.07% | 100.4% | 100.4% |
| Class C | -0.01% | -0.05% | 0.00% | 100.4% | 100.6% | 0.00% | -0.19% | 0.04% | 100.6% | 100.4% |
| Class E | -0.02% | 0.05% | 0.01% | 100.4% | 100.2% |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.02% | 100.2% | 100.1% | -0.01% | -0.13% | 0.02% | 100.3% | 100.2% |
| Class D | 0.03% | -0.18% | -0.11% | 100.8% | 100.8% | 0.03% | -0.14% | 0.07% | 100.8% | 100.2% |
| Class F | -0.45% | -0.22% | -0.26% | 100.2% | 102.1% | -0.25% | -0.23% | -0.27% | 100.2% | 100.3% |
| Class TGM | -0.53% | -0.57% | -0.52% | 100.1% | 102.7% | -0.40% | -0.43% | -0.41% | 100.0% | 100.3% |

*: RA: QP22, DaylightRoad2; QP22, QP27, ParkRunning3; In total 3 points in test data copy from anchor.

For complementary results, Table 2 and Table 3 show the simulation results of Test 2.6a and Test 2.6b which were withdrawn due to its largely overlapped coding gain with Test 2.6c.

Table 2: Performance of EE2-2.6a

|  | AI |  |  |  |  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | 0.01% | 100.0% | 100.0% | 0.00% | -0.06% | 0.05% | 99.8% | 99.3% |
| Class A2 | -0.01% | -0.05% | -0.01% | 100.3% | 99.4% | -0.04% | -0.06% | -0.09% | 100.1% | 99.6% |
| Class B | 0.00% | 0.03% | 0.06% | 100.2% | 100.3% | 0.01% | -0.03% | 0.04% | 100.1% | 99.8% |
| Class C | -0.01% | -0.02% | -0.06% | 100.1% | 100.2% | 0.02% | -0.09% | 0.17% | 100.0% | 99.9% |
| Class E | -0.02% | 0.00% | 0.06% | 100.0% | 100.1% |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.01% | 100.1% | 100.0% | 0.00% | -0.06% | 0.05% | 100.0% | 99.7% |
| Class D | 0.03% | -0.03% | -0.23% | 100.4% | 100.9% | 0.00% | -0.18% | 0.00% | 100.2% | 99.8% |
| Class F | -0.31% | -0.42% | -0.39% | 100.1% | 101.5% | -0.23% | -0.34% | -0.10% | 100.1% | 100.2% |
| Class TGM | -0.45% | -0.47% | -0.44% | 100.0% | 102.3% | -0.33% | -0.29% | -0.40% | 100.0% | 100.2% |

Table 3: Performance of EE2-2.6b

|  | AI |  |  |  |  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | -0.03% | 99.5% | 99.2% | -0.01% | -0.01% | 0.03% | 99.8% | 100.5% |
| Class A2 | -0.01% | -0.07% | 0.01% | 100.5% | 99.1% | -0.02% | -0.17% | 0.00% | 99.9% | 100.3% |
| Class B | 0.00% | 0.06% | -0.03% | 100.2% | 100.1% | 0.01% | -0.03% | -0.05% | 99.9% | 100.8% |
| Class C | 0.00% | -0.06% | 0.00% | 100.9% | 100.2% | 0.01% | -0.12% | 0.04% | 100.0% | 100.1% |
| Class E | -0.02% | 0.13% | 0.05% | 100.4% | 99.3% |  |  |  |  |  |
| Overall | -0.01% | 0.01% | -0.01% | 100.3% | 99.7% | 0.00% | -0.08% | 0.00% | 99.9% | 100.5% |
| Class D | 0.02% | -0.17% | -0.08% | 101.3% | 100.8% | 0.02% | -0.11% | 0.25% | 99.9% | 99.1% |
| Class F | -0.33% | -0.31% | -0.20% | 100.1% | 101.6% | -0.21% | -0.06% | -0.33% | 99.9% | 100.2% |
| Class TGM | -0.44% | -0.49% | -0.42% | 99.8% | 102.2% | -0.28% | -0.30% | -0.28% | 99.8% | 100.1% |

Conclusion

In this contribution, the results of EE2 Test 2.6c on IBC with non-adjacent spatial candidates are reported. It is recommended to adopt Test 2.6c into ECM.

References

- C. Ma, X. Xiu, W. Chen, H.-H. Jhu, C.-W. Kuo, N. Yan, X.Wang, “Non-EE2: IBC with non-adjacent spatial candidates,” JVET-AD0216, April 2023.

- Y. Wang, K. Zhang, L. Zhang, “Non-EE2: Non-adjacent spatial candidates for IBC,” JVET-AD0231, April 2023.

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Strom, F. Wang, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AD2024, April 2023.

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, April 2023.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).