| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0095-v1 |
| --- | --- |

| Title: | AHG8/AHG9: proposed changes to the candidate new object mask information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe de Lagrange, Edouard François, Didier Doyen Jie Chen, Shurun Wang, Yan Ye | Tel: Email: | philippe.delagrange@interdigital.com jiechen.cj@alibaba-inc.com |
| Source: | InterDigital, Alibaba Group |  |  |

_____________________________

# Abstract

This contribution proposes an update to the candidate new object mask information (OMI) SEI message proposed in contribution JVET-AD0175. The OMI SEI message is used in conjunction with auxiliary pictures that code object masks. Objects can be detected from the auxiliary pictures thanks to their ID, signaled in the SEI message. One potential issue is when lossy coding is used for those pictures, in which case the sample values inside the mask auxiliary pictures are not guaranteed to perfectly match the object IDs. The proposal aims at addressing this issue, by adding in the SEI a parameter specifying a tolerance range around the object ID values. In addition, bounding box parameters that delimit the masks’ location are also proposed to be included.

# Motivation of OMI SEI message

The Annotated region (AR) SEI message adopted in VSEI, HEVC and AVC is used for the object detection and tracking applications, which are a common workload for end-to-end video systems. With the AR SEI message, the object detection and tracking task only needs to be performed in encoder-side and the results are sent to the decoder through the SEI message so that the decoder doesn’t need to perform the video analysis again. It has many advantages compared with performing those video tasks in decoder-side. First, the power consumption of the decoder can be reduced, and second the object detecting and tracking results will be more accurate as usually the encoder is more computationally powerful than the decoder, and the encoder can perform the video analysis on the original video instead of the compressed one.

However, AR SEI message uses a bounding box to describe the region covered by an object of interest, and the detailed shape information of the object is lost. In some applications, information about the object shape is essential. For example, in the intelligent traffic system, the shape information of people and vehicles is used to recognize the traffic accident. And in the video conferencing system, the shape information could help to switch the virtual background. To have detailed shape information signaled in the bitstream, mask information was proposed for AR SEI message in JCTVC-AF0024 [1] in July 2018. However, JCTVC-AF0024 proposes to send the mask itself (which is a binary picture) inside the SEI. It was commented at the JCTVC-AF meeting that “this is basically sending coded bi-level pictures inside an SEI message and that this seems like it is not what we would ordinarily do with SEI messages. It could also require a substantial payload.” And some experts suggest using auxiliary picture as the mask picture. As more video is being coded for machine analysis purposes, it is appropriate to revisit carrying mask information as part of the video bitstream because this allows important information obtained from machine tasks such as instance segmentation performed at the encoder to be sent to the decoder. Thus, it was proposed to add a new auxiliary picture type for object mask and the associated SEI messages to VSEI in JVET-AD0175.

# A brief introduction to JVET-AD0175

JVET-AD0175 [2] proposed to add a new auxiliary picture type for object mask. That is, the object mask is coded as an auxiliary picture and transmitted in the bitstream together with the associated primary picture.

Usually, the mask is a binary matrix wherein “0” represents background and “1” represents the foreground. But for the auxiliary picture with luma sample bit-depth equal to BitDepthY, there are 1<<BitDpethY different values for each sample. To distinguish different masks within one picture, JVET-AD0175 uses the sample value of auxiliary picture as the ID of the mask. That is to say, in the auxiliary picture, the samples with the same value form a mask. And the regions with different sample values represent different masks. For example, as shown in Figure 1, it is a 16x8 auxiliary picture, and numbers in the figure denote the sample values. So, there are four different masks in the auxiliary. The samples with value 5 (the blue region) form a mask with ID equal to 5; the samples with value 10 (the green region) form a mask with ID equal to 10; the samples with value 20 (the yellow region) form a mask with ID equal to 20; and the samples with value 0 (the while region) form a mask with ID equal 0 (the special mask could be labelled as “background”.

Figure 1. An example of masks representation in auxiliary picture.

As the mask can be overlapped, multiple object mask auxiliary pictures (one auxiliary picture in one layer) can used for one primary picture to handle overlapping case. In that case, the samples with same position but in the different mask picture could belong to different masks overlapped with each other. For example, as shown in Figure 2, there are two 4x4 object mask auxiliary pictures. In the picture 0, the top-left 2x2 block is a mask with ID 5, and in the picture 1, the center 2x2 block is a mask with ID 10. When a decoder receives these two auxiliary pictures, it is clear that there are two masks being overlapped at position (1,1).

Figure 2. An example of overlapped mask case.

To support object mask auxiliary picture, a new type SEI message named object mask information (OMI) SEI message was also proposed in JVET-AD0175. The proposed OMI SEI message uses persistence mechanism and only needs to be re-sent when there are any changes of the object masks. In the case of sending the OMI message, only the updated part of mask information needs to be sent.

# Problem statement

As the exact sample value is used as the object mask ID in JVET-AD0175, when the auxiliary pictures are coded with lossy coding, the distorted sample value will result in a wrong mask ID, which could be a serious problem in some applications.

# Proposal

To address the above-mentioned issue in lossy coding and help consolidating the process of object mask identification from the auxiliary pictures and OMI SEI parameters, two main additions are proposed to the OMI SEI message syntax.

Firstly, a new parameter is added per object ID or per auxiliary picture to indicate a tolerance range around the object ID value, for which the decoded mask sample values are expected to be included.

Secondly, it is suggested to insert parameters related to the bounding box delimiting the object mask borders.

# Proposed syntax and semantics changes

## Change 1: add a tolerance parameter around the object ID value

In the first proposed syntax change, on top of the syntax proposed in JVET-AD0175, a new parameter omi_mask_id_delta[ i ][ j ][ k ] is signalled to indicate a tolerance level around the value of the object ID associated with the ID parameter omi_mask_id[ i ] [ j ] [ k ]. There are two options for change 1:

- option 1: omi_mask_id_delta[ i ] [ j ] [ k ] is signalled for each omi_mask_id[ i ] [ j ] [ k ]. The related syntax and semantics changes are highlighted with green.

- option 2: omi_mask_id_delta[ i ] [ j ] is signalled for each auxiliary picture. The related syntax and semantics changes are highlighted with blue.

## Change 2: add bounding box parameters to delimit the object masks in the auxiliary picture

The second proposed syntax change is to signal a bounding box to delimit the object mask borders. The related syntax and semantics changes are highlighted with yellow.

Table 1. Syntax of OMI SEI message for VSEI.

| object_mask_info( payloadSize ) { | Descriptor |
| --- | --- |
| omi_cancel_flag | u(1) |
| if( !om_cancel_flag ) { |  |
| //high level info |  |
| omi_aux_id_minus128 | ue(v) |
| omi_num_primary_pic_layer_minus1 | ue(v) |
| for(i = 0; i <= omi_num_primary_pic_layer_minus1; i++ ) |  |
| omi_primary_pic_layer_id[ i ] | ue(v) |
| omi_mask_id_length_minus8 | ue(v) |
| omi_mask_confidence_info_present_flag | u(1) |
| if( omi_mask_confidence_info_present_flag ) |  |
| omi_mask_confidence_length_minus1 | u(4) |
| omi_mask_depth_info_present_flag | u(1) |
| if( omi_mask_depth_info_present_flag ) |  |
| omi_mask_depth_length_minus1 | u(4) |
| omi_mask_label_info_present_flag | u(1) |
| if( omi_mask_label_info_present_flag ) { |  |
| omi_mask_label_language_present_flag | u(1) |
| if( omi_mask_label_language_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| omi_bit_equal_to_zero | f(1) |
| omi_mask_lable_language | st(v) |
| } |  |
| } |  |
| // individual mask information |  |
| for( i = 0; i <= omi_num_primary_pic_layer; i++ ) { |  |
| for( j = 0; j < numAuxLayer[ omi_primary_pic_layer_id[ i ] ]; j++ ){ | u(1) |
| omi_mask_id_delta[ i ][ j ] | u(v) |
| omi_mask_pic_update_flag[ i ][ j ] | f(1) |
| if( omi_mask_pic_update_flag[ i ][ j ]) { |  |
| omi_num_mask_in_pic_update[ i ][ j ] | ue(v) |
| for( k = 0; k < omi_num_mask_in_pic_update[ i ][ j ]; k++ ) { |  |
| omi_mask_id[ i ][ j ][ k ] | u(v) |
| omi_mask_id_delta[ i ][ j ][ k ] | u(v) |
| omi_mask_top[ i ][ j ][ k ] | u(v) |
| omi_mask_left[ i ][ j ][ k ] | u(v) |
| omi_mask_width[ i ][ j ][ k ] | u(v) |
| omi_mask_height[ i ][ j ][ k ] | u(v) |
| if( maskIdExist[ i ][ j ][ on_mask_id[ i ][ j ][ k ] ] ) { |  |
| omi_mask_cancel[ i ][ j ][ k ] | u(1) |
| maskIdExist[ i ][ j ][ on_mask_id[ i ][ j ][ k ] ]= !on_mask_cancel[ i ] [ j ][ k ] |  |
| } |  |
| else |  |
| maskIdExist[ i ][ j ][ on_mask_id[ i ][ j ][ k ] ] = 1 |  |
| if( maskIdExist[ i ][ j ][ on_mask_id[ i ][ j ][ k ] ] ) { |  |
| if( omi_mask_confidence_info_present_flag ) |  |
| omi_mask_confidence[ i ][ j ][ k ] | u(v) |
| if( omi_mask_depth_info_present_flag ) |  |
| omi_mask_depth[ i ][ j ][ k ] | u(v) |
| while( !byte_aligned( ) ) |  |
| omi_bit_equal_to_zero | f(1) |
| if( omi_mask_label_info_present_flag ) |  |
| omi_mask_label[ i ][ j ][ k ] | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

The variable numAuxLayer[ i ] indicates the number of the auxiliary picture layers associated with primary picture layer with nuh_layer_id equal to i. The variable associatedAuxLayer[ j ][ i ] indicates the value of nuh_layer_id of i-th auxiliary picture layer associated with primary picture layer with nuh_layer_id equal to j. numAuxLayer[ i ] and associatedAuxLayer[ j ][ i ] are derived by SDI SEI message as follows.

for( i = 0; i <= sdi_max_max_layers_minus1; i++ ) {

numAuxLayer[ sdi_layer_id[ i ] ] = 0;

}

for( i = 0; i <= sdi_max_layers_minus1; i++ ){

if( sdi_aux_id[ i ] == omi_aux_id_minus128 + 128 ){

for( j = 0; j < sdi_num_associated_primary_layers_minus1[ i ]; j++ ) {

primaryLayerId = sdi_layer_id[ sdi_associated_primary_layer_idx[ i ][ j ] ];

associatedAuxLayer[ primaryLayerId ][ numAuxLayer[ primaryLayerId ] ] = sdi_layer_id[ i ];

numAuxLayer[ primaryLayerId ]++;

}

}

}

The object mask information (OMI) SEI message provides information about object mask pictures coded as auxiliary pictures. Object mask auxiliary pictures have nuh_layer_id equal to nuhLayerIdA, sdi_layer_id[ i ] equal to nuhLayerIdA and sdi_aux_id [ i ] in the range of 128 to 159, inclusive, for any value of i in range of 0 to sid_max_layers_minus1, inclusive.

When an access unit contains an auxiliary picture picA in a layer, with nuh_layer_id equal to nuhLayerIdA, that is indicated as an object mask auxiliary layer by an OMI SEI message, a primary picture picB in a layer, with nuh_layer_id equal to nuhLayerIdB, that is indicated as a primary layer by the OMI SEI message, OMI SEI message persists in output order until one or more of the following conditions are true:

– A CLVS containing the auxiliary picture picA ends.

– A CLVS containing the primary picture picB ends.

– A CVS ends.

– The bitstream ends.

The value of sdi_aux_id[ sdi_associated_primary_layer_idx[ i ][ j ] shall be equal to 0.

omi_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous object mask information SEI message in output order that is associated with one or more primary picture layers to which this SEI applies. om_cancel_flag equal to 0 indicates that object mask information signaled in this SEI message applies.

omi_aux_id_minus128 plus 128 indicates the value of sdi_aux_id of object mask auxiliary picture layer. om_aux_id_minus128 shall be in the range of 0 to 31, inclusive.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to om_aux_id_minus128 + 128 for at least one value of i, no picture in the CVS shall be associated with an OMI SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to om_aux_id_minus128 + 128 for at least one value of i and an OMI SEI message, the SDI SEI message shall precede the OMI SEI message in decoding order.

omi_num_primary_pic_layer_minus1 plus 1 indicates the number of primary picture layers associated with the object mask auxiliary picture layers to which this SEI message applies. The value of omi_num_primary_pic_layer_minus1 shall be in the range of 0 to sdi_max_layers_minus1.

omi_primary_pic_layer_id[ i ] specifies the nuh_layer_id value of the i-th primary picture layer to which this OMI SEI message applies. The value of sdi_aux_id[ j ] shall be equal to 0 for any value of j in the range of 0 to sid_max_layers_minus1 if sdi_layer_id[ j ] equal to omi_primary_pic_layer_id[ i ]

omi_mask_id_length_minus8 plus 8 indicates the number of bits used for coding omi_mask_id[ i ] [ j ][ k ] syntax elements.

omi_mask_confidence_info_present_flag equal to 1 indicates that omi_mask_confidence[ i ][ j ][ k ] syntax elements are present. omi_mask_confidence_info_present_flag equal to 0 indicates that omi_mask_confidence[ i ][ j ][ k ]syntax elements are not present. It is a requirement of bitstream conformance that the value of omi_mask_confidence_info_present_flag shall be the same for all object_mask_info( ) syntax structures within a CLVS.

omi_mask_confidence_length_minus1 plus 1 specifies the length, in bits, of the omi_mask_confidence[ i ][ j ][ k ] syntax elements. It is a requirement of bitstream conformance that the value of omi_mask_confidence_length_minus1 shall be the same for all object_mask_info( ) syntax structures within a CLVS.

omi_object_depth_info_present_flag equal to 1 indicates that omi_object_depth[ i ][ j ][ k ] syntax elements are present. omi_object_depth_info_present_flag equal to 0 indicates that omi_object_depth[ i ][ j ][ k ]syntax elements are not present. It is a requirement of bitstream conformance that the value of omi_object_depth_info_present_flag shall be the same for all object_mask_info( ) syntax structures within a CLVS.

omi_object_depth_length_minus1 plus 1 specifies the length, in bits, of the omi_object_depth[ i ][ j ] [ k ] syntax elements. It is a requirement of bitstream conformance that the value of omi_object_depth_length_minus1 shall be the same for all object_mask_info( ) syntax structures within a CLVS.

omi_mask_label_info_present_flag equal to 1 indicates that omi_mask_label_language_present_flag and omi_mask_label[ i ][ j ][ k ] are present. omi_mask_label_info_present_flag equal to 0 indicates that omi_mask_label_language_present_flag and omi_mask_label[ i ][ j ][ k ] are not present.

omi_mask_label_language_present_flag equal to 1 indicates that omi_mask_lable_language is present. omi_mask_label_language_present_flag equal to 0 indicates that omi_mask_lable_language is not present and that the language of the mask label is unspecified.

omi_bit_equal_to_zero shall be equal to 0.

omi_mask_lable_language contains a language tag as specified by IETF RFC 5646 followed by a null termination byte equal to 0x00. The length of the omi_mask_lable_language syntax element shall be less than or equal to 255 bytes, not including the null termination byte. When not present, the language of the label is unspecified.

omi_mask_delta[ i ][ j ] indicates the tolerance range around the mask identifier of the object masks in the j-th object mask auxiliary picture associated with i-th primary picture.

The samples of the k-th object mask in the j-th object mask auxiliary picture associated with i-th primary picture are expected to be in the range ( omi_mask_id[ i ][ j ][ k ] – omi_mask_id_delta[ i ][ j ]) to ( omi_mask_id[ i ][ j ][ k ] + omi_mask_id_delta[ i ][ j ] – 1), inclusive.

omi_mask_pic_update_flag[ i ][ j ] equal to 1 indicates the mask information of j-th object mask auxiliary picture associated with i-th primary picture is signalled. omi_mask_pic_update_flag[ i ][ j ] equal to 0 indicates the mask information of j-th object mask auxiliary picture associated with i-th primary picture is not signalled. When the mask information of j-th object mask auxiliary picture associated with i-th primary picture is not present, the persistence mechanism is used, that is the information is inherited from the last OMI SEI message which signals the mask information of j-th object mask auxiliary picture associated with i-th primary picture.

omi_num_mask_in_pic_update[ i ][ j ] indicates the number of object masks in the j-th auxiliary picture associated with i-th primary picture to be signalled. omi_num_mask_in_pic_update [ i ][ j ] shall be in the range of 0 to (1<<BitDepthY) – 1, inclusive, where BitDepthY is the bit depth for the samples of the luma component.

omi_mask_id[ i ][ j ][ k ] indicates the identifier of k-th object mask in the j-th object mask auxiliary picture associated with i-th primary picture..

The variable maskId[ i ][ j ][ k ] specifying the object mask identifier of k-th object mask of the j-th object mask auxiliary picture associated with i-th primary picture in the SEI message is derived as follows:

for( i = 0; i <= omi_num_primary_pic_layer_minus1; i++ ) {

for( j = 0; j <= numAuxLayer[omi_primary_pic_layer_id[ i ]]; j++ ) {

for( k = 0; k <= on_num_mask_in_pic[ i ]; k++ ) {

maskId[ i ][ j ][ k ] = omi_mask_id[ i ][ j ][ k ] + (1<<BitDepthY)*j

}

}

}

omi_mask_delta[ i ][ j ][ k ] indicates the tolerance range around the mask identifer of k-th object mask in the j-th object mask auxiliary picture associated with i-th primary picture.

The samples of the k-th object mask in the j-th object mask auxiliary picture associated with i-th primary picture are expected to be in the range ( omi_mask_id[ i ][ j ][ k ] – omi_mask_id_delta[ i ][ j ][ k ] ) to ( omi_mask_id[ i ][ j ][ k ] + omi_mask_id_delta[ i ][ j ][ k ] – 1), inclusive.

omi_mask_top[ i ][ j ][ k ], omi_mask_left[ i ][ j ][ k ], omi_mask_width[ i ][ j ][ k ], and omi_mask_left[ i ][ j ][ k ], specify the coordinates of the top-left corner and the width and height, respectively, of the bounding box of the object identified by the identifier omi_mask_id[ i ] [ j ] [ k ].

The value of omi_mask_left[ i ][ j ][ k ] shall be in the range of 0 to ( CroppedWidth / SubWidthC – 1 ), inclusive, CroppedWidth and SubWidthC being associated to the the j-th object mask auxiliary picture associated with i-th primary picture.

The value of omi_mask_top[ i ][ j ][ k ] shall be in the range of 0 to ( CroppedHeight / SubHeightC – 1 ), inclusive, CroppedHeight  and SubHeightC  being associated to the the j-th object mask auxiliary picture associated with i-th primary picture.

The value of omi_mask_width[ i ][ j ][ k ] shall be in the range of 0 to ( CroppedWidth / SubWidthC − omi_mask_top[ i ][ j ][ k ] ), inclusive.

The value of omi_mask_left[ i ][ j ][ k ] shall be in the range of 0 to ( CroppedHeight / SubHeightC − omi_mask_left[ i ][ j ][ k ] ), inclusive.

Variable p[ i ][ j ][ x ][ y ] is the decoded value of the sample at the relative sample location (x, y) in the j-th object mask auxiliary picture associated with the i-th primary picture.

for( i = 0; i <= omi_num_primary_pic_layer_minus1; i++ ) {

for( j = 0; j <= numAuxLayer[omi_primary_pic_layer_id[ i ]]; j++ ) {

for( k = 0; k <= on_num_mask_in_pic[ i ]; k++ ) {

if( p[ i ][ j ][ x ][ y ] >= omi_mask_id[ i ][ j ][ k ] – omi_mask_id_delta[ i ][ j ][ k ]

&& p[ i ][ j ][ x ][ y ] < omi_mask_id [ i ][ j ][ k ] + omi_mask_id_delta[ i ][ j ][ k ]

&& x >= omi_mask_left[ i ][ j ][ k ] && x < omi_mask_left[ i ][ j ][ k ] + omi_mask_width[ i ][ j ][ k ]

&& y >= omi_mask_top[ i ][ j ][ k ] && y < omi_mask_top[ i ][ j ][ k ] + omi_mask_height[ i ][ j ][ k ] )

sample (x, y) is associated with mask with the identifier of maskId[ i ][ j ][ k ]

}

}

}

omi_mask_cancel[ i ][ j ][ k ] equal to 1 cancels the persistence scop of object mask with identifier equal to om_mask_id[ i ][ j ][ k ]. omi_mask_cancel[ i ][ j ][ k ] equal to 0 indicates the information of object mask with identifier equal to omi_mask_id[ i ] [ j ] is signaled.

The variable maskIdExist[ i ][ j ][ k ] equal to 1 indicates the object mask with identifier k in the j-th object mask auxiliary picture associated with i-th primary picture exists. The variable maskIdExist[ i ][ j ] [ k ] equal to 0 indicates the object mask with identifier equal to k in the j-th object mask auxiliary picture associate with i-th primary picture does not exist. maskIdExist[ i ][ j ][ k ] is initialized with 0 before decoding the current CVS.

omi_mask_confidence[ i ][ j ][ k ] indicates the degree of confidence associated with the k-th object mask in the j-th object mask auxiliary picture associated with i-th primary picture, in units of 2-( omi_mask_confidence_length_minus1 + 1 ), such that a higher value of omi_mask_confidence[ i ][ j ][ k ] indicates a higher degree of confidence. The length of the omi_mask_confidence[ i ][ j ][ k ] syntax element is omi_mask_confidence_length_minus1 + 1 bits.

omi_mask_depth[ i ][ j ][ k ] indicates the object depth associated with the k-th object mask in the j-th object mask auxiliary picture associated with i-th primary picture. A smaller value of omi_mask_depth indicates a shorter distance to the object. The length of the omi_mask_depth[ i ][ j ][ k ] syntax element is omi_object_depth_length_minus1 + 1 bits.

omi_mask_label[ i ][ j ][ k ] specifies the contents of the label associated with k-th object mask in the j-th object mask auxiliary picture associated with i-th primary picture. The length of the omi_mask_label[ i ][ j ][ k ] syntax element shall be less than or equal to 255 bytes, not including the null termination byte.

# Conclusion

The object mask auxiliary picture and object mask information SEI message were proposed in JVET-AD0175 to enhance VVC’s capability of handling information from machine analysis tasks such as object detection, tracking, and segmentation. To solve the potential issues when lossy coding is used, a tolerance range around the object ID values as well as the bounding box parameters that delimit the masks’ location are included in this contribution.

The proposed additional syntax elements aim at securing the object masks identification from the mask auxiliary pictures.

For example, the identification of an object with mask ID equal to maskId[ i ][ j ][ k ] can be detected by identifying the area in the j-th auxiliary picture associated with the i-th primary picture such as samples p(x,y) satisfy the following conditions:

- p(x,y) >= omi_mask_id[ i ][ j ][ k ] – omi_mask_id_delta[ i ][ j ][ k ]

- p(x,y) < omi_mask_id[ i ][ j ][ k ] + omi_mask_id_delta[ i ][ j ][ k ]

- x >= omi_mask_left[ i ][ j ][ k ]

- x < omi_mask_left[ i ][ j ][ k ] + omi_mask_width[ i ][ j ][ k ]

- y >= omi_mask_top[ i ][ j ][ k ]

- y < omi_mask_top[ i ][ j ][ k ] + omi_mask_height[ i ][ j ][ k ]

It is asserted that with the increasing importance of machine analysis and artificial intelligence applications, JVET should support more non-normative ways of signaling information obtained from machine analysis tasks at the encoder side. Therefore, it is suggested to adopt the proposed SEI message to VSEI.

# Reference

X. Zheng, X. Feng, L. Zhang, C. Zhao, “Mask information for Annotated region SEI message”, JCTVC-AF0024, July 2018.

J. Chen, S. Wang, Y. Ye, “AHG8/AHG9: On object mask auxiliary picture and object mask information SEI message for VSEI and HEVC”, JVET-AD0175, July 2023.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).