| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0099-v1 |
| --- | --- |

| Title: | [AHG8] NNPF and post-filter hint SEI messages for the technical report |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christopher Hollmann, Martin Pettersson, Rickard Sjöberg, Mitra Damghanian | Email: | {christopher.hollmann, martin.m.pettersson, rickard.sjoberg, mitra.damghanian} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In this contribution new subsections for the draft technical report on optimizations for encoders and receiving systems for machine analysis of coded video content are proposed. The new subsections describe the neural-network post-filter SEI messages and the post-filter hint SEI message.

# Introduction

Since its 28th meeting in October 2023, JVET has been working on a draft for a technical report on the optimization of encoders and receiving systems for machine analysis of coded video content. The newest version of this draft contains a section on various SEI messages that might be useful for machine vision applications [1]. However, the only currently described SEI message is the annotated regions SEI, which can be used to convey information about bounding boxes and marked regions of a picture.

In this contribution it is proposed to expand the section on SEI messages with a text describing the neural-network post-filter SEI messages and the post-filter hint SEI message of VSEI v3. While VSEI v3 is not yet finalized, it is asserted it will be technically frozen at the JVET meeting in July 2023 and while some technical details are still to be finalized the intended purposes of these SEI messages are not expected to be changed. The proposed text is based on draft 4 of the additional SEI messages for VSEI [2].

The changes to the draft TR can be found below. They consist of adding a pair of new subsections, number 9.2 and 9.3, to the most recent version in JVET-AD2030 [1].

# Proposed changes

The following text is proposed to be added to the draft TR:

9.2		Neural-network post-filter SEI

Two SEI messages that are specified in version 3 of VSEI are the neural-network post-filter (NNPF) characteristics (NNPFC) SEI message and the NNPF activation (NNPFA) SEI message.

The NNPFC SEI message specifies a neural network that can be used to filter pictures after decoding and such filtering may be beneficial for machine analysis tasks. This SEI message may contain an indication of the purpose of the post-filter such as general visual quality improvement, chroma upsampling, resolution resampling, picture rate resampling, bit depth upsampling, or colorization. However, the variety of post-filters is not restricted to these purposes and post-filters suitable for the application at hand may be indicated with this SEI message as well. The filter parameters for the neural network may be signaled in the NNPFC SEI message itself or could be identified by a uniform resource identifier (URI).

The NNPFA SEI message is used to enable or disable a post-filter specified by a referenced NNPFC SEI message.

Several different post-filters may be specified with multiple NNPFC SEI messages. In this case, the NNPFA SEI message can be used to indicate which of the available post-filters are to be applied to the current picture.

9.3		Post-filter hint SEI

Similar to the NNPF SEI messages, the post-filter hint SEI message can be used to signal the coefficients of a post-filter or alternatively correlation information for the design of a post-filter. The post-filter hint SEI message is specified in AVC, in HEVC and in version 3 of the VSEI specification.

This SEI message can be used to signal coefficients for a two-dimensional finite impulse response (FIR) filter, a one-dimensional FIR filter, or a cross-correlation matrix that can be used to derive a post-filter. FIR filters may for instance be used for smoothening, sharpening, edge enhancement and noise reduction operations. Such filtering may be beneficial for machine analysis tasks. Based on the transmitted parameters, the post-filter signaled in the post-filter hint SEI message may be applied to the current decoded picture only or to multiple decoded pictures.

# References

| [1] | J. Chen, C. Hollmann and S. Liu, "Optimization of encoders and receiving systems for machine analysis of coded video content (draft 2)," JVET-AD2030, Antalya, TR, April 2023. |
| --- | --- |
| [2] | S. McCarthy, T. Chujoh, M. M. Hannuksela, G. J. Sulivan and Y.-K. Wang, "Additional SEI messages for VSEI (Draft 4)," JVET-AD2006, Antalya, TR, April 2023. |

# Patent rights declaration

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).