| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0111-v1 |
| --- | --- |

| Title: | Additional conformance tests of spatial scalability for multilayer coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Salmon-Legagneur P. de Lagrange F. Urban | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes additional conformance test of spatial scalability functionality for multilayer coding. This bitstream exercises VVC with a multilayer configuration, and particularly interlayer-prediction from the first picture, and multiple reference frames together with inter-layer prediction scalability. The proposed conformance bitstreams, generated with VTM 20.2 are attached in the contribution.

# Spatial scalability

Figure 1 shows on frame of the two layers. Base layer (layer 0) is the downscaled version of the enhancement layer (layer 1).

Figure 1. Spatial scalability with 2x scaling ratio. Left: base layer, Right: enhancement layer.

Figure 2 shows an example of the GOP structure used. Both layers use GOP 16 configuration. The enhancement layer uses inter-layer prediction from the first pictures, with no intra-picture prediction.

Figure 2.GOP 16 example with inter-layer dependencies and no intra picture on enhancement layer. The first picture of the dependent layer is a B-Frame and uses inter-layer prediction.

# Proposed conformance bitstream

## Test Bitstream ILPRL_B_Interdigital

Specification: The bitstream is coded in GOP 16 configuration with two layers, where base layer is a downscaled version of layer 1 source video, with a ratio (0.5, 0.5). Layer 1 has no intra picture and has both inter-layer and intra-layer reference pictures.

Functional stage: High-level syntax, reference picture lists.

Purpose: in this bitstream, two features are tested:

- Avoid intra pictures in dependent layer.

- Exercise multiple reference pictures with mixed inter and intra layer dependencies.

## Bitstream generation

The bitstream was generated with VTM-20.2+. (commit 59e0673a03c5314a7daf4d8f589c7b652f938a7e)

The built-in functionality of the VTM was used to downscale the video on the fly with input parameters SourceScalingRatioHor=0.5, SourceScalingRatioVer=0.5 to obtain base layer directly from the full resolution source.

Intra-ref image was avoided in dependent layer with AvoidIntraInDepLayers set to true in the encoder configuration (default value).

command line: bin/EncoderAppStatic -c cfg/encoder_randomaccess_vtm_gop16.cfg -c c cfg/per-sequence/RaceHorsesC.cfg -l0 -q 30 -l0 -c cfg/per-sequence/RaceHorsesC.cfg -l0 -i C_RaceHorses_832x480_30Hz_8bit_P420.yuv  -l1 -q 36 -l1 -c cfg/per-sequence/RaceHorsesC.cfg -l1 -i C_RaceHorses_832x480_30Hz_8bit_P420.yuv  -ip 32 -fs 0 -b ILPRL_B_interdigital.bit  --InternalBitDepth=10 --OutputBitDepth=10 --SEIDecodedPictureHash=1  --SIMD=SSE42 --Verbosity=6 --PrintHexPSNR=1 -c two_layers_4K.cfg  -l0 --Level=2 -l1 --Level=3.1  -l0 --SourceScalingRatioHor=0.5 -l0 --SourceScalingRatioVer=0.5 -f 33

Note: one opl file was generated per-OLS, by running the decoder once per OLS.

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).