| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0113-v1 |
| --- | --- |

| Title: | AHG9: Extendibility and code word length of nnpfc_purpose |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Martin Pettersson | Email: | rickard.sjoberg@ericsson.com martin.m.pettersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes two alternative options for changing the length of the nnpfc_purpose syntax element in the NN post-filter characteristics (NNPFC) SEI message:

Option a: Change the coding of nnpfc_purpose from u(16) to u(24).

Option b: Change the coding of nnpfc_purpose from u(16) to ue(v).

Additionally, this contribution suggests a mechanism on how to add additional NNPFC purposes in a future v4 version of VSEI such that, according to the proponents, a v4 decoder would decode v3 NNPFC SEI messages to indicate “unknown” for those additional purposes. The mechanism is claimed to work without any text changes to the v3 text, so no other change but the change to u(24) or ue(v) is being proposed.

The authors prefer option b since adding new purposes according to the proposed mechanism is then claimed to be possible without splitting nnpfc_purpose into multiple syntax elements.

# Problem Statement

At the 30th JVET meeting in April 2023, document JVET-AD0384 argued that if a future version introduces additional NNPFC purposes, bitstreams conforming to v3 would have the bits corresponding to the new purposes equal to 0 and thereby explicitly state that the NNPFC does not have those purposes.

# Proposal

## Codeword length for nnpfc_purpose

We believe that the 16 bits used for nnpfc_purpose may be insufficient. We therefore suggest two alternative options for extending this codeword:

Option a: Change the coding of nnpfc_purpose from u(16) to u(24).

Option b: Change the coding of nnpfc_purpose from u(16) to ue(v).

Syntax for the two options are shown below with added text as yellow red text and deleted text as yellow strikethrough text.

### Option a: Making nnpfc_purpose 24 bits

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) u(24) |
| nnpfc_id | ue(v) |
| … |  |

### Option b: Code nnpfc_purpose with ue(v)

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) ue(v) |
| nnpfc_id | ue(v) |
| … |  |

## Mechanism for adding additional purposes in a future version

Assume that the specification text as of JVET-AD2005-v1 is published as version v3 and that a future version v4, as an example, adopts a new purpose for deinterlacing.

The text updates shown in section 3 below, or similar, could then be used for such a future version v4. The texts have the following properties:

- Any v3 bitstream will have the marker bit set equal to 0, so a v4 decoder will according to equation (76) set deinterlacingFlag to “unknown”.

- Any v4 bitstream will have marker bit set equal to 1, so a v4 decoder will according to equation (76) set deinterlacingFlag equal to the value of nnpfc_purpose & 0x40 or 0x80 for options a and b respectively.

- Any v4 NNPFC SEI message will be ignored by a v3 decoder since the marker bit, which must be equal to 1 in a v4 bitstream, will make the nnpfc_value in v3 equal to or greater than 64. Note that an encoder that is implemented after v4 is published may output either a v3 or v4 NNPFC SEI message.

# Example NNPFC text in a future version

## For option a: Making nnpfc_purpose 24 bits

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose_1 | u(24) u(6) |
| marker_bit | u(1) |
| nnpfc_purpose_2 | u(17) |
| nnpfc_id | ue(v) |
| … |  |

nnpfc_purpose_1 and nnpfc_purpose_2 indicates the purpose of the NNPF as specified in Table 20, where the variable nnpfc_purpose is set equal to nnpfc_purpose_1 + (nnpfc_purpose_2 << 6) [Ed note: rename the variable nnpfc_purpose, for now it is used to minimize the amount of change marks].

…

The value of nnpfc_purpose shall be in the range of 0 to 63 127, inclusive, in bitstreams conforming to this edition of this document. Values of 64 128 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 64 128 to 65 535, inclusive.

Table 20 – Definition of nnpfc_purpose

| bitMask | Interpretation |
| --- | --- |
| 0x01 | General visual quality improvement |
| 0x02 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| 0x04 | Resolution resampling (increasing or decreasing the width or height) |
| 0x08 | Picture rate upsampling |
| 0x10 | Bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| 0x20 | Colourization |
| 0x40 | Deinterlacing |

The variables chromaUpsamplingFlag, resolutionResamplingFlag, pictureRateUpsamplingFlag, bitDepthUpsamplingFlag, deinterlacingFlag, and colourizationFlag, specifying whether nnpfc_purpose indicates the purpose of the NNPF to include chroma upsampling, resolution resampling, picture rate upsampling, bit depth upsampling, deinterlacing and colourization, respectively, are derived as follows:

chromaUpsamplingFlag = ( ( nnpfc_purpose & 0x02 ) > 0 ) ? 1 : 0
resolutionResamplingFlag = ( ( nnpfc_purpose & 0x04 ) > 0 ) ? 1 : 0
pictureRateUpsamplingFlag = ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0	(76)
bitDepthUpsamplingFlag = ( ( nnpfc_purpose & 0x10 ) > 0 ) ? 1 : 0
colourizationFlag = ( ( nnpfc_purpose & 0x20 ) > 0 ) ? 1 : 0
deinterlacingFlag = ( ( nnpfc_purpose & 0x40 ) > 0 ) ? 1 : ( (marker_bit == 1 ) ? 0 : unknown )

NOTE 2– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

…

marker_bit shall be equal to 1 in bitstreams conforming to this edition of this document.

## For option b: Code nnpfc_purpose with ue(v)

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | ue(v) |
| nnpfc_id | ue(v) |
| … |  |

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20

…

The value of nnpfc_purpose shall be in the range of 0 64 to 63 255, inclusive, in bitstreams conforming to this edition of this document. Values of 64 256 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 64 256 to 65 535, inclusive.

The variable markerBit is set equal to value of (nnpfc_purpose & 0x40).

[Ed note: note that the range of nnpfc_purpose starts at 64 which means that the value of markerBit shall be equal to 1 in  bitstreams conforming to this edition of this document].

Table 20 – Definition of nnpfc_purpose

| bitMask | Interpretation |
| --- | --- |
| 0x01 | General visual quality improvement |
| 0x02 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| 0x04 | Resolution resampling (increasing or decreasing the width or height) |
| 0x08 | Picture rate upsampling |
| 0x10 | Bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| 0x20 | Colourization |
| 0x80 | Deinterlacing |

The variables chromaUpsamplingFlag, resolutionResamplingFlag, pictureRateUpsamplingFlag, bitDepthUpsamplingFlag, deinterlacingFlag, and colourizationFlag, specifying whether nnpfc_purpose indicates the purpose of the NNPF to include chroma upsampling, resolution resampling, picture rate upsampling, bit depth upsampling, deinterlacing and colourization, respectively, are derived as follows:

chromaUpsamplingFlag = ( ( nnpfc_purpose & 0x02 ) > 0 ) ? 1 : 0
resolutionResamplingFlag = ( ( nnpfc_purpose & 0x04 ) > 0 ) ? 1 : 0
pictureRateUpsamplingFlag = ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0	(76)
bitDepthUpsamplingFlag = ( ( nnpfc_purpose & 0x10 ) > 0 ) ? 1 : 0
colourizationFlag = ( ( nnpfc_purpose & 0x20 ) > 0 ) ? 1 : 0
deinterlacingFlag = ( ( nnpfc_purpose & 0x80 ) > 0 ) ? 1 : ( (markerBit == 1 ) ? 0 : unknown )

NOTE 2– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

…

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).