| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0117-v1 |
| --- | --- |

| Title: | Non-EE2: Enhanced subblock-based motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

This proposal presents the combined solution which was initially proposed in JVET-AC0187 [1] and JVET-AD0235 [2]. Three aspects are proposed to enhance the subblock-based motion compensation.

Aspect #1: TM-based subblock motion refinement.

Aspect #2: Interweaved affine prediction.

Aspect #3: RMVF candidate derivation with multiple CUs as input.

It is reported that on top of ECM-9.0, simulation results of the proposed method are as below:

RA: -0.10 % (Y), -0.11 % (U), -0.01 % (V), 102.3% (EncT), 100.8% (DecT)

LB: -0.05% (Y), -0.05% (U), -0.15 % (V), 106.8% (EncT), 102.7% (DecT)

- Introduction

In JVET-AC0187 [1], it is proposed to apply template matching (TM) to affine and SbTMVP mode. The control points of affine candidates and the motion shift of SbTMVP mode are refined using TM. In JVET-AD0235 [2], subblock-based motion compensation is further boosted by introducing the interweaved affine prediction method. Besides, Regression-based Motion Vector Field (RMVF) candidate derivation is enhanced by allowing the MVF from multiple CUs as input. In this proposal, a joint solution of methods in JVET-AC0187 and JVET-AD0235 is presented.

- Proposed methods

Three aspects are presented to enhance subblock motion compensation:

Aspect #1: TM-based subblock motion refinement.

Aspect #2: Interweaved affine prediction.

Aspect #3: RMVF candidate derivation with multiple CUs as input.

  - TM-based subblock motion refinement

It is proposed to refine control points of uni-predicted affine merge candidates and the motion shift of SbTMVP candidates using TM. For a uni-predicted affine merge candidate, a unified MV offset is assigned to all the CPMVs, and the TM cost of the affine candidate is calculated accordingly. The optimal CPMV offset with the minimum TM cost can be used to refine the corresponding affine candidate. For a SbTMVP candidate, the initial motion shift can be refined with TM, and then the refined motion shift will be utilized to derive subblock temporal motion information.

  - Interweaved affine prediction

With the interweaved prediction, a coding block is divided into subblocks with two different dividing patterns. Then two auxiliary predictions are generated by Affine Motion Compensation (AMC) with the two dividing patterns. The final prediction is calculated as a weighted sum of the two auxiliary predictions.

First, a coding block is divided into subblocks with two different dividing patterns. The first dividing pattern is the same as that in ECM as pattern 0 shown in Fig. 1, while the second dividing pattern also divides the coding block into 4×4 subblocks but with a 2×2 offset as pattern 1 shown in Fig. 1.

Second, two auxiliary predictions are generated by AMC with the two dividing patterns. The MV for each subblock in a dividing pattern is derived from CPMVs by the affine model.

The final prediction is calculated as a weighted sum of the two auxiliary predictions, formulated as:

| . | (1) |
| --- | --- |

As shown in Fig. 2, an auxiliary prediction sample located at the center of a subblock is associated with a weighting value 3, while an auxiliary prediction sample located at the boundary of a subblock is associated with a weighting value 1.

The subblocks associated with two dividing patterns are respectively refined by PROF unless the width or height is smaller than 4. Besides, subblock boundary deblocking/OBMC for affine mode is disabled.

Fig. 1. Interweaved prediction procedure.

Fig. 2. Weighting values in a subblock.

  - RMVF candidate derivation with multiple CUs

It is proposed to derive additional RMVF affine candidates by feeding the MVF of multiple CUs as regression input. In particular, MVF of M (M=2 in the test) previously affine-coded CUs are simultaneously used to generate a RMVF candidate. The additional RMVF candidates are reordered together with other RMVF candidates through ARMC.

- Experimental results

The proposed method has been implemented on top of ECM-9.0 [3]. Results under RA/LDB configurations are presented in Table 1. The simulations were performed in accordance with the common test conditions [4]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method.

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.18% | 0.11% | 102.2% | 98.1% |  |  |  |  |  |
| Class A2 | -0.22% | -0.08% | -0.11% | 102.0% | 97.4% |  |  |  |  |  |
| Class B | -0.08% | 0.04% | -0.02% | 102.5% | 101.8% | -0.09% | -0.13% | -0.05% | 106.2% | 102.4% |
| Class C | -0.07% | -0.27% | 0.00% | 102.4% | 104.3% | -0.08% | -0.33% | -0.28% | 106.8% | 105.2% |
| Class E |  |  |  |  |  | 0.05% | 0.47% | -0.15% | 107.9% | 99.8% |
| Overall | -0.10% | -0.11% | -0.01% | 102.3% | 100.8% | -0.05% | -0.05% | -0.15% | 106.8% | 102.7% |
| Class D | 0.03% | -0.08% | -0.26% | 102.5% | 100.0% | -0.13% | 0.30% | 0.01% | 109.9% | 107.2% |
| Class F | -0.03% | 0.03% | 0.09% | 100.8% | 102.5% | -0.12% | 0.00% | -0.32% | 103.3% | 102.6% |

- Conclusions

Enhanced subblock motion compensation is presented in this contribution. Simulation results indicate 0.10%/0.05% coding gain in RA/LDB configurations with 102.3%/100.8% and 106.8%/102.7% complexity increase. It is recommended to include the proposed methods in the next round of EE2.

- References

[1] L. Zhao, K. Zhang, L. Zhang, “Non-EE2: Template matching-based subblock motion refinement,” JVET-AC0187, Jan. 2023.

[2] L. Zhao, K. Zhang, L. Zhang, “Non-EE2: Enhanced subblock-based motion compensation,” JVET-AD0235, Apr. 2023.

[3] ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0.

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).