| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0119 -v1 |
| --- | --- |

| Title: | AHG14: The extension of SADL library |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijie Bao, Huairui Wang, Zhengzhong Chen | Email: | {baoweijie, wanghr827, zzchen}@whu.edu.cn |
| Source: | Wuhan University |  |  |

_____________________________

- Abstract

This contribution presents the extensions in the Small AdHoc Deep Learning (SADL) library from Wuhan University. These extensions include features such as PReLU, Slicing, and ScatterND, along with a bug fix. All of these extensions have been verified and merged.

Overview of extensions

Table 1 below summarizes the extensions in the SADL library [1].

Table 1: The overview of extensions in the SADL library.

| Language | Pure C++, header only |
| --- | --- |
| Compatibility | Support PyTorch and TensorFlow, while also updating the Onnx to SADL converter |
| Extended features | PReLU, Slicing, and ScatterND |
| Bug fixes | Fix the bug related to the even number of uses of Conv2D |
| Type support | float, int32, int16, int8 |
| Status | merged |
| License | BSD 3-Clause |

Extended features

PReLU

Feature description: PReLU takes input data tensor and alpha tensor as input and produces one output data tensor where the function f(x)=alpha * x for x < 0, f(x) = x for x >= 0., is applied to the data tensor elementwise.

Extension description: In the original SADL library, PReLU is limited to utilizing a global alpha parameter for a tensor and is categorized as LeakyReLU. This contribution treats PReLU as a separate feature and expands the capabilities of PReLU by introducing support for both a global alpha parameter and independent alpha parameters for each channel.

Slicing

Feature description: Slicing uses the parameters including starts, ends, axes, and steps to select a sub-tensor of its input data tensor.

Extension description: In the original SADL library, Slicing is initially supported only for TensorFlow. This contribution expands the capabilities of Slicing by introducing support for PyTorch. If the parameter steps of Slicing is set to 1, then in PyTorch-ONNX, the Slicing will include the parameter steps, while in TensorFlow-ONNX, the Slicing will not include the parameter steps. This contribution modifies the convert/main.py file without modifying the SADL library. Specifically, in PyTorch-ONNX, the parameter steps of Slicing is restricted to 1, and it is not processed further.

ScatterND

Feature description: ScatterND takes three inputs tensors, including data tensor, indices tensor, and updates tensor. The output of ScatterND is produced by creating a copy of the input data, and then updating its value to values specified by updates at specific index positions specified by indices.

Extension description: This contribution introduces a new feature called ScatterND, which supports replacing a portion of data in one tensor with another tensor.

Taking PyTorch-ONNX as an example, for data tensor with shape [1, 2, 4, 4] and updates tensor with shape [1, 1, 4, 4], the data[:, 1:, :, :] = updates operation in SADL can be represented as follows:

[INFO] id: 0 op Placeholder - name: onnx::Slice_0 - inputs: - dim: ( 1 4 4 2 ) - q: 0 [INFO] id: 1 op Placeholder - name: onnx::Expand_1 - inputs: - dim: ( 1 4 4 1 ) - q: 0 [INFO] id: 2 op Const - name: /Concat_output_0 - inputs: - tensor: ( 4 4 1 4 ) - data: 0 0 0 1 ... - quantizer: 0 [INFO] id: 3 op ScatterND - name: 65 - inputs: 0 1 2

where layer 0 represents the data tensor, layer 1 represents the updates tensor, layer 2 represents the indices tensor, and layer 3 represents the new feature ScatterND.

Bug fixes

reshapeConv2DFilters

Bug description: In the original SADL library, when using a Conv2D N time (N > 1), there will be N Conv2D layers in the ONNX file, but they all point to the same initializer for their weights. During the conversion from ONNX to SADL, these Conv2D layers in the SADL library are pointing to the same filter. Therefore, when N is even, the function reshapeConv2DFilters will be executed an even number of times on the same filter, leading to an error.

Fix description: This contribution modifies the reshapeConv2DFilters function to restrict each filter to be reshaped only once. Now, in the SADL library, a Conv2D can be used an even number of times without causing any errors.

Changes details

Changes are also available in the repository at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/sadl

- Extend PReLU to support independent alpha parameters for each channel

- Extend Slicing to support PyTorch

- Add a new feature called ScatterND

- Fix the function reshapeConv2DFilters

Conclusion

This contribution presents the extensions in the Small AdHoc Deep Learning (SADL) library from Wuhan University. All of these extensions have been verified and merged.

Reference

[1] F. Galpin, T. Dumas, P. Bordes, E. François. AhG14: SADL v5 changes. JVET-AD0164, 30th Meeting, Antalya, TR, 21-28 April 2023.

Acknowledgement

We are grateful to InterDigital Inc. for their valuable guidance and assistance. Their contributions have made a significant impact on this work.

Patent rights declaration(s)

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.