| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0125 |
| --- | --- |

| Title: | EE2-4.1: Shifting quantization center |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) | Muhammet Balcilar Karam Naser Franck Galpin Fabrice Le Léannec | Email: | muhammet.balcilar@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports EE2-4.1 test results on the shifting quantization center where the shifting amount is driven by gradient of the entropy. In this contribution, we assume the gradient of the entropy is inverse proportional to the quantization index. Thus, we shift the quantization center by the inverse proportional to the quantization indices.

The following results are obtained:

- AI   :	{-0.10%, -0.12%, -0.11% ;100%, 100%}.

- RA  :	{-0.06%, -0.26%, -0.18% ;100%, 100%}.

- LDB:	{-0.03%, -0.07%,  0.09%; 100%, 100%}.

# Introduction

The idea of shifting latent representations (transform coefficients) by the gradient of the rate was first proposed for end-to-end compression that results in a 1-2% coding gain [1]. To implement the same idea in conventional compression, the dequantized transform coefficients (or quantization centers) need to be shifted. The amount of shifting must be proportional to the gradient of the rate by theory [1]. However, in conventional compression having explicit gradient of the rate needs extra complexity in decoder. In practice, we can bypass this extra complexity by using some approximation of the rate and its gradient.

# Proposal

In this proposal, we assume simple proxy of rate prediction where each quantization indices are independent, and rate increases by logarithm of the absolute value of the quantization index as in Eq.(1).

Ryi≅a.log2(yi)+b                                                               (1)

Here, yi ∈ Z is the quantization index of i-th transform coefficient and a,b ∈ R. Using this assumption, we can write gradient of the rate in Eq.(2).

∇yiRyi≅a/ln2 yi=αyi,     where yi≠0                                    (2)

Since α=a/ln⁡(2) is constant scalar, gradient of the rate is inverse proportional to the quantization index in Eq.(2). It yields that dequantized transform coefficients or quantization centers to be shifted inverse proportional to the quantization indices.

In implementation, we prepared an integer valued look up table T=0,α,α2,α3…,αα where each element is the shifting amount for each possible unique absolute quantization indices. Shifting of dequantized coefficients are performed by Eq.(3).

- xi←1024-T[yi]*Q-1yi+T[yi]*Q-1y'i>>10    if  T>yi>0                  (3)

Here, T is length of the lookup table, yi ∈ Z is the quantization index of i-th transform coefficient, Q-1yi is the dequantized value of the i-th transform coefficient, xi is the dequantized coefficient of our proposal, y'i is the auxiliary quantization index that can be calculated in Eq.(4).

- y'i=yi+(yi>0?1:-1)                                                              (4)

In this contribution, we use α=63 that gives the 64-length lookup table in Eq.(5).

T=[0,63, 31, 21, 15, 12, 10, 9, 7, 7, 6, 5, 5, 4, 4, 4, 3, 3, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 1,…,1]       (5)

If the absolute quantization index is 0 or above 63, Eq.(3) is not applied but the default dequantization is used.

# Simulation Results

The proposed test is implemented on top of ECM-9.0. Simulations are performed following the common test conditions of EE2. The simulation results are shown in Table 1-2 and 3.

Table 1: Proposed method on top of ECM-9.0 under AI settings

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.16% | -0.23% | 101% | 100% |
| Class A2 | -0.06% | -0.16% | -0.04% | 99% | 98% |
| Class B | -0.12% | -0.04% | -0.14% | 101% | 98% |
| Class C | -0.08% | -0.19% | -0.12% | 100% | 102% |
| Class E | -0.13% | -0.07% | 0.01% | 100% | 101% |
| Overall | -0.10% | -0.12% | -0.11% | 100% | 100% |
| Class D | -0.06% | -0.10% | -0.11% | 101% | 102% |
| Class F | -0.05% | -0.12% | -0.10% | 103% | 100% |
| Class TGM | -0.03% | -0.07% | -0.02% | 102% | 98% |

Table 2: Proposed method on top of ECM-9.0 under RA settings

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.39% | -0.09% | 99% | 98% |
| Class A2 | -0.04% | -0.09% | -0.25% | 95% | 98% |
| Class B | -0.07% | -0.13% | -0.19% | 103% | 102% |
| Class C | -0.05% | -0.46% | -0.20% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.26% | -0.18% | 100% | 100% |
| Class D | -0.04% | -0.22% | -0.24% | 101% | 101% |
| Class F | -0.08% | -0.11% | -0.27% | 100% | 101% |
| Class TGM | -0.02% | -0.04% | 0.00% | 99% | 99% |

Table 3: Proposed method on top of ECM-9.0 under LDB settings

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | 0.12% | 0.30% | 100% | 101% |
| Class C | 0.02% | -0.37% | -0.15% | 100% | 99% |
| Class E | -0.07% | 0.01% | 0.06% | 101% | 101% |
| Overall | -0.03% | -0.07% | 0.09% | 100% | 100% |
| Class D | -0.08% | 0.77% | 0.08% | 101% | 101% |
| Class F | -0.01% | -0.94% | -1.38% | 98% | 99% |
| Class TGM | -0.04% | 0.04% | -0.03% | 102% | 102% |

# References

[1] Balcilar, M., Damodaran, B. B., Naser, K., Galpin, F., & Hellier, P. Vector Quantization and Shifting: Exploiting Latent Properties to Optimize Neural Codecs. https://openreview.net/forum?id=QVSoh6VM4nG

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).