| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0126 |
| --- | --- |

| Title: | AHG9: NNPF cleanup and editorial changes for VSEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Wei Jia1, Jizheng Xu1, Chaoyi Lin2 18910 University Center Lane San Diego, CA 92122, USA 2Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China | Email: | yekui.wang@bytedance.com wei.jia@bytedance.com xujizheng@bytedance.com linchaoyi.cy@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes various cleanup and editorial changes, where the cleanup changes refer to technical changes that are targeting for design cleanness only, not for any functionality change, while editorial changes refer to asserted purely editorial changes.

The proposed changes that the authors deem worthing a summary are summarized as follows:

- Move the syntax present when nnpfc_mode_idc is equal to 1 to be immediately after the syntax present when nnpfc_mode_idc is equal to 0, such that information about the network topology and parameters of the NNPF is after the NNPF properties regardless of the value of nnpfc_mode_idc. This also allows for sharing of the byte alignment syntax for nnpfc_mode_idc equal to 0 and nnpfc_mode_idc equal to 1, including merging of two syntax elements nnpfc_reserved_zero_bit_a and nnpfc_reserved_zero_bit_b into one syntax element nnpfc_reserved_zero_bit.

- Change nnpfc_reserved_zero_bit(_a/_b) to nnpfc_alignment_zero_bit(_a/_b). It is asserted that there is no need to reserve the value 1 for this byte alignment bit, as the VVC and VSEI specs almost always just simply specify it as "shall be equal to 0". Only one of all cases in the latest VVC and VSEI specs is specified as a reserved bit, that is ptl_reserved_zero_bit. Even that one is not really necessary; however, that one is a bit special, as the bits in the PTL syntax structure are way more precious than other places, because the PTL syntax structure are often directly copied into top-level systems signalling/negotiation of capabilities.

- Rename the syntax element nnpfc_input_pic_output_flag[ i ] to be nnpfc_input_pic_filtering_flag[ i ] and change the semantics accordingly, as this flag indicates whether for the i-th input picture the NNPF generates a corresponding filtered picture stored in the output tensor, not whether the NNPF outputs a corresponding picture, which is controlled by the nnpfa_output_flag[ i ] instead. This also includes changing of the name of the variable numOutputPics to numPicsInOutputTensor.

- Change the local variables chromaUpsamplingFlag, resolutionResamplingFlag, pictureRateUpsamplingFlag, bitDepthUpsamplingFlag, and colourizationFlag to be global variables ChromaUpsamplingFlag, ResolutionResamplingFlag, PictureRateUpsamplingFlag, BitDepthUpsamplingFlag, and ColourizationFlag, because most, if not all, of them are used in more than one (sub)clauses of the text.

- Change the condition for the constraint on the either nnpfcOutputPicWidth is not equal to 0 or nnpfcOutputPicHeight is not equal to CroppedHeight from "when nnpfc_pic_width_num_minus1, nnpfc_pic_width_denom_minus1, nnpfc_pic_height_num_minus1, and nnpfc_pic_height_denom_minus1 are present" to "when resolutionResamplingFlag is equal to 1". From the meaning point of view, it should be better to use resolutionResamplingFlag equal to 1 as the condition instead of the presence of the syntax elements, as the essence herein is really to disallow not changing either the width or height when the NNPF purpose includes resolution resampling.

- Change all instances of "Values of se_a greater than y shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use" to "The value of se_a shall be in the range of 0 to y, inclusive", as the latter is a wording consistent with the usual way of expressing the value range for a ue(v)-coded syntax element in AVC/HEVC/VVC/VSEI specifications.

- Move nnpfa_target_base_flag to be immediately after nnpfa_persistence_flag, such that the persistence flag, when present, immediately follows the cancel flag, thus these two flags specifying the persistence scope are together, same as all other SEI message syntax containing a cancel flag and a persistence flag.

The proposed changes are included in an attachment, with all the proposed changes marked relative to JVET-AD2006-v1.

# Patent rights declaration(s)

To the authors' knowledge, Bytedance Inc. does not have current or pending patent rights relating to the changes proposed in this contribution.