| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0135-v1 |
| --- | --- |

| Title: | AHG9: On NNPF picture rate upsampling constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Ye-Kui Wang 8910 University Center Lane, San Diego, CA92122, USA | Email: | xujizheng@bytedance.com yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to make the following changes to the NNPF SEI messages design:

- Add a note to clarify common use cases that include picture rate upsampling.

- Clarify the constraint of output time instance by either 1) clearly defining the output time instance for each interpolated picture or 2) disallowing ambiguous cases.

# Introduction

At the previous JVET meeting, it was agreed that after applying activated NNPFs for pictures in a bitstream, there shall be no more than one NNPF output picture pertaining to any particular output time instance.

There are two problems:

- In some common use cases of picture rate upsampling, it is easy to violate such a constraint.

- The output time instance for an interpolated picture is vague, which may lead to undesired behaviors.

5

6

Input pictures:

Output tensor:

NNPF

0

2

4

1

3

currPic

Problem a) can be demonstrated by an exemplar picture rate upsampling NNPF shown in Figure 1.

Figure 1. An exemplar picture-rate-upsampling NNPF.

The NNPF takes 3 pictures, (picture 0, picture 2, picture 4) in the example, as input and outputs two interpolated pictures (picture 1, picture 3) in the example. Such an NNPF activated by an NNPFA SEI message with nnpfa_persistence_flag equal to 1 is a common use case. While applying the NNPF at picture 6 (because nnpfa_persistence_flag is equal to 1), picture 3 will be output again, which violates the constraint of no more than one NNPF output picture pertaining to any particular output time instance.

Input pictures:

Output tensor:

NNPF

0

6

12

2

4

9

Problem b) can be illustrated by another exemplar NNPF, as shown in Figure 2.

Figure 2. Another exemplar picture-rate-upsampling NNPF for demonstration of problem b).

currPic

0

6

12

18

2

4

9

Input pictures:

Output tensor:

NNPF

The exemplar NNPF takes 3 pictures as input and output two interpolated pictures between the first pair of input pictures and one additional interpolated picture between the second pair of input pictures.

Figure 3. The NNPF activated at picture 12 will output picture 2, picture 4 and picture 9.

We assume that the interpolated pictures are uniformly located between each pair of consecutive input pictures. When the NNPF is activated at picture 12, (picture 0, picture 6, picture 12) is the input and the output includes (picture 2, picture 4, picture 9). When the NNPF is activated at picture 18 (by the same NNPFA SEI message with nnpfa_persistence_flag is equal to 1 or another new NNPFA SEI message), the output includes (picture 8, picture 10, picture 15). The process can be illustrated in Figures 3 and 4. It does not violate the constraint of no more than one NNPF output picture pertaining to any particular output time instance but one undesirable behavior is that picture 9 is output before picture 8 in such a case. To support such a behavior, more pictures should be buffered to make the final output following the correct output time instance.

currPic

0

6

12

18

2

4

9

Input pictures:

Output tensor:

NNPF

15

8

10

Figure 4. The NNPF activated at picture 18 will output picture 8, picture 10 and picture 15.

Furthermore, the assumption that the interpolated pictures are uniformly located between each pair of consecutive input pictures may not be guaranteed (as currently that is not specified), therefore, such NNPFs activated at different pictures may also easily violate the constraint of no more than one NNPF output picture pertaining to any particular output time instance. This makes the constraint really vague.

# Proposed solutions

To solve problem a), we propose to add a note marked in turquoise for those common cases:

NOTE – When pictureRateUpsamplingFlag is equal to 1 for an NNPF and the NNPFA SEI message that activated this NNPF has nnpfa_persistence_flag equal to 1, nnpfc_interpolated_pics[ i ] is greater than 0 only for a single value of i in the range of 0 to numInputPics − 1, inclusive.

There are at least different solutions for problem b):

- Clearly specify that the interpolated pictures are uniformly located between each pair of consecutive input pictures.

- Indicate an output time instance for each interpolated picture, e.g., through signalling a POC delta between the POC values of the interpolated picture and the input picture closest and succeeding in output order.

- Disallow generating NNPF output pictures between any particular pair of consecutive input pictures more than once.

With solutions 1) and 2), the behavior described above (picture 9 is output before picture 8) is still allowed but it is clear to tell if the output time instance constraint is violated or not. With solution 3, the constraint won't be violated and the behavior of outputting picture 9 before picture 8 is avoided.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).