| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0137-v1 |
| --- | --- |

| Title: | Non-EE2: On TIMD Fusion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P. Andrivon M. Blestel | Email: | pandrivon@ofinno.com mblestel@ofinno.com |
| Source: | Ofinno LLC |  |  |

_____________________________

# Abstract

The contribution presents the simulation results of a modified TIMD fusion process. On top of ECM-9.0, experimental results reportedly show BD-rate -0.04% / 0.06% / 0.04%, 100%, 101% for AI conditions.

Introduction

The TIMD process is based on a template analysis in which several intra modes are tested. The two INTRA modes providing the smallest SATD cost are used for a fusion process. In this contribution, the INTRA modes selection and the fusion process are modified.

# Proposed methods

The proposed test is implemented on top of the ECM-9.0. Simulations are performed following the common test conditions of EE2. The simulation results are shown in Table 1.

Table 1: Modified TIMD process vs ECM-9.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,04% | 0,06% | -0,02% | 100,1% | 99,9% |
| Class A2 | -0,04% | 0,02% | 0,07% | 99,6% | 100,5% |
| Class B | -0,05% | 0,03% | 0,05% | 100,1% | 100,9% |
| Class C | -0,03% | 0,00% | 0,05% | 100,4% | 101,2% |
| Class E | -0,03% | 0,21% | 0,07% | 102,0% | 101,7% |
| Overall | -0,04% | 0,06% | 0,04% | 100,4% | 100,9% |
| Class D | 0,02% | -0,01% | 0,04% | 101,0% | 102,5% |
| Class F | 0,09% | 0,20% | 0,22% | 100,5% | 100,9% |
| Class TGM | -0,01% | -0,04% | -0,03% | 99,2% | 97,9% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# References

- M. Coban, F. Le Léannec, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 8 (ECM 8),” JVET-AC2025, Jan. 2023.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Ofinno LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).