| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET- AE0146-r1 |
| --- | --- |

| Title: | AHG 12: Fixes H-CCP Table for CUs in Refreshed Areas of GDR/Recovering Pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungwook Hong Limin Wang Krit Panusopone | Email: | seungwook.hong@nokia.com limin.2.wang@nokia.com krit.panusopone@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution presents a solution to address the GDR leak caused by the adoption of JVET-AD0188 (JVET-AC0176 and JVET-AC0315) in ECM. JVET-AD-0188 introduces a H-CCP table that retains CCP model information of the most recent coded CUs in CCP model. The entries of the H-CCP table may be included in the CCP merge candidate list. In the case of GDR, if an entry in the H-CCP table originates from coded CUs in a non-refreshed area of a GDR/recovering picture, it may result in a leak. To avoid such leak issue, a separate H-CCP is proposed for CUs in the refreshed area, which would encompass only the CCP model information of the last coded CUs in the refreshed area. This fix has been implemented in ECM 9.0 master. commit 2e1d547d. The simulations over JVET test sequences revealed no instances of leak occurring after implementing the proposed fix.

# Introduction

In the 30th JVET meeting, JVET-AD0188 was adopted, in which a CCP (Cross Component Prediction) merge candidate list is introduced in ECM. The merge candidate list is built using spatial adjacent, spatial non-adjacent, and history-based candidates.

The history-based candidates are stored in a table called the H-CCP (History-based CCP) table. At the start of each CTU row, the H-CCP table is reset, and it is updated when a CU in the CCP model is coded.

The CCP merge candidate list is populated with spatial adjacent and non-adjacent candidates first. If the CCP merge candidate list is not filled completely with these candidates, the CCP models stored in the history-based table are added to the list.

However, in the case of GDR(Gradual Decoding Refresh), a potential issue arises. If any entry in the H-CCP table corresponds to a CU in the non-refreshed area of a GDR/recovering picture, a leak may occur. This is because the decoder cannot guarantee the correct decoding of coding information in non-refreshed areas.

# Proposal

To address this concern, a fix is proposed. It suggests creating a separate H-CCP table for CUs in the refreshed area, which includes only the CCP models of the last coded CUs in the refreshed area.

To address the concern regarding the potential leak in GDR scenario, a proposed fix recommends the creation of a separate H-CCP table specifically for CUs located in the refreshed area. This separate H-CCP table will only contain the CCP models of the last coded CUs within the refreshed area. With this separate H-CCP table, the CCP models of CUs in the non-refreshed areas will not be included in coding of CUs in refreshed areas, thus preventing potential leaks.

# Experimental Results

The fix was implemented in ECM 9.0 master commit 2e1d547d.  Simulations are conducted over JVET sequences under low-delay B configuration (i.e., encoder_lowdelay_vtm.cfg) with the following settings.

- Anchor: the first picture is coded as IDR picture and then a CRA is inserted every second followed by 4 regular B pictures.

- GDR: the first picture is coded as IDR picture and then a GDR picture (with ph_pic_recovery_cnt = frame rate) is inserted every second followed by 4 regular B pictures.

No instances of leaks were observed.

Table 1 provides BD rate of GDR over anchor. The overall loss is around xx%. Note that a certain loss of GDR over anchor is expected that because CUs in refreshed areas of GDR/recovering picture cannot use the coding information of non-refreshed areas of the current picture and reference pictures.

Table 1. The left GDR over anchor

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over GDR Anchor (ECM 9.0 master commit 2e1d547d) on Linux |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 1.43% | 3.83% | 4.66% | 104% | 104% |
| Class E | 6.11% | 17.77% | 16.29% | 105% | 107% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 2.47% | 7.51% | 8.76% | 103% | 104% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusions

This contribution suggests a solution to address GDR leak issue in ECM with adoption of JVET-AD0188. It introduces a H-CCP table to store the CCP models of the last coded CUs. However, when GDR is applied, a potential leak can occur if an entry of H-CCP table is from a CU in the non-refreshed area. To prevent such leaks, the proposal recommends the creation and maintenance of a separate H-CCP table specifically for CUs in the refreshed area. This refreshed area H-CCP table would include only the CCP models of the last CUs in the refreshed area. The proposed fix has been successfully implemented and tested in ECM 9.0 master. Through simulations conducted using JVET test sequences, it has been verified that no leaks occur after applying the fix. Therefore, it is recommended to adopt the solution in ECM.

# References

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0.

- Kai Zhang, Li Zhang, Zhipin Deng, “Non-EE2: Non-Local Cross-Component Prediction”, JVET-AC0176, Jan. 2023.

- Chia-Ming Tsai, Hsin-Yi Tseng, Cheng-Yen Chuang, Chih-Wei Hsu, Ching-Yeh Chen, Tzu-Der Chuang, Olena Chubach, Yi-Wen Chen, Yu-Wen Huang, Shaw-Min Lei, “Non-EE2: Cross-component merge mode for chroma intra coding”, JVET-AC0315, Jan., 2023.

- Kai Zhang, Li Zhang, Zhipin Deng, Chia-Ming Tsai, Hsin-Yi Tseng, Cheng-Yen Chuang, Chih-Wei Hsu, Ching-Yeh Chen, Tzu-Der Chuang, Olena Chubach, Yi-Wen Chen, Yu-Wen Huang, Shaw-Min Lei, “EE2-1.6: Non-local cross-component prediction and cross-component merge mode”, JVET-AD0188, April 2023.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).