| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0147-v2 |
| --- | --- |

| Title: | AHG12: Updating context model parameters for ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao Yan Ye Jie Chen Xinwei Li | Tel: Email: | {ruling.lrl, yan.ye}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

# Abstract

This contribution proposes to update context initialization parameters for I-slice. Specifically, the initial probabilities, window sizes and adaptive weights for I-slice context models are retrained and updated. It is reported that on top of ECM-9.0, the overall coding performance impact for {Y, U, V} is {-0.10%, 0.07%, 0.15%} for AI, and slightly coding performance impact for RA and LB.

# Introduction

The initial probabilities, window sizes, adaptive weights for multi-hypothesis-based probability and window adjustments for context have not been retrained since ECM-5.0. These parameters may not be suitable for current ECM because multiple coding tools are added.

In this contribution, we propose to retrain context initialization parameters for I-slice. The training scripts are modified from the one used in VVC’s CE on CABAC initialization. For B-slice and P-slice, the context initialization parameters remain the same since the context parameters are predicted from previous coded inter slices and some issues are encountered when training parameters together with this method.

# Simulation results

The retrained context initialization parameters are implemented on top of ECM-9.0 and are tested under CTC for enhanced compression tool testing with all intra, random access and low delay B configurations.

Table 1. Simulation results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.18% | 0.15% |  |  |
| Class A2 | -0.05% | 0.08% | 0.16% |  |  |
| Class B | -0.08% | 0.06% | 0.25% | 100% | 100% |
| Class C | -0.14% | 0.04% | 0.20% | 99% | 99% |
| Class E | -0.17% | 0.00% | -0.11% | 102% | 101% |
| Overall | -0.10% | 0.07% | 0.15% |  |  |
| Class D | -0.13% | -0.07% | 0.05% | 99% | 97% |
| Class F | -0.28% | -0.11% | 0.01% |  |  |
| Class TGM | -0.23% | -0.13% | -0.14% | 101% | 101% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.12% | 0.01% |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.17% | -0.05% | 100% | 99% |
| Class C | -0.03% | -0.16% | 0.13% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.00% | -0.35% | -0.08% | 100% | 100% |
| Class F | -0.17% | -0.21% | -0.04% | 100% | 100% |
| Class TGM | -0.06% | -0.03% | -0.10% | 100% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.08% | -0.27% | -0.25% | 102% | 101% |
| Class E | 0.05% | -0.71% | 1.29% | 99% | 99% |
| Overall |  |  |  |  |  |
| Class D | -0.13% | 0.96% | -0.26% | 102% | 101% |
| Class F |  |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

# Conclusion

This contribution proposes to update the initialization parameters for I-slice context models. It is reported that 0.10% BD-rate reduction is achieved in AI configuration, and slightly impact on BD-rate in RA and LB configurations. Considering the coding efficiency improvement, we recommend to adopt this contribution to next version of ECM.

# Patent rights declaration(s)

Alibaba group does not have any current or pending patent rights relating to the technology described in this contribution.