| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0151-v1 |
| --- | --- |

| Title: | EE2-5.1: CCSAO with extended edge classifiers and history offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Che-Wei Kuo, Xiaoyu Xiu, Wei Chen, Hong-Jheng Jhu, Ning Yan, Changyue Ma, Xianglin Wang | Email: | cheweikuo@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports results of EE2 Test 5.1. There are two sub-tests in the EE test. In 5.1a, temporal historical CCSAO offsets are stored for the usage of future frames. On top of 5.1a, the edge classifier is extended with additional edge/band combinations and the component used for edge classification can be selected from one of all three components in 5.1b.

Compared to ECM-9.0 anchors, the BD-rate impact is summarized as below. For class A to E results of 5.1b, one and three points are copied from anchor for random access and low delay B configurations, respectively.

| Class A to E |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| 5.1a | AI | - | - | - | - | - |
|  | RA | -0.05% | -0.24% | -0.17% | 100.4% | 100.3% |
|  | LB |  |  |  |  |  |
| 5.1b | AI | -0.04% | 0.00% | -0.09% | 100.3% | 100.1% |
|  | RA* | -0.11% | -0.33% | -0.32% |  |  |
|  | LB* | -0.12% | -0.70% | -0.85% |  |  |
| Class F |  | Y | U | V | EncT | DecT |
| 5.1a | AI | - | - | - | - | - |
|  | RA | -0.11% | -0.22% | -0.21% | 100.3% | 101.2% |
|  | LB | -0.07% | -0.50% | -0.92% | 100.5% | 99.7% |
| 5.1b | AI | -0.24% | -0.36% | -0.36% | 101.5% | 100.9% |
|  | RA | -0.32% | -0.38% | -0.34% |  |  |
|  | LB | -0.31% | 0.02% | -0.13% |  |  |
| Class TGM |  | Y | U | V | EncT | DecT |
| 5.1a | AI | - | - | - | - | - |
|  | RA | -0.11% | -0.08% | -0.14% | 100.3% | 99.7% |
|  | LB |  |  |  |  |  |
| 5.1b | AI | -0.41% | -0.11% | -0.09% | 101.5% | 100.8% |
|  | RA | -0.44% | -0.05% | -0.09% |  |  |
|  | LB | -0.69% | -0.13% | -0.22% |  |  |

# Introduction

In ECM-9.0 [4], Cross-component Sample Adaptive Offset (CCSAO) enhances the reconstructive quality of samples by classifying them into different categories and adding one offset to the reconstructed samples of each category. Both band classifier [1] and edge classifier [2] are supported and switched in CTB level. A maximum of 4 classifiers can be used in one slice. Classifier parameters/offsets are derived at encoder and signaled for each slice independently. The band classifier uses the band indices of three collocated samples to classify one given sample, as formulated in (1). The selected luma collocated position, i.e., Ycol, and the number of divided bands (NY, NU, NV) are decided by encoder and signaled in SH.

The edge classifier combines luma edge and the band index of the sample at the corresponding collocated position to determine the classification of one given sample. As in (2), the category index i is derived based on the combination of an edge index iE and a band index iB. Specifically, iE is calculated based on two edge indices iEa and iEb which are calculated by comparing a sample difference d in one direction with a threshold T. Similar to the band classifier, the band index iB is derived based on the selected sample Bcol that divides the range of sample values to NB bands. The number of edge categories is fixed to 16 (4*4), while the number of band categories ranges from 1 to 4. Same to SAO, the edge direction can be selected from horizontal, vertical, 135° diagonal and 45° diagonal, as illustrated in Figure 1. The selected edge direction, Bcol, NB, and T are decided by encoder and signaled in SH.

bandY=Ycol∙NY≫BDbandU=Ucol∙NU≫BDbandV=Vcol∙NV≫BDi=bandY∙NU∙NV+bandU∙NV+bandVC'rec=Clip1Crec+σCCSAOi                       (1)

&&iE=d<0?d<-T?0:1:d<T?2:3&&iB=Bcol∙NB≫BD&&&i=iB∙16+iEa∙4+iEb&&C'rec=Clip1Crec+σCCSAOi                        (2)

Figure 1 Four 1-D patterns for CCSAO EO: horizontal, vertical, 135° diagonal, and 45° diagonal.

# Proposal

## History offsets

It is noticed that there is strong correlation among the CCSAO offsets and classifier parameters of different pictures. To reduce the signaling overhead, one CCSAO inheritance scheme is proposed, where the offsets/parameters of some coded pictures are stored at both encoder and decoder which are allowed to be used as the CCSAO offsets/classifiers of future pictures. An index is signaled in SH to indicate which candidate in the CCSAO parameter storage is selected for the current slice. The candidates in the CCSAO parameter storage are updated in a FIFO manner and refreshed at IDR pictures.

## Extended edge classifiers

One extended edge classifier is proposed in the EE test. As shown in (3), beside the existing CCSAO edge classification, one new edge classification is added in the propose scheme, which is one subset of the original one with less edge range divisions. This allows more flexible edge/band combinations to adapt to the local characteristics of video sequences. Specifically, the number of edge categories ranges from 16 to 4 (4*4 to 2*2), and the corresponding maximum number of band categories can range from 4 to 14. Additionally, the component used for edge classification can be selected from one of all three components. Same to the existing CCSAO design, the selected edge classifier and edge component are decided by encoder and signaled in SH.

&&orginal: iE=d<0?d<-T?0:1:d<T?2:3&&added:  iE=                     d<T?0:1                     (3)

# Simulation results

The proposed methods were implemented on top of the reference software ECM-9.0 [4] and tested using the common test conditions as defined in [5].

## Test 5.1a: History offsets

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.25% | -0.08% | 100.6% | 100.9% |
| Class A2 | -0.08% | -0.15% | -0.20% | 100.4% | 99.6% |
| Class B | -0.04% | -0.25% | -0.23% | 100.4% | 100.4% |
| Class C | -0.04% | -0.30% | -0.15% | 100.3% | 100.4% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.24% | -0.17% | 100.4% | 100.3% |
| Class D | -0.06% | -0.28% | -0.10% | 100.5% | 100.6% |
| Class F | -0.11% | -0.22% | -0.21% | 100.3% | 101.2% |
| Class TGM | -0.11% | -0.08% | -0.14% | 100.3% | 99.7% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |
| Class C | -0.06% | -0.23% | -0.52% | 100.8% | 99.8% |
| Class E | -0.04% | 0.24% | -0.42% | 99.9% | 99.8% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |
| Class D | -0.16% | -0.25% | -0.92% | 101.1% | 100.0% |
| Class F | -0.07% | -0.50% | -0.85% | 100.5% | 99.7% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |

## Test 5.1b: Test 5.1a + extended edge classifier

# References

- C.-W. Kuo, X. Xiu, Y.-W. Chen, H.-J. Jhu, W. Chen, X. Wang, “EE2-5.1: Cross-component Sample Adaptive Offset”, JVET-W0066, Jul. 2021, Teleconference.

- A. M. Kotra, N. Hu, V. Seregin, M. Karczewicz, C.-W. Kuo, X. Xiu, Y.-W. Chen, H.-J. Jhu, W. Chen, N. Yan, X. Wang, “EE2-5.1: Edge classifier for Cross-component Sample Adaptive Offset (CCSAO)”, JVET-Y0106, Jan. 2021, Teleconference.

- C.-W. Kuo, X. Xiu, W. Chen, H.-J. Jhu, N. Yan, C. Ma, X. Wang, “AHG12: CCSAO with extended edge classifiers and history offsets”, JVET-AD0218, Jun. 2023, Antyala.

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, Jun., 2023.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).