| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0152-v1 |
| --- | --- |

| Title: | EE2-5.3: Combination of Test 5.1b and Test 5.2c |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Che-Wei Kuo, Xiaoyu Xiu, Wei Chen, Hong-Jheng Jhu, Ning Yan, Changyue Ma, Xianglin Wang Marta Karczewicz, Nan Hu, Hongtao Wang, Vadim Seregin | Email: | cheweikuo@kwai.com martak@qti.qualcomm.com nanh@qti.qualcomm.com hongtaow@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Kwai Inc., Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports results of EE2 Test 5.3, which is the combination of Test 5.1b and Test 5.2c. Compared to ECM-9.0 anchors, the BD-rate impact is summarized as below.

| Class A to E |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| Test 5.3 | AI |  |  |  |  |  |
|  | RA |  |  |  |  |  |
|  | LB |  |  |  |  |  |
| Class F |  | Y | U | V | EncT | DecT |
| Test 5.3 | AI |  |  |  |  |  |
|  | RA |  |  |  |  |  |
|  | LB |  |  |  |  |  |
| Class TGM |  | Y | U | V | EncT | DecT |
| Test 5.3 | AI |  |  |  |  |  |
|  | RA |  |  |  |  |  |
|  | LB |  |  |  |  |  |

# Introduction

At the JVET-AD meeting, JVET-AD0218 [1] and JVET-AD0220 [2] are proposed to improve the coding efficiency of the ECM and two contributions were included in EE2 as Test-5.1 and Test-5.2 for further evaluation. This contribution reports results of EE2 Test 5.3, which is the direct combination of Test 5.1b and Test 5.2c, as described as:

In 5.1b, temporal historical CCSAO offsets are stored for the usage of future frames. Also, the edge classifier is extended with additional edge/band combinations and the component used for edge classification can be selected from one of all three components.

In 5.2c, firstly, a fixed filter is also applied to samples before DBF. Secondly, the classifiers of the fixed filters are extended. For each 2x2 block, the variance of a surrounding window is calculated. The variance is then quantized by a scaling factor which is determined based on the activity value derived from a Laplacian classifier. With i=0, 1, let Ci denote the classifier from the classifier of i-th fixed filter in ECM. The new class index is a combination of Ci and the quantized variance. Thirdly, the second fixed filter is applied to outputs of the first fixed filter (instead of ALF input) and samples before DBF.

# Simulation results

The proposed methods were implemented on top of the reference software ECM-9.0 [3] and tested using the common test conditions as defined in [4]. Note runtime is not reliable and one point in class C results for low delay B is copied from anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.40% | -0.03% | -0.03% | 103.2% | 98.4% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.36% | -0.09% | -0.08% | 105.4% | 107.7% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.66% | -0.35% | -0.22% | 104.9% | 98.9% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.76% | -0.62% | -0.53% | 106.0% | 101.7% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.68% | -1.06% | -0.88% | #VALUE! | #VALUE! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.90% | -0.43% | -1.17% | 108.7% | 105.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# References

- C.-W. Kuo, X. Xiu, W. Chen, H.-J. Jhu, N. Yan, C. Ma, X. Wang, “AHG12: CCSAO with extended edge classifiers and history offsets”, JVET-AD0218, April 2023.

- M. Karczewicz, N. Hu, V. Seregin, “Non-EE2: Improved fixed filters for ALF”, JVET-AD0220, April 2023.

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, Jun., 2023.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).