| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0169-v1 |
| --- | --- |

| Title: | EE2-2.2/EE2-2.3/EE2-2.4: Extended IBC GPM, bi-predictive IBC, and IBC MBVD list derivation for natural and screen content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yoshitaka Kidani, Haruhisa Kato, Kei Kawamura (KDDI) 2Changyue Ma, Xiaoyu Xiu, Wei Chen, Hong-Jheng Jhu, Che-Wei Kuo, Ning Yan, Xianglin Wang (Kwai) 3Gleb Verba, Zhi Zhang, Pavel Nikitin, Han Huang, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | 1yo-kidani@kddi.com 2machangyue@kwai.com 3gverba@qti.qualcomm.com, zhizhang@qti.qualcomm.com |
| Source: | KDDI Corporation. (KDDI Research, Inc.), Kwai Inc., and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the experimental results of Test 2.2c (bi-predictive IBC GPM), Test 2.3 (bi-predictive IBC including Test 2.2c), and Test 2.4 (IBC MBVD including Test 2.3) in the context of Exploration Experiments 2 (JVET-AD2024).

Test 2.2c proposes a bi-predictive IBC GPM for screen content coding. Test 2.3 suggests a bi-predictive IBC, i.e., IBC BVP-merge and bi-predictive IBC merge for natural content coding. Test 2.4 presents a adaptive IBC MBVD list derivation.

The experimental results reported show the best performance in Test 2.4c over ECM-9.0 as follows:

AI:

Overall : {-0.16%, -0.20%, -0.13%, 101.3%, 100.1%}, Class F: {%, %, %, %, %}, Class TGM : {%, %, %, %, %};

RA:

Overall : {%, %, %, %, %}, Class F: {%, %, %, %, %}, Class TGM : {%, %, %, %, %};

# Introduction

In the 30th JVET meeting, an adaptation of bi-predictive IBC for camera captured and screen contents was investigated in JVET-AD0134 and JVET-AD0215. The technologies in the two proposals can be summarized in two aspects. The first aspect is an adaptation of bi-predictive IBC GPM only for screen content. The second aspect is that IBC BVP-merge and bi-predictive IBC merge for natural content. The two aspects are studied as Test 2.2c and Test 2.3.

In addition, in the 30th JVET meeting, IBC MBVD list derivation was also investigated in JVET-AD0396. The performance of IBC MBVD list derivation stand-alone and the combination with IBC GPM and bi-predictive IBC are studied as Test 2.4.

# Test description

## Summary of testing elements in Test 2.2, Test 2.3, and Test 2.4

Table 1 describes the summary of testing elements in Test 2.2c, Test 2.3, and Test 2.4 regarding the adaptation for natural content and screen content.

Table 1 Summary of testing elements in Test 2.2c, Test 2.3, and Test 2.4

| # | Camera captured content | Screen content |
| --- | --- | --- |
| 2.2a | N/A | Bi-predictive IBC GPM |
| 2.2c | N/A | Bi-predictive IBC GPM |
| 2.3a | IBC BVP-merge | N/A |
| 2.3b | 2.3a + bi-predictive IBC merge | N/A |
| 2.3c | 2.3a + bi-predictive IBC merge | 2.2c |
| 2.4a | IBC MBVD list derivation | IBC MBVD list derivation |
| 2.4b | 2.4a + 2.3c | 2.4a |
| 2.4c | 2.4a + 2.3c | 2.4a + 2.3c |

## Bi-predictive IBC GPM (Test 2.2)

Bi-predictive IBC GPM generates the prediction samples for both GPM partitions using the different IBCs. In the bi-predictive IBC GPM, the core IBC GPM design (e.g., 48 GPM modes, IBC merge candidate list) is kept the same as that of the uni-predictive IBC GPM in in ECM-9.0. Here, the uni-predictive IBC GPM generates the prediction samples for each GPM partition using the IBC and intra mode, respectively.

Only the signaling and blending schemes are slightly modified from the existing uni-predictive IBC GPM. For signaling, two flags are signaled to indicate the prediction modes of two partitions. For blending, GPM adaptive blending for inter GPM is also utilized in bi-predictive IBC GPM

The difference of Test 2.2c from Test 2.2a is the span of BV and intra prediction mode (IPM) storage, which is imported from Test 2.2b. Table 2 shows the detail as follows.

Table 2 The difference between Test 2.2a and Test 2.2c in span of BV and IPM storage

| # | GPM IBC/IBC | GPM IBC/intra |
| --- | --- | --- |
| 2.2a | BV: store BV of each partition IPM: store IPM of partition indicated by merge_gpm_idx0 by CU-level | BV: store BV of IBC partition by CU-level IPM: store IPM of intra partition by CU-level |
| 2.2c | BV: store BV of each partition IPM: store IPM referred by BV of each partition | BV: store BV of IBC partition as-is, while store zero-BV in intra partition IPM: store IPM of intra partition as-is, while store IPM referred by BV in IBC partition |

## IBC BVP-merge and bi-predictive IBC merge (Test 2.3)

IBC-BVP-merge, inspired by AMVP-merge, derives the two required BVs from IBC block vector prediction (BVP) and IBC merge. Two different indices for the IBC BVP and the IBC merge candidates are signalled from the encoder to the decoder, different from the AMVP-merge mode.

Bi-predictive IBC merge derives the two BVs from the existing IBC merge candidate list, utilizing two different indices. The two indices are signaled from the encoder to the decoder. The target of the bi-predictive IBC merge is not only IBC regular merge but also IBC MBVD and IBC GPM. Specifically, bi-predictive IBC MBVD is enabled in natural and screen content, while bi-predictive IBC GPM is enabled only in screen content, i.e., Test 2.3 includes the aspect of Test 2.2c.

## IBC MBVD list derivation (Test 2.4)

IBC MBVD list derivation allows adaptive BVD offsets along MBVD directions. Specifically, the MBVD candidates search is a two-step algorithm, which starts with checking template SAD costs of offsets added to BVP along each direction with the interval of M-pel. The second step of the search checks template SAD costs for intermediate candidates around the selected candidates. The candidates with the lowest TM cost are included into the final MBVD list.

# Experimental results

The proposed methods were implemented on top of the ECM-9.0 and tested according to the common test condition of AI and RA configuration as specified in the JVET-AD2017 and JVET-AD2024.

Experimental results of Test 2.2a, Test 2.2c, Test 2.3a, Test 2.3c, Test 2.4a, Test 2.4b, and Test 2.4c are summarized below.

Table 3 Results of Test 2.2a

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | 0.00% | 0.00% | 0.00% | 99.9% | 100.0% | 0.00% | 0.00% | 0.00% | 100.3% | 100.5% |
| A2 | 0.00% | 0.00% | 0.00% | 100.0% | 99.8% | 0.00% | 0.00% | 0.00% | 100.3% | 100.2% |
| B | 0.00% | 0.00% | 0.00% | 100.4% | 100.4% | 0.00% | 0.00% | 0.00% | 100.3% | 100.4% |
| C | 0.00% | 0.00% | 0.00% | 100.2% | 100.5% | 0.00% | 0.00% | 0.00% | 99.8% | 99.9% |
| E | 0.00% | 0.00% | 0.00% | 100.1% | 100.3% |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100.1% | 100.3% | 0.00% | 0.00% | 0.00% | 100.2% | 100.2% |
| D | 0.00% | 0.00% | 0.00% | 99.6% | 99.4% | 0.00% | 0.00% | 0.00% | 100.1% | 99.9% |
| F | -0.13% | -0.25% | -0.19% | 100.0% | 99.8% | -0.16% | -0.29% | -0.20% | 100.1% | 100.0% |
| TGM | -0.41% | -0.40% | -0.35% | 99.6% | 99.1% | -0.31% | -0.24% | -0.32% | 99.8% | 99.6% |

Table 4 Results of Test 2.2c

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | 0.00% | 0.00% | 0.00% | % | % | 0.00% | 0.00% | 0.00% | % | % |
| A2 | 0.00% | 0.00% | 0.00% | % | % | 0.00% | 0.00% | 0.00% | % | % |
| B | 0.00% | 0.00% | 0.00% | % | % | 0.00% | 0.00% | 0.00% | % | % |
| C | 0.00% | 0.00% | 0.00% | % | % | 0.00% | 0.00% | 0.00% | % | % |
| E | 0.00% | 0.00% | 0.00% | % | % | 0.00% | 0.00% | 0.00% | % | % |
| Overall | 0.00% | 0.00% | 0.00% | % | % | 0.00% | 0.00% | 0.00% | % | % |
| D | 0.00% | 0.00% | 0.00% | % | % | 0.00% | 0.00% | 0.00% | % | % |
| F | % | % | % | % | % | % | % | % | % | % |
| TGM | % | % | % | % | % | % | % | % | % | % |

Table 5 Results of Test 2.3a

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | 0.01% | -0.15% | 0.01% | 98.9% | 100.7% |  |  |  |  |  |
| A2 | -0.06% | -0.13% | -0.05% | 99.4% | 100.0% |  |  |  |  |  |
| B | -0.08% | -0.10% | 0.03% | 99.5% | 99.7% |  |  |  |  |  |
| C | 0.00% | -0.02% | -0.01% | 100.4% | 100.4% |  |  |  |  |  |
| E | -0.05% | 0.04% | 0.12% | 99.4% | 99.9% |  |  |  |  |  |
| Overall | -0.04% | -0.07% | 0.02% | 99.5% | 100.1% |  |  |  |  |  |
| D | 0.03% | 0.11% | 0.07% | 101.0% | 100.2% |  |  |  |  |  |
| F |  |  |  |  |  |  |  |  |  |  |
| TGM |  |  |  |  |  |  |  |  |  |  |

Table 6 Results of Test 2.3c, i.e., Test 2.3b + Test 2.2c, over ECM-9.0.

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.01% | -0.02% | -0.02% | 100.1% | 100.6% | 0.02% | -0.11% | 0.11% | 100.1% | 100.3% |
| A2 | -0.25% | -0.28% | -0.21% | 100.4% | 100.8% | -0.09% | -0.03% | -0.11% | 100.2% | 99.2% |
| B | -0.22% | -0.19% | -0.27% | 100.9% | 100.0% | -0.09% | -0.04% | -0.15% | 100.2% | 99.7% |
| C | -0.08% | -0.06% | -0.13% | 102.3% | 100.3% | 0.01% | -0.03% | 0.06% | 100.3% | 100.1% |
| E | -0.16% | -0.09% | -0.17% | 101.2% | 100.6% |  |  |  |  |  |
| Overall | -0.15% | -0.13% | -0.17% | 101.0% | 100.4% | -0.04% | -0.05% | -0.03% | 100.2% | 99.8% |
| D | -0.02% | -0.04% | -0.05% | 103.1% | 101.2% | -0.02% | -0.19% | 0.14% | 100.2% | 100.1% |
| F | % | % | % | % | % | % | % | % | % | % |
| TGM | % | % | % | % | % | % | % | % | % | % |

Table 7 Results of Test 2.4a over ECM-9.0.

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | % | % | % | % | % | % | % | % | % | % |
| A2 | % | % | % | % | % | % | % | % | % | % |
| B | % | % | % | % | % | % | % | % | % | % |
| C | % | % | % | % | % | % | % | % | % | % |
| E | % | % | % | % | % | % | % | % | % | % |
| Overall | % | % | % | % | % | % | % | % | % | % |
| D | % | % | % | % | % | % | % | % | % | % |
| F | % | % | % | % | % | % | % | % | % | % |
| TGM | % | % | % | % | % | % | % | % | % | % |

Table 8 Results of EE2-2.4b over ECM-9.0.

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | 0.00% | 0.03% | -0.02% | 100.6% | 100.1% |  |  |  |  |  |
| A2 | -0.25% | -0.31% | -0.21% | 100.7% | 99.5% |  |  |  |  |  |
| B | -0.23% | -0.28% | -0.24% | 101.1% | 99.7% |  |  |  |  |  |
| C | -0.09% | -0.17% | -0.08% | 102.3% | 101.2% |  |  |  |  |  |
| E | -0.18% | -0.18% | -0.04% | 101.4% | 99.9% |  |  |  |  |  |
| Overall | -0.16% | -0.20% | -0.13% | 101.3% | 100.1% |  |  |  |  |  |
| D | -0.03% | -0.14% | 0.07% | 101.2% | 99.3% |  |  |  |  |  |
| F | % | % | % | % | % | % | % | % | % | % |
| TGM | % | % | % | % | % | % | % | % | % | % |

Table 9 Results of EE2-2.4c over ECM-9.0.

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | 0.00% | 0.03% | -0.02% | 100.6% | 100.1% |  |  |  |  |  |
| A2 | -0.25% | -0.31% | -0.21% | 100.7% | 99.5% |  |  |  |  |  |
| B | -0.23% | -0.28% | -0.24% | 101.1% | 99.7% |  |  |  |  |  |
| C | -0.09% | -0.17% | -0.08% | 102.3% | 101.2% |  |  |  |  |  |
| E | -0.18% | -0.18% | -0.04% | 101.4% | 99.9% |  |  |  |  |  |
| Overall | -0.16% | -0.20% | -0.13% | 101.3% | 100.1% |  |  |  |  |  |
| D | -0.03% | -0.14% | 0.07% | 101.2% | 99.3% |  |  |  |  |  |
| F | % | % | % | % | % | % | % | % | % | % |
| TGM | % | % | % | % | % | % | % | % | % | % |

# References

- Y. Kidani, et al., “Non-EE2: Bi-predictive IBC for natural and screen content,” JVET-AD0134, April 2023.

- C. Ma, et al., “Non-EE2: Extension of IBC-GPM,” JVET-AD0215, April 2023.

- Z. Zhang, et al., “EE2-related: IBC MBVD for coding of camera captured content,” JVET-AD0396, April 2023.

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-9.0

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, April. 2023.

- V. Seregin, et al., “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AD2024, April 2023.

# Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).