| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0172-v2 |
| --- | --- |

| Title: | Update on multilayer coding support for VVenC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution reports an information update on multilayer coding support for VVenC, which is firstly reported in JVET-AB0086, where the initial support of multilayer coding was implemented on VVenC-1.4.0. The base software was updated to VVenC-1.6.1 and the latest results of VVenC-multilayer over VTM-20.2 is reported as follows:

Faster		BD-rate: {36.69%, 33.07%, 35.95%}, Encoder speedup:170.8x

Fast		BD-rate: {29.02%, 24.08%, 29.52%}, Encoder speedup: 94.2x

Medium	BD-rate: {10.59%, 10.54%, 10.37%}, Encoder speedup: 35.4x

Slow		BD-rate: {5.07%, 4.16%, 3.25%}, Encoder speedup: 10.5x

Slower		BD-rate: {0.52%, 0.29%, -0.98%}, Encoder speedup: 1.4x

The latest version of multilayer support on top of VVenC-1.6.1 is available at GitHub managed by NHK (https://github.com/nhkrd/vvenc.git).

Introduction

The optimized VVC software encoder VVenC is developed by Fraunhofer HHI (https://github.com/fraunhoferhhi/vvenc), where single layer encoding is supported. The implementation of VVenC with multilayer coding functionality (a.k.a. VVenC-multilayer) is firstly reported in JVET-AB0086 to accelerate the investigation of the optimum rate/quality settings for base and enhancement layers in spatial scalability scenario. The initial release of VVenC-multilayer is implemented on top of VVenC-1.4.0. This contribution reports the base software of VVenC-multilayer is updated to VVenC-1.6.1 and the software VVenC-multilayer is published in GitHub managed by NHK.

https://github.com/nhkrd/vvenc.git

There are two branches (feature_multilayer_v1.4.0 and feature_multilayer_v1.6.1) in the repositories, the base version of which is VVenC-1.4.0 and VVenC-1.6.1, respectively.

Usage of VVenC-multilayer

Multilayer coding functionality is ported from VVC Test Model (VTM) and encoding experiments can be conducted in a similar fashion to VTM. Build instruction for VVenC can be also used.

In addition to the default configuration files, two_layers.cfg is added in the cfg directory for multilayer coding configuration. Similar to VTM, per-layer configuration can be provided using “-l” options, i.e. the command line option following to -l0 is options for layer 0 and the command line option following -l1 is  options for layer 1. Sample command line for 2K/4K spatial scalable coding is as follows:

vvencFFapp -c two_layers.cfg -c randomaccess_{slower/slow/medium/fast/faster}.cfg -l0 -c sequence2K.cfg -l1 -c sequence4K.cfg -l0 -q 22 -l1 -q 22 -l1 --Level=6.1 --VerCollocatedChroma=1

Test setting

Test sequences

4K test sequences distributed by ITE (Institute of Image Information and Television Engineers, Japanese academic society) are used for the test. The color gamut of the sequences for the test is BT.2020 (SDR) and chroma location is Type 2. The detailed information on the test sequence can be obtained from https://www.ite.or.jp/content/test-materials/uhdtv_a/. 2K sequences generated by 4K sequences using SHVC down-sampling filter are coded as base layer and original 4K sequences are coded as enhancement layer.

The version of TAppDownConvert is SHM-12.4 and the command line arguments for the down conversion are as follows:

TAppDownConvertStatic 3840 2160 input.yuv 1920 1080 output.yuv 10 10 0 0 0 300 -phase 0 0 0 0

where the number of frames to be encoded is set equal to 300 in the experiments. The thumbnails of the test sequences used for the test is listed in Table 1.

Table 1. Thumbnails of the test sequences

| a03_TrainC | a04_Expressway |
| --- | --- |
| a06_Festival | a08_JapaneseMaple |
| a09_Maiko | a10_Umbrella |
| a11_LayeredKimono | a12_Railcar |

Anchor

To evaluate the coding efficiency and encoding runtime of VVenC-multilayer-1.6.1, the VTM anchor is generated using VTM-20.2. For QP settings for base and enhancement layers, {(22, 22), (27, 27), (32, 32), (37, 37)} and {(22, 24), (27, 29), (32, 34), (37, 39)} are used.

An example of command line arguments are as follows:

EncoderAppStatic -c encoder_random_access_vtm.cfg -c multi_layer/two_layers.cfg -l0 -c 2K_sequence.cfg -l1 -c 4K_sequence.cfg -l0 -q 22 -l1 -q 24 --VerCollocatedChroma=1

Tested configuration

A total of 5 presets (slower, slow, medium, fast, and faster) are defined as is the case with VVenC-1.6.1. Coding tool selection and partitioning constraints for those presents are not changed from the original VVenC definition.

Simulation results

In the simulation, the 2K sequence generated by SHVC down sampling filter and the original 4K sequence are encoded using both VTM-20.2 and VVenC-multilayer-1.6.1. The BD-rate values of {Y, Cb, Cr} components of VVenC-multilayer-1.6.1 with different presets over VTM-20.2 are calculated from PSNR values of the enhancement layer and total bitrate. The encoder runtime ratios against VTM-20.2 are calculated using a single thread coding. Table 2 and 3 show the BD-rate results of the case where QP for enhancement layer is equal to QP for base layer, and the case where QP for enhancement layer is greater that QP for base layer by 2, respectively.

Table 2. BD-rate of VVenC-multilayer over VTM-16.0 multilayer coding
calculated from PSNR of EL and total bitrate (QPE=QPB)

| Presets | BD-rate Y | BD-rate U | BD-rate V | EncTime | Speedup |
| --- | --- | --- | --- | --- | --- |
| slower | 0.52% | 0.29% | -0.98% | 69.66% | 1.4 |
| slow | 5.07% | 4.16% | 3.25% | 9.48% | 10.5 |
| medium | 10.59% | 10.54% | 10.37% | 2.83% | 35.4 |
| fast | 24.66% | 24.08% | 29.52% | 1.06% | 94.2 |
| faster | 36.69% | 33.07% | 35.95% | 0.59% | 170.8 |

Table 3. BD-rate of VVenC-multilayer over VTM-16.0 multilayer coding
calculated from PSNR of EL and total bitrate (QPE=QPB+2)

| Presets | BD-rate Y | BD-rate U | BD-rate V | EncTime | Speedup |
| --- | --- | --- | --- | --- | --- |
| slower | 1.05% | -0.43% | -0.55% | 71.37% | 1.4 |
| slow | 5.59% | 3.92% | 3.88% | 9.45% | 10.6 |
| medium | 11.28% | 10.83% | 11.22% | 2.97% | 33.6 |
| fast | 24.51% | 25.69% | 30.77% | 1.19% | 84.0 |
| faster | 37.34% | 34.71% | 39.96% | 0.66% | 150.6 |

It is confirmed that encoder speedups between 1.4x to 170x can be achieved. The runtime and BD-rate performance for different presets are illustrated in Figure 1. To compare the speedup tendency against the previously reported VVenC-multilayer-1.4.0, The runtime and BD-rate performance of VVenC-multilayer-1.4.0 over VTM-16.0 is also illustrated in the figure. From the figure 1, it is confirmed that the encoding speedup tendency is mostly preserved compared with the original VVenC-1.6.1.

Figure 1. Run time vs BD-rate for different presets (QPE=QPB)

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).