| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0173 |
| --- | --- |

| Title: | [AHG9] Clarification and improvements of signaling of NNPF update |  |  |
| --- | --- | --- | --- |
| Status: | Input contribution to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Youngkwon Lim 665 Clyde Avenue Mountain View, CA 94043 USA | Tel: Email: | young.L@samsung.com |
| Source: | Samsung Research America |  |  |

_____________________________

# Abstract

Current design of NNPFC SEI messages in Draft v4 only allows the updates can be applied individually to base NNPF. This can be a serious limitation to flexible use of NNPF. In addition, there are some confusing definition of flags of NNPFA SEI messages several combinations of them can be confusing. This contribution proposes several updates to NNPFA SEI messages to improve flexible control of NNPF updates.

# Limitation of NNPFC SEI message and NNPFA SEI message for signaling updates to NNPF

Current design of NNPFC SEI message in Draft v4 (JVET-AD2006, April 2023) includes the flag to indicate that the message includes updates to the existing NNPF, nnpfc_base_flag. It indicates whether a message includes initial configuration information of NNPF, base NNPF or updates relative to the base NNPF. However, there is strong limitation of update signaling capability as updates cannot be cumulative but rather each update is applied on the base NNPF.

The NNPFA SEI message includes several flags which can represent various combinations to indicate how to activate or cancel the updates of NNPF, nnpfa_cancel_flag, nnpfa_target_base_flag and nnpfa_persistence_flag. However, there are some conflict definitions and some cases whose definition is quite confusing.

•	nnpfa_cancel_flag equal to 1 indicate activation of the NNPF for the current picture only or the current picture and all subsequent pictures depending on the value of nnpfa_persistence_flag. However, definition of nnfpc_base_flag of NNPFC SEI message indicates that the SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, which means that nnpfa_persistence_flag cannot be 0.

•	The target NNPF of the NNPFA message with nnpfa_cancel_flag equal to 1 and nnpfa_target_base_flag equal to 0 shall not the NNPF which is not active.

•	The decoder behavior is not clear when there are one or more NNPFC message with updates but only one NNPFA message with nnpfa_cancel_flag equal to 0 and nnpfa_target_base_flag equal to 1as NNPFC message with updates cannot be repeated and NNPF updates are not cumulative

•	Updates of NNPF can be explicitly activated but it cannot be deactivated. De-activation can only be applied to entire NNPF.

# Proposed improvement for signaling updates to NNPF

Here are the list of proposed updates of NNPFA SEI messages.

- Proposal 1. 
NNPFA message with nnpfa_cancel_flag equal to 0 and nnpfa_target_base_flag equal to 0 shall only be sent to the NNPF which is not active. The NNPFA messages with nnpfa_cancel_flag equal to 0 and nnpfa_target_base_flag equal to 1 shall be ignored if the NNPF whose nnpfc_id equal to nnpfa_target_id of the NNPFA message is not active when the message is received by the receiver.

- Proposal 2. 
NNPFA message nnpfa_cancel_flag equal to 0 and nnpfa_target_base_flag equal to 1 can be sent to the receiver after more than one NNPFC messages with nnpf_base_flag equal to 0. Then all updates from the NNPFC SEI messages with nnpf_base_flag equal to 0 before the NNPFA message after the NNPFA message with nnpf_base_flag equal to 1 is sent are cumulatively applied to the base NNPF.

- Proposal 3. 
Target of NNPFA message with nnpfa_cancel_flag is decided by the value of nnpfa_target_base_flag. If nnpfa_target_base_flag is equal to 0 then the target NNPF is completely deactivated and is no longer used until it is activated by another NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 0. If nnpfa_target_base_flag is equal to 1 then the updates applied to the base NNPF is deactivated, i.e. the base NNPF is applied to the pictures following the current NNPFA SEI message in decoding order.

# Proposed updates to the specification text

Please see attached file.

# Patent rights declaration(s)

Samsung Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).