| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0176-v1 |
| --- | --- |

| Title: | EE2-related: Enhancements on CCRM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This proposal presents the results of a multi-model CCRM mode and a CCRM merge mode. In EE2-3.1a, a single-model CCRM is investigated. In this contribution, two methods are presented to further improve the coding efficiency of CCRM. In method #1, a multi-model CCRM mode is proposed. Samples are separated into two groups and each group can apply its own CCRM model. There is no flag signalling involved in the proposed method, and the final CCRM mode is implicitly determined by comparing decoder derived costs of the two CCRM modes with single or multiple models. In method #2, a CCRM merge mode is proposed, wherein the CCRM model is inherited from a candidate in the CCRM merge list. Simulation results based on ECM-9.0 are reported as below:

Method #1 : RA {-0.xx%, -0.xx%, -0.xx%, 10x%, 10x%}; LDB {}

Method #2 : RA {-0.xx%, -0.xx%, -0.xx%, 10x%, 10x%}; LDB {}

Simulation results versus EE2-3.1a are listed as below :

Method #1 : RA {}; LDB {}

Method #2 : RA {}; LDB {}

# Introduction

At the JVET-AD meeting, a cross-component residual model (CCRM) [1] based chroma prediction is proposed to improve the compression efficiency of inter slices in ECM. With CCRM, chroma samples of an inter or IBC coded block are predicted from reconstructed luma samples. The CCRM model consists of 6 spatial non-downsampled luma samples, a nonlinear term, and a bias term, with 8 coefficients in total. The CCRM model coefficients are derived from the correlation between the reference luma samples and reference chroma samples. A TU level flag is signalled to indicate this mode. The encoder performs an RD decision in the transform selection loop for the chroma components when luma Cbf is non-zero and the CU’s predMode is either MODE_INTER or MODE_IBC. The CCRM was included into EE2 for further investigation.

# Proposed method

## Method #1

In this method, a multi-model CCRM is proposed, wherein samples in a block are classified into two groups and each group derives an individual CCRM model. The threshold for separating luma and chroma sample pairs is computed from the average of the non-downsampled luma sample values in the reference block.

The proposed method takes use of an implicit CCRM mode derivation method, thus there is no need to signal extra bits. Specifically, to determine whether to apply single-model CCRM or multi-model CCRM, a decoded derived cost is calculated by applying the CCRM model to the reference luma samples. The SAD between the predicted reference chroma sample values and the true reference chroma sample values is treated as the cost. Thereby, the CCRM mode with a lower cost is finally selected for a block.

Furthermore, as the rationale of CCRM is highly dependent on the luma residues, the CCRM is disabled when the luma coefficients are small.

## Method #2

In this method, a CCRM merge mode is proposed. A CCRM merge list is constructed from previous CCRM-coded blocks. Instead of calculating a CCRM model from samples in the reference area on-the-fly, the CCRM model can be inherited from a CCRM merge candidate, and directly applied to the current luma samples to generate predicted chroma samples. The first available CCRM merge candidate is used without index signalling.

A flag is signalled conditioned on the TU level CCRM flag, to indicate whether the current block is coded with regular CCRM or CCRM merge mode.

# Experimental results

The proposed method is implemented on top of ECM-9.0 [2] and tested following the common test conditions [3]. The simulation results of the proposed method #1 and method #2 are summarized in Table 1-2.

Table 1: Coding performance of the proposed method #1

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  | Over EE2-3.1a |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | -1.53% | -1.24% |  |  | -0.06% | -0.33% | -0.09% |  |  |
| Class C | 0.04% | -0.60% | -0.61% |  |  | -0.04% | 0.30% | 0.11% |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.03% | -0.77% | -0.52% |  |  | -0.06% | 0.12% | 0.03% |  |  |
| Class F | -0.01% | -0.39% | -0.40% |  |  | -0.06% | 0.23% | 0.26% |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  | Over EE2-3.1a |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | 0.04% | -1.67% | -2.32% |  |  |  |  |  |  |  |
| Class E | -0.05% | -1.15% | -1.31% |  |  | -0.15% | -0.72% | 0.05% |  |  |
| Overall (Ref) |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F | -0.02% | -3.59% | -3.97% |  |  |  |  |  |  |  |

Table 2: Coding performance of the proposed method #2

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  | Over EE2-3.1a |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | 0.05% | -0.83% | -0.84% |  |  | -0.03% | 0.06% | -0.13% |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.13% | -0.74% | -0.74% |  |  | 0.04% | 0.16% | -0.19% |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  | Over EE2-3.1a |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.05% | -2.38% | -2.78% |  |  | -0.03% | 0.42% | -0.05% |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

# Conclusions

The proposed method can achieve decent BD-rate gains with minor runtime change. It is recommended to include the proposed methods to the next EE2 for a further investigation.

# References

- P. Astola, J. Lainema, “AHG12: Cross-component residual model (CCRM) for inter prediction”, JVET-AD0108, Feb. 2023.

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).