| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0181-v1 |
| --- | --- |

| Title: | Scaling window support for VTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution is a follow-up contribution of JVET-AD0101 on scaling window support for VTM. Scaling window functionality is reported to be useful for content layering application which is one of the effective use cases of multilayer coding. However, the latest version of VTM does not yet support scaling window. To enable the evaluation of scaling windows functionality, this contribution proposes to integrate implementation of scaling window into VTM. The merge request was submitted to VTM repository as a draft implementation as MR!2581.

Introduction

One of the efficient use cases using multilayer coding is content layering, where main content is coded as base layer and additional overlaid content is coded as enhancement layer. In the previous Antalya meeting, it was proposed to include bitstreams with scaling window enabled into the additional conformance test for multilayer configurations. Potential use case using scaling window is, for instance, sign language content, where main content is down-scaled and sign language interpreter is overlaid. For this use case, scaling window with multilayer coding works quite efficiently.

Figure 1. Typical use case for scaling window with multilayer coding

Implementation of scaling window to VTM

Scaling window functionality is implemented on top of master branch of VTM. Since parameters of scaling window are associated with picture parameter set (PPS), scaling window setting can be time-varying in VVC specifications. However, the implementation proposed in this contribution only supports per-layer-basis fixed scaling window for the entire sequence. Command line parameters for scaling window introduced in accordance with those of conformance window are listed in Table 1.

Table 1 Command line parameters for scaling window support

| Command line option | Description |
| --- | --- |
| --ScalingWindow | Scaling window mode (0: disabled, 1: enabled) |
| --ScalWinLeft | Left offset for scaling window in luma sample (when disabled, set equal to 0) |
| --ScalWinRight | Right offset for scaling window (when disabled, set equal to 0) |
| --ScalWinTop | Top offset for scaling window (when disabled, set equal to 0) |
| --ScalWinBottom | Bottom offset for scaling window (when disabled, set equal to 0) |

Sample command line for generating conformance bitstream for PPSScalWin_A_NHK, which is one of the added conformance tests for scaling window, is as follows:

EncoderAppStatic -c encoder_randomaccess_vtm.cfg -c two_layers.cfg -c RaceHorses.cfg -l1 -i RaceHorses_312x180_30_w_Lshape_margin.yuv -l1 --ScalingWindow=1 -l1 --ScalWinLeft=0 -l1 --ScalWinRight=208 -l1 --ScalWinTop=0 -l1 --ScalWinBottom=120 -l1 --Level=3 -f 9 -q 37

where RaceHorses_312x180_30_w_Lshape_margin.yuv is generated by SHVC down converter and padding operation. Specifically, SHVC down converter is firstly applied using the following command:

TAppDownConvertStatic 416 240 RaceHorses_416x240_30.yuv 312 180 RaceHorses_312x180_30_w_Lshape_margin.yuv,

and then the padding operation is applied to fill the L-shape area by grey.

Figure 2. Scaling window parameters for PPSScalWin_A_NHK

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).