| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0182-v1 |
| --- | --- |

| Title: | AHG9: SEI message extension of VVC for computer-generated hologram use |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keisuke Nonaka, Ryota Koiso, Hiroki Kojima, Kei Kawamura, Haruhisa Kato 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 70-3508-5821 ki-nonaka@kddi.com ry-koiso@kddi.com hk-kojima@kddi.com ki-kawamura@kddi.com ha-katou@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes an extension of SEI message on VVC for realizing computer-generated holography use. Transmission of object wave, wich is the intermidiate data representation of computer-generatad hologram, is suitable for the practical use in terms of high computational complexity on object wave generation and the data flexibility of object wave. Thus, we focus on how to transmit object wave by using the existing video codec. In this document, we show the flow of the object wave transmission for computer-generated hologram use and the results of preliminary study of the flow by using the exsisting video codec. Our proposed extension provides the ability to compress the object wave and convert object wave into computer-generated holograms tailored to the playback device, enabling a wide range of use of computer-generated holograms.

Introduction

Computer holography is a promising 3D display technology that realizes highly realistic 3D video experiences by taking a 3D model as input and recording its light propagation information in a digital fringe pattern called a computer-generated hologram (CGH) as shown in Fig. 1. An example of CGH is shown on the web site [1]. This 3D display technology can deliver visual experiences without the burden on the user, e.g. virtual reality sickness, and are not inferior to the real world. Because of these characteristics, computer holography is desired for practical use which requires realistic virtual visual experiences, such as remote meeting, remote coaching and remote surgery.

To realize CGH applications over network, the following problems should be addressed:

- The CGH calculation requires a considerable amount of computation and computing resources, and it is hard to generate CGH by using received 3D model data on a low-performance user terminal such as a mobile device.

- The CGH generation process requires information on the playback device (optical system), making it impossible to broadcast one data to multiple devices.

Sending object wave, which is the intermediate data for CGH calculations, instead of CGH overcomes the above restrictions. The object wave can be adjust to the playback device optics. Thus, transmitting object wave to playback devices solves the problem of data flexibility in a broadcast distribution described in the problem 2. In addition, the computational complexity of conversion from object wave to CGH is much lower than the one of object wave from a 3D model.

The object wave is complex data and consists of two parts: a real part and an imaginary part in the 2D plane. Each part of the data has similar structure to video format. Thus, existing video codecs can be applied by several processing shown in Section 2. In addition, object wave adjustment to the playback device optics can be carried out by using information that used in the object wave calculation. These indicates some supporting information (SEI message) needs to be sent to the playback devices for realization of object wave transmission using existing video codecs.

This document proposes the framework and the SEI message extension of the existing video codec, VVC, for object wave compression that realizes the future CGH use.

Figure 1: Flow for generation and reproduction of computer holography

Holography Overview

Workflow in CGH playback with object wave generation and compression

The workflow of CGH playback, through object wave generation and compression, is shown in Fig. 2 and follows:

- Object wave calculation (Fig. 2 (i)): Object wave is calculated as the light from a 3D model object to a two-dimensional plane by using light wave propagation techniques. The two-dimensional light wave distribution on the target plane in 3D space is obtained as light amplitude and phase distributions. In the usual calculation process, the amplitude and phase are converted to complex numbers.

- Mapping from float to integer (Fig. 2 (ii)): The complex representation of object wave is stored as a floating-point number of the real and imaginary part for each sampling point, respectively. In other words, object wave cannot be input to video encoders as it is. Therefore, object wave must be converted to integers to available video format. As mentioned above, an object wave consists of a pair of two-dimensional distribution data of real and imaginary parts. To treat the pair of the data as a single bitstream, these are packed into one frame (Fig. 2 (b)).

- Encoding and decoding with video codec (Fig. 2 (iii)): The integerized object wave is input to the encoder, and an encoded bit stream is generated. Then, the bitstream is sent to the receiver and the integerized object wave is decoded from the bitstream.

- Inverse mapping from integer to float and unpacking (Fig. 2 (iv)): By using the decoded object wave, we perform an inverse transformation from the integerized object wave to the original floating-point representation. In addition, we unpack the real and imaginary parts of the packed data for the following operations.

- Adjustment of the object wave to match the optics of the receiver device (Fig. 2 (v)): The resolution and the inverse-mapped object wave is adjusted according to the pixel pitch of the display and the wavelength of the reference light on the playback devices by using the metadata included in SEI.

- Conversion from object wave to hologram: A CGH is calculated as the interference fringe pattern between the inverse-mapped object wave and the reference light defined by the optics of the playback device. The CGH is displayed by the light modulator on the playback device, and the 3D image is reproduced by irradiating the reference light.

Figure 2: Flow of CGH playback through object wave generation and compression

Preliminary study of object wave compression

Object wave consists of two-dimensional distribution of real and imaginary parts on complex numbers, which can be treated as a type of image data, e.g., an image with 8 bits at each sampling point. Thus, the object wave can be compressed by VVC. Here, we show the results of a preliminary study on object wave compression by VVC. The detail of this study is as follows.

We use two 3D models named “butterfly” and “checker_ball” as the input for object wave calculation (see Fig. 3). We use WaveField Tools, which is distributed on the website [2] for the object wave calculation, and the calculated object wave is shown in Figs. 4 and 5. Note that the original object wave is represented as the floating-point number for each sampling point, but the figures are the images that have already been integerized by a non-linear function for visualization and making it input data of VVC. We use the following function for the non-linear transformation called as μ-low algorithm, which is defined in ITU-T G. 711 and is used for companding operation in digital audio transmission.

f=sgnxlog1+μxlog1+μ,#⋯1

where, the variable x indicates the input floating-point number of object wave that range is normalized from -1.0 to 1.0, and the variable μ indicates a hyperparameter that determines the strength of the companding. The function sgnx means sign function. Output f is finally mapped onto the integer value determined by the bit depth of the image, e.g., from 0 to 255 in the case of an 8-bit image. Figure 6. shows the graph of this function in the case of μ=255, which is used in this experiment. The inverse function is also defined as follows:

x=sgnf1μ1+μf-1.#⋯2

As Figs. 4 and 5 show, the object waves look like gray images, but these have fine patterns according to the contents. In addition, the object wave has RGB channels for each of the real and imaginary parts as in standard video formats, but Figs. 4 and 5 only show the result of the blue color channel.

- butterfly                    (b) checker_ball

Figure 3: Input 3D model for the preliminary study

Figure 4: object wave of the blue color channel of the 3D model (butterfly)

Figure 5: Object-wave of the blue color channel of the 3D model (checker_ball)

- function from float-point to integer (Eq. (1))  (b) function from integer to float-point (Eq. (2))

Figure 6: mapping function for the non-linear transformation

We compressed the target (integerized) object waves by using VTM-20.0 intra mode with several QPs (QP=22, 27, 32, 37, 42). The object waves have 8-bit depth with RGB color (4:4:4) format and 4K x 4K resolution for each real part and imaginary part, respectively, and these real and imaginary parts are finally packed into 4K x 8K format. Since this is the preliminary study to analyze the basic behavior of object wave compression, we compress only one frame of object wave for each content.

The simulation of reconstructed images from the object waves are shown in Figs. 7 and 8. These images are rendered from three viewpoints, in the front of the object (Center), position 10° off-center to the left (Left 10°), and position 10° off-center to the right (Right 10°). We also use WaveField Tools for the simulation. The images in Fig. 7 show that the object’s appearance changes depending on the viewpoint. In addition, the images in Fig. 8 also show that higher codec QP results in overall light scattering (especially for the blue light) and slight quality degradation in the reconstructed images.

Figure 7: Reconstructed simulation image from the object wave without compression (butterfly). Descriptions at the bottom of the image show the viewpoint position.

Figure 8: Reconstructed simulation image from the object wave with compression (butterfly). Descriptions at the bottom of the image show the viewpoint position.

We also evaluate the rate-distortion (RD) curve as the objective evaluation. We carried out two experiments measuring the distortion in two data domain; Experiment 1 calculates PSNR between the input object wave and the decoded object wave, and Experiment 2 calculates PSNR between the reconstructed images from the corresponding object waves, as shown in Fig. 9. Figure 10 shows the RD curve of Experiment 1, and Fig. 11 shows the RD curve of Experiment 2. The PSNRs shown in these figures are calculated by averaging the PSNR of each color channel. We can see that both the object wave and the reconstructed image show the PSNR degradation with decreasing bit rate, just like the existing video.

Figure 9: Outline of RD curve calculation

Figure 10: RD curve of Experiment 1 (coding rate vs PSNR of the object wave). The numbers described near the plots indicate QP values

Figure 11: RD curve of Experiment 2 (coding rate vs PSNR of the reconstructed image). The numbers described near the plots indicate QP values

Syntax and semantics of our proposed SEI message

Our SEI message provides the necessary information to realize the flow of CGH playback shown in Section 2.1. The syntax and semantics are described as follows:

Table 1 Syntax of object wave parameters

| object_wave_parameters( payloadSize ) { | Descriptor |
| --- | --- |
| mapping_type | u(4) |
| if( mapping_type == 0 ) { |  |
| real_max | se(v) |
| real_min | se(v) |
| imaginary_max | se(v) |
| imaginary_min | se(v) |
| } |  |
| else if( mapping_type == 1 ) { |  |
| real_max | se(v) |
| real_min | se(v) |
| imaginary_max | se(v) |
| imaginary_min | se(v) |
| non_linear_param | ue(v) |
| } |  |
| pixel_pitch_x | ue(v) |
| pixel_pitch_y | ue(v) |
| wavelength_r | ue(v) |
| wavelength_g | ue(v) |
| wavelength_b | ue(v) |
| object_depth | ue(v) |
| } |  |

mapping_type equal to 0 specifies that object light expressed in floating-point is converted into integer by linear transformation and to convert it into video format. If mapping_type is equal to 1, the object light expressed in floating-point is converted into integer by a nonlinear transformation using the function that is used in the preliminary study.

Table 2 Mapping functions corresponding to the parameter, mapping_type

| Value | Mapping type | Informative Remark |
| --- | --- | --- |
| 0 | Linear |  |
| 1 | 𝑥=sgn(𝑓)1/𝜇 {(1+𝜇)^¦𝑓¦ −1} | ITU-T G.711 |
| 2 … 16 | Reserved | For future use by ITU-T ¦ ISO/IEC |

real_max specifies the maximum value of the real part of the object wave represented in floating point before integerization. This value and real_min are used to convert the decoded integer object wave value to floating-point number.

real_min specifies the minimum value of the real part of the object wave represented in floating point before integerization. This value and real_max are used to convert the decoded integer object wave value to floating-point number.

imaginary_max specifies the maximum value of the imaginary part of the object wave represented in floating point before integerization. This value and imaginary_min are used to convert the decoded integer object wave value to floating-point number.

imaginary_min specifies the minimum value of the imaginary part of the object wave represented in floating point before integerization. This value and imaginary_max are used to convert the decoded integer object wave value to floating-point number.

non_linear_param specifies a parameter μ, which is used in the nonlinear transformation defined by mapping_type

pixel_pitch_x specifies pixel pitch along x-axis used for object wave calculation on the encoder side. On the decoder side, this value is used to compensate for pixel pitch mismatch between the calculation and the pixel pitch of the display on the playback device.

pixel_pitch_y specifies pixel pitch along y-axis used for object wave calculation on the encoder side. On the decoder side, this value is used to compensate for pixel pitch mismatch between the calculation and the pixel pitch of the display on the playback device.

wavelength_r specifies wavelength of red color used for object wave calculation on the encoder side. On the decoder side, this value and object_depth are used to compensate for wavelength mismatch between the calculation and the reference light on the playback device.

wavelength_g specifies wavelength of green color used for object wave calculation on the encoder side. On the decoder side, this value and object_depth are used to compensate for wavelength mismatch between the calculation and the reference light on the playback device.

wavelength_b specifies wavelength of blue color used for object wave calculation on the encoder side. On the decoder side, this value and object_depth are used to compensate for wavelength mismatch between the calculation and the reference light on the playback device.

object_depth specifies object_depth used for object wave calculation on the encoder side. On the decoder side, this value and the wavelength (wavelength_r, wavelength_g, and wavelength_b) are used to compensate for wavelength mismatch between the calculation and the reference light on the playback device.

We propose the following timeline for standardization.

- July 2024: WD

- October 2024: CD

- October 2025: FDIS

Conclusion

In this contribution, we propose the SEI message extension on existing video codec, VVC. This extension provides the ability to compress the object wave and convert object wave into computer-generated holograms tailored to the playback devices, enabling a wide range of use of computer-generated holograms.

We propose the following timeline for standardization.

- July 2024: WD

- October 2024: CD

- October 2025: FDIS

References

[1] KDDI Research and Kansai University, “Achieving color animation of holography while maintaining large size and a wide viewing angle, “ KDDI Research, https://www.kddi-research.jp/english/newsrelease/2022/050901.html, accessed on June 2023.

[2] Kyoji Matsushima, “WaveField Tools, “ Kansai University, http://www.laser.ee.kansai-u.ac.jp/wavefieldtools/, accessed on June 2023.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).