| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0188 |
| --- | --- |

| Title: | Crosscheck of JVET-AE0097 (AHG12: On the cross-component merge mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Kai Zhang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1(858)-380-8522 zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of JVET-AE0097. The simulation results reportedly match those provided by the proponent.

# Verification

In JVET-AE0097, it is proposed to remove the template based offset update process for the cross-component merge candidate.

Tests are performed with the provided source code based on ECM-9.0 summarized in Table 1. The running time is not reliable.

Table 1: Results on ECM-9.0

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | -0.08% | 87% | 77% | -0.06% | 0.06% | -0.56% | 81% | 79% |
| Class A2 | 0.00% | -0.20% | 0.06% | 85% | 77% | -0.02% | -0.25% | -0.12% | 82% | 80% |
| Class B | 0.01% | 0.09% | -0.04% | 88% | 79% | -0.01% | 0.19% | -0.19% | 87% | 84% |
| Class C | -0.02% | 0.01% | 0.07% | 93% | 84% | -0.01% | -0.09% | -0.06% | 93% | 97% |
| Class E | -0.02% | 0.04% | -0.02% | 89% | 84% |  |  |  |  |  |
| Overall | -0.01% | 0.01% | 0.00% | 89% | 80% | -0.02% | 0.00% | -0.22% | 86% | 85% |
| Class D | -0.01% | 0.01% | 0.07% | 95% | 110% | 0.01% | -0.28% | -0.22% | 95% | 108% |
| Class F | 0.01% | -0.10% | -0.14% | 91% | 100% | -0.03% | -0.21% | -0.20% | 91% | 92% |