| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0196-v1 |
| --- | --- |

| Title: | EE2-3.4: OBMC with HPel flag and BCW weights |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Antoine Robert Fabrice Le Léannec Franck Galpin Tangi Poirier Ya Chen | Email: | antoine.robert@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2 test 3.4 that proposes to use the HPel flag and the BCW weights of the neighboring and current CUs in the OBMC process.

The reported simulation results of the proposed method on top of the ECM-9.0 show gain of {0.01%, -0.02%, -0.02%} and {0.01%, 0.26%, 0.06%} for YUV in RA and LDB respectively with an unchanged complexity of the encoder and decoder.

It has to be noticed that the drop in gain on ECM-9.0 compared to the initial proposal JVET-AD0183 is due to the implementation in ECM9.0 of the adopted contribution JVET-AD0213 on improvements of LIC that turns out to incorporate most of the aspects of this proposal.

# Introduction

Prior to the OBMC process, the prediction of the current CU has already been calculated by using all the motion information, including the HPel flag and BCW weight. While the motion compensation of the neighboring blocks and subblocks performed by OBMC only considers the motion vectors and reference frames.

# Proposal

This EE2 test 3.4 [1] aims at using the HPel flags and the BCW weights of the neighboring blocks and subblocks into the OBMC process as both BCW and AMVR indexes can modify the predicted samples in the OBMC motion compensation. It is then proposed to incorporate them into the OBMC process to better fit the real predicted signal.

When processing the top and left borders, the neighboring BCW weights and HPel flags are used in addition to the motion vectors and reference frames. The neighboring HPel flag allows setting the correct AMVR index. For the internal subblock boundaries, the BCW and AMVR indexes of the current CU are also used.

# Experimental results

The proposal is implemented on top of the ECM-9.0 software [2] under the CTC conditions [3]. The simulation results in RA and LDB are depicted in Table 1.

Table 1: EE2 test 3.4 on top of ECM-9.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.07% | 0.10% | 99% | 101% |
| Class A2 | 0.01% | -0.05% | -0.09% | 100% | 101% |
| Class B | -0.01% | -0.03% | -0.02% | 101% | 100% |
| Class C | 0.02% | -0.07% | -0.07% | 100% | 100% |
| Overall | 0.01% | -0.02% | -0.02% | 100% | 100% |
| Class D | 0.01% | -0.16% | -0.13% | 100% | 99% |
| Class F (optional) | -0.01% | -0.07% | -0.07% | 101% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.03% | 0.20% | 0.03% | 100% | 99% |
| Class C | 0.00% | 0.29% | 0.00% | 101% | 101% |
| Class E | 0.00% | 0.31% | 0.20% | 100% | 100% |
| Overall | 0.01% | 0.26% | 0.06% | 101% | 100% |
| Class D | -0.08% | 0.51% | 0.37% | 100% | 99% |
| Class F (optional) | -0.12% | 0.29% | -0.15% | 100% | 101% |

It has to be noticed that the drop in gain on ECM-9.0 compared to the initial proposal JVET-AD0183 [4] is due to the implementation in ECM9.0 of the adopted contribution JVET-AD0213 [5] on improvements of LIC that turns out to incorporate most of the aspects of this proposal.

# References

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang, “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AD2024, April 2023.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, January. 2022.

- A. Robert, F. Galpin, T. Poirier, Y. Chen, “Non-EE2: SIF flag and BCW weight usage in OBMC”, JVET-AD0183, April 2023.

- X. Xiu, N. Yan, C. Ma Hong-J. Jhu, C.-W. Kuo, W. Chen, X. Wang, “EE2-Test2.7: Improvements on local illumination compensation”, JVET-AD0213, April 2023.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).