| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0226-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AE0100 (EE2-2.1: Block vector guided CCCM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mohsen Abdoli | Email | mohsen.abdoli@b-com.com |
| Source: | IRT b-com, Cesson-Sévigné, France |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results for JVET-AE0100. The cross-check has been done for AI and RA configurations of the single test proposed in JVET-AE0100 contribution. Simulations were performed using the JVET CTC. As of the first version of this document, the reported rate-distortion results are identical to those provided by the proponents. The encoding and decoding run-times have also been cross-checked. The source code of contribution JVET-AE0100 has been analyzed and run according to the ECM common test conditions [1]. Runtimes are also cross-checked.

# Introduction

The method in JVET-AE0100 proposes a new convolutional cross-component model which uses block vectors of the co-located IBC or intraTMP coded luma blocks to identify the reference area for deriving the CCCM model. The proposed block vector guided CCCM (BVG-CCCM) method uses an 11-tap filter for cross-component prediction. The mode is signalled with a context coded flag which is conditioned on co-located luma block’s mode. Two tests are conducted in this EE.

## Results of Test 2.1a

In Test 2.1a, the block vectors of IBC coded blocks from co-located luma are used for determining the CCCM reference area.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.00% | 101% | 100% |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.10% | -0.13% | 101% | 100% |
| Class C | 0.00% | -0.07% | -0.02% | 100% | 100% |
| Class E | 0.01% | -0.02% | 0.09% | 100% | 100% |
| Overall |  |  |  |  |  |
| Class D | 0.01% | 0.04% | 0.14% | 100% | 100% |
| Class F | 0.01% | -0.40% | -0.13% | 100% | 99% |
| Class TGM | -0.57% | -1.03% | -0.94% | 100% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.03% | -0.11% | 0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.05% | -0.48% | 0.13% | 100% | 100% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

## Results of test-2.1b

In Tests 2.1b, the mode can use block vectors of both IBC and intraTMP coded blocks from co-located luma to determine the CCCM reference area.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.03% | 100% | 100% |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.19% | -0.22% | 100% | 101% |
| Class C | 0.00% | -0.08% | -0.01% | 100% | 100% |
| Class E | 0.00% | 0.02% | -0.01% | 100% | 100% |
| Overall |  |  |  |  |  |
| Class D | 0.01% | 0.05% | 0.21% | 100% | 100% |
| Class F | -0.04% | -0.33% | -0.30% | 100% | 100% |
| Class TGM | -0.61% | -1.09% | -1.06% | 101% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.01% | -0.05% | 0.18% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.04% | -0.11% | 0.27% | 101% | 100% |
| Class F |  |  |  |  |  |
| Class TGM | -0.14% | -0.31% | -0.26% | 99% | 99% |

# References

[1] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, Oct. 2021.

# Patent rights declaration

IRT b-com have no current or pending patent rights relating to the technology described in this contribution.