| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0229-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AE0112(EE1-5.1: Deep Reference Frame Generation for Inter Prediction Enhancement) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhihuang Xie | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the cross-check results of JVET-AE0112, which is also known as EE1-5.1. In EE1-5.1, a simplified network architecture is studied for inter prediction to get a better trade-off between complexity and performance. Both training and inference are conducted in this crosscheck.

From the observation of the completed results, the performance difference between proponent’s model and crosschecker’s model is ~0.2% for random access. It is asserted that the performance gap is small.

Introduction

Fig. 1: The framework of the proposed DRF method

The framework of the EE1-5.1 is depicted in Figure 1. Two reconstructed frames chosen from the DPB are input into the deep reference frame generation network to synthesize a new frame. This synthesized frame is then added to both reference picture list0 and list1, using the same POC as the current encoding frame. It serves as an additional reference frame for subsequent encoding processes.

Crosscheck status

Training crosscheck

As required by the proponent of JVET-AE0112[1], the training crosscheck was conducted during this meeting cycle. All training scripts were provided by proponents of EE1-5.1. The crosschecker tried the best to match the training environment as same as the proponent’s. Python3.7 and pytorch1.9 were also required.

Vimeo dataset[2] was converted to YUV files then encoded by pure VTM anchor to generate the training dataset.

The whole training process took almost 4 days to finish 200 epoch on a V100.

The best performance in validation dataset was chosen to be the final model to conduct inference.

Inference crosscheck

The proposed method was implemented on top of NNVC-5.0, and model was converted to floating sadl.

Simulation results

The simulation was conducted under common test condition[3], but only neural network based inter prediction was enabled.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -3.08% | -7.12% | -6.28% | -3.68% | -9.14% | -7.78% | 934% | #NUM! |
| Class C | -4.32% | -6.77% | -6.88% | -5.74% | -9.57% | -9.62% | 505% | #NUM! |
| Class E |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -5.22% | -5.68% | -6.28% | -5.47% | -10.21% | -9.94% | 898% | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |

Reference

[1] W. Bao, W. Meng, J. Jia, Y. Zhang, H. Wang, Z. Chen, Z. Liu, X. Xu, and S. Liu. “EE1-5.1: Deep Reference Frame Generation for Inter Prediction Enhancement”. JVET-AE0112, 31th Meeting, Geneva, 11–20 July 2023.

[2] http://data.csail.mit.edu/tofu/dataset/vimeo_triplet.zip

[3] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AD2016, 30th Meeting, Antalya, 21-28 April.