| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0273-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AE0146 (AHG 12: Fixes H-CCP Table for CUs in Refreshed Areas of GDR/Recovering Pictures) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Lei Zhao | Author(s) or Contact(s): | zhaolei.0324@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

In JVET-AE0146, a fix relating to HCCP is proposed to address the leak issue when GDR is enabled. This contribution reports cross-check results of this fix.

- Introduction

In ECM-9.0, CCP (Cross Component Prediction) merge candidate list is constructed by using spatial adjacent, spatial non-adjacent, and history-based candidates.

In the case of GDR (Gradual Decoding Refresh), it is asserted that if any entry in the H-CCP table corresponds to a CU in the non-refreshed area of a GDR/recovering picture, a leak may occur. This is because the decoder cannot guarantee the correct decoding of coding information in non-refreshed areas.

In JVET-AE0146, it is proposed to address the leak issue by managing a separate H-CCP table specifically for CUs located in the refreshed area. This separate H-CCP table will only contain the CCP models of the last coded CUs within the refreshed area. With this separate H-CCP table, the CCP models of CUs in the non-refreshed areas will not be included in coding of CUs in refreshed areas, thus preventing potential leaks.

Cross-check results

The cross-check results are illustrated in Table 1, where the source code was provided by the proponent based on ECM 9.0 master commit 2e1d547d. Note that a non-CTC anchor is used in accordance with proponent’s instructions.

Inconsistant numbers are observed compared with the results in JVET-AE0146 (in Table 2), which was immediately reported to the proponent. Later the proponent claimed this deviation is caused by a code issue relating to the integration of JVET-AD0222, which is triggered in case of GDR. As suggested by the proponent, a new simulation is launched with JVET-AD0222 disabled. So far, the deviation disappears based on the finished results.

Table 1: Crosscheck results of JVET-AE0146.

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over GDR Anchor (ECM 9.0 master commit 2e1d547d) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 1.49% | 4.00% | 4.84% | 105.3% | 104.0% |
| Class E | 6.17% | 17.89% | 16.77% | 106.3% | 108.2% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 2.47% | 7.51% | 8.76% | 104.9% | 103.7% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 2: Simulation results from the proponent.

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over GDR Anchor (ECM 9.0 master commit 2e1d547d) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 1.43% | 3.83% | 4.66% | 104% | 104% |
| Class E | 6.11% | 17.77% | 16.29% | 105% | 107% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 2.47% | 7.51% | 8.76% | 103% | 104% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).