| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0278-v1 |
| --- | --- |

| Title: | AHG6/Non-EE2: Suggestions on memory reduction for ECM encoder |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121, USA | Tel: Email: | nanh@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the encoder of ALF and CCALF of ECM-9.0, co-variance matrices of CTUs request a high amount of memory. It is proposed to optimize ECM-9.0 encoder to reduce that memory usage.

# Introduction

In the encoder of ECM-9.0, adaptive loop filter (ALF) and cross component adaptive loop filter (CCALF) require co-variance matrices of coding tree units (CTUs) to derive the signalled filter coefficients and apply rate distortion optimization.

In ECM-9.0, a co-variance matrix is defined as

double [MaxAlfNumClippingValues][MaxAlfNumClippingValues][MAX_NUM_ALF_LUMA_COEFF][MAX_NUM_ALF_LUMA_COEFF],

where the constant MaxAlfNumClippingValues is the number of clipping values, which is equal to 4; and the constant MAX_NUM_ALF_LUMA_COEFF is the max number of coefficients of a filter, which is equal to 42. Co-variance matrices of luma ALF, chroma ALF and CCALF are defined in the same way, though the number of coefficients in each filter is different and CCALF doesn’t have nonlinearity as ALF.

A signalled luma ALF filter is applied to the ALF input samples, samples before the deblocking filter, outputs of fixed filters, output of a Gaussian filter and the residual data. For fixed filters, a CTU can have two candidates. Therefore, two co-variance matrices are needed (one for each candidate). The difference between these two co-variance matrices are only the fixed filter outputs, while other values are identical.

# Proposed method

- It is proposed to define the co-variance matrix of a filter based on the actual filter size.

In ECM-9.0, the numbers of coefficients of a luma ALF filter, a chroma ALF filter and a CCALF filter are 39, 20 and 24, respectively. Defining a co-variance matrix by assuming the numbers of coefficients of all filters are 42 allocates unnecessary memory.

- It is proposed to remove the non-linearity of co-variance matrices for CCALF.

- To avoid storing identical information in two co-variance matrices, it is proposed to use one big matrix to store both the identical info (the ALF input samples, samples before the deblocking filter, output of a gaussian filter and the residual data) and different info (outputs of fixed filters from different candidate).

- It is proposed to change the data type of a co-variance matrix into float from double.

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.