| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0281-v1 |
| --- | --- |

| Title: | Unified LOP filter design, training procedure and filter usage |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy, Yun Li, Marta Karczewicz (Qualcomm) Tong Shao, Peng Yin, Sean McCarthy (Dolby) Jay N. Shingala, Ajay Shyam(Ittiam) Junru Li, Yue Li, Chaoyi Lin, Kai Zhang, Li Zhang (Bytedance) | Email: | {dmytror, yuli30, martak} @qti.qualcomm.com {tshao, pyin}@dolby.com {lijunru, yue.li, linchaoyi.cy, zhangkai.video, lizhang.idm} @bytedance.com |
| Source: | Qualcomm, Dolby, Ittiam Systems, Bytedance |  |  |

_____________________________

# Abstract

A Unified Filter Architecture for the High-performance Operation Point (HOP) was defined at JVET-AD meeting and includes model architecture, training process and training test set. Additionally, a Low-complexity Operation Point (LOP) in-loop filter, with complexity of 16.2 kMAC/pixel (block-wise), was also adopted in JVET-AD meeting to the NNVC 5.0 software to serve as an anchor for further HOP development.

For JVET-AE meeting, two EE1 tests and one AHG11 proposal have been submitted in LOP category: EE1-4.1 (JVET-AE0067), EE1-4.4 (JVET-AE0165), and JVET-AE0238. EE1-4.1 tests presented a significant complexity reduction achieved by luma-chroma split in the NN architecture, whereas EE1-4.1 presented HOP based architecture of LOP complexity and utilizes HOP training process. JVET-AE0238 presented study on unified filter usage interfaces of HOP and LOP filters.

This contribution proposes a unified filter architecture and filter usage for LOP study in EE1. Proposed architecture combines key components of EE1-4.1, EE1-4.4, and JVET-AE0238 tests and is configured to meet complexity constraints of the current NNVC5.0 LOP anchor (16kMAC/pixel) and employs unified filter usage for LOP.

Defined training is HOP aligned: utilizes HOP dataset, training script and training parameters with changes limited to the scheduler. Filter usage, output interface is subject of study in sub-test.

It is proposed to introduce this architecture as EE1 test for study on LOP.

# Introduction

For JVET-AE meeting, two EE1 tests have been submitted in LOP category: EE1-4.1 (JVET-AE0067) [1] and EE1-4.4 (JVET-AE0165) [2]. Additionally, a contribution discussing LOP filter usage has been discussed in JVET-AE0238 [3].

In the test EE1-4.1 the NNVC5.0 LOP architecture has been modified to achieve lower computational complexity. Complexity reduction to 10kMAC/pixel is achieved by introducing luma/chroma split with different complexity for Luma and Chroma branches. Block diagram of the architecture proposed in EE1-4.1 is shown in Figure 1.

In the EE1-4.4, the HOP filter architecture, the training dataset and end-to-end training process was used to derive a filter that meets the complexity constraints of LOP. In order to reduce the complexity of the filter to the range 14-18kMAC/pixel, the following changes have been introduced. The complexities of the head block were reduced by decreasing the number of the extracted feature maps. In the backbone, the number of residual blocks was reduced to 11, and the complexity of the residual blocks was further reduced by employing CP decomposition along with the reduction of the features. In the tail block following the backbone, the convolution was also decomposed. In addition, the activations layer of the head blocks was removed, and some layers of the CP decomposition were merged. The proposed architecture is shown in Figure 2. Training was conducted closely following HOP training procedure with alternations to scheduler justified by the change of the architecture.

Ky

NxN

KC

Channel Concat

LY

NxN

LC

3x3 CPD 10xM

Leaky ReLu

3x3 CPD KYxLY

1x1 conv MxM

Leaky ReLu

3x3 CPD KxK

1x1 conv MxK

Leaky ReLu

3x3 CPD KYxKY

1x1 conv MYxKY

BS info

QStep

Yx4+U+V 72x72x10

NxN

1x1 conv KYxMY

Leaky ReLu

3x3 CPD KYxKY

1x1 conv MYxKY

Y’x4+U’+V‘ 64x64x6

N’xN’

NY Hidden Layers (Luma)

NC Hidden Layers (Chroma)

1x1 conv MCxKC

1x1 conv KCxMC

3x3 CPD KCxKC

1x1 conv MCxKC

3x3 CPD KCxKC

1x1 conv KCxMC

Leaky ReLu

3x3 CPD KCxLC

Leaky ReLu

1x1 conv KYxMY

Layers approximated with CP decomposition

Separable Convolution layers

3x3 CPD KYxKY

Rank RY CP decomposition

1x1 conv KYxRY

3x1 Sep conv RY

1x1conv RYxKY

1x3 Sep conv RY

Figure 1 Filter architecture of EE1-4.1

Figure 2 Filter Architecture of EE1-4.2

In the JVET-AE0238, operations namely slice-level model selection, among 2 available for I and 4 available for P/B is introduced to adapt various filtering models in the unified filter usage. Common operations in the HOP filter and the LOP filter, such as residual scaling, on/off control in slice level, on/off control in CTU level maintains the same.

# Proposed Unified LOP Filter Architecture

In this contribution a unified filter architecture and filter usage for LOP study in EE1 is presented. Proposed architecture combines key components of EE1-4.1, EE1-4.4, and JVET-AE0238 and is designed to be closely aligned to the HOP filter architecture and to employ a modification of the HOP training procedure.

The input interface of the filter is aligned to HOP, with number of features defined in EE1-4.4 for JVET-AD meeting. To address arguments on granularity of the number of input features in the headblock, sub-test on the optimization of the headblock design will be conducted.

The backbone for luma and chroma consists of a sequence of the residue blocks defined in the EE1-4.4 as aligned with HOP architecture. The number of residue blocks in luma backbone and chroma backbone, as well as its complexity configurations is subject of EE1 sub-test.

The split of Luma, Chroma with different complexity for luma backbone and chroma backbone  is introduced in EE1-4.1. Further optimization of the luma backbone and chroma backbone complexity is to be conducted in EE1 subtest.

The filter usage is similar to current HOP in terms of interface of input, output. Additional changes may include adaptive block size, slice-level model selection, on/off control at low block level and can be a subject of EE1 sub-test.

Figure 3 Proposed unified architecture for LOP EE1 test

# Training process

Training will be conducted following the procedure specified in JVET-AD2023 for training of the HOP.

It is expected that HOP training parameters changes will be limited to the scheduler, whereas the dataset, parameters of augmentation, generation of coded data (QP points) to be identical to the HOP training parameters.

# Planned EE1 LOP tests:

Test LOP1: testing proposed LOP filter architecture at NNVC5.0 LOP complexity levels (16kMAC/pixel).

Test LOP2: Optimization of the headblock design toward improving hardware friendliness.

Test LOP3: Optimization of the split luma and chroma backbone design toward better complexity-performance tradeoff.

Test LOP4: Study on filter usage and combine interfaces for usage of HOP and LOP filters, namely model selection, adaptive block sizes, on/off control at low block level.

# Conclusion

This contribution proposes a unified filter architecture for LOP study in EE1. Proposed architecture combines key components of EE1-4.1, EE1-4.4, and JVET-AE0238 and is designed to be closely aligned to the HOP filter architecture and to employ a modification of the HOP training procedure and to unify the filter usage for LOP filter.

It is proposed to introduce this architecture as EE1 test for study on LOP.

# References

- EE1-4.1: Neural-network loop filters with further complexity reduction, JVET-AE0067, J.N. Shingala, A. Shyam, A. Suneja, S. P Badya (Ittiam), T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy (Dolby).

- EE1-4.4: Low complexity NN filter with design elements of Unified Filter Architecture and EE1-1.2 and EE1-1.3, JVET-AE0165, Y. Li, D. Rusanovskyy, M. Karczewicz. (Qualcomm)

- AHG11: A unified design of NN-based loop-filters at low and high operation points, JVET-AE0238, J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang (ByteDance)

# Patent rights declaration(s)

Dolby may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).