| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE1016-v1 |
| --- | --- |

| Title: | AVC with extensions and corrections (Draft 1) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | B. Bross T. Ikai G. J. Sullivan A. Tourapis Y.-K. Wang | Email: | benjamin.bross@hhi.fraunhofer.de ikai.tomohiro@sharp.co.jp gary.sullivan@dolby.com atourapis@apple.com yekui.wang@bytedance.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains the draft text for changes of the Advanced Video Coding (AVC) standard (Rec. ITU-T H.264 | ISO/IEC 14496-10). Text modifications are provided for 1) the support of additional colour type identifiers; 2) some technical corrections and editorial improvements to the 10th edition text of AVC; and 3) the AVC-specific supports for some supplemental enhancement information (SEI) messages that may be included in AVC bitstreams but are not to be specified in the AVC specification. These SEI messages include three other SEI messages, namely the neural network post-filter characteristics SEI message, the neural-network post-processing filter activation SEI message, and the phase indication SEI message, that are to be specified in a new edition of the Versatile Supplemental Enhancement Information messages for coded video bitstreams (VSEI) standard (ITU-T H.274 | ISO/IEC 23002-7).

The affected sections are provided relative to the 2022 edition of ISO/IEC 14496-10, i.e. the 10th edition.

Draft 1 incorporated items:

- JVET-AD1008 Additional colour type identifiers for AVC

- JVET-AE0101 Use of neural network post filter and phase indication SEI messages for AVC

- JVET-AE1004 Errata items on AVC

- Fix for film grain synthesis equation in FGC SEI message (see notes under BoG report JVET-AE0294)

# Changes to the specification text of AVC

In subclause 0.6.1, make the following changes:

Rec. ITU-T H.274 | ISO/IEC 23002-7 specifies the syntax and semantics of some video usability information (VUI) parameters and supplemental enhancement information (SEI) messages that do not affect the conformance specifications in Annex C and subclauses F.8, G.8, H.8, and I.8. Among these SEI messages, those for which the syntax and semantics …"

In subclause 7.2, make the following changes:

next_bits( n ) provides the next bits in the bitstream for comparison purposes, without advancing the bitstream pointer. It provides a look at the next n bits in the bitstream with n being its argument. When used within the byte stream format as specified in Annex B and fewer than n bits remain within the byte stream, next_bits( n ) returns a value of 0.

In subclause C.4, make the following changes:

For output timing decoder conformance, the HSS operates as described above, with delivery schedules selected only from the subset of values of SchedSelIdx for which the bit rate and CPB size are restricted as specified in Annex ‎A, Annex ‎G, Annex ‎H, and Annex ‎I for the specified profile and level, or with "interpolated" delivery schedules as specified below for which the bit rate and CPB size are restricted as specified in Annex ‎A, Annex ‎G, Annex ‎H, and Annex ‎I. The same delivery schedule is used for both the HRD and DUT.

When the HRD parameters and the buffering period SEI messages are present with cpb_cnt_minus1 greater than 0, the decoder shall be capable of decoding the bitstream as delivered from the HSS operating using an "interpolated" delivery schedule specified as having peak bit rate r, CPB size c( r ), and initial CPB removal delay ( f( r ) ÷ r ) and initial CPB removal delay offset ( o( r ) ÷ r ) as follows:

 = ( r − BitRate[ SchedSelIdx − 1 ] ) ÷ ( BitRate[ SchedSelIdx ] − BitRate[ SchedSelIdx − 1 ] ),	(C-17)

c( r ) =  * CpbSize[ SchedSelIdx ] + (1 −  * CpbSize[ SchedSelIdx−1 ],	(C-18)

f( r ) = initial_cpb_removal_delay[ SchedSelIdx ] * BitRate[ SchedSelIdx ] + 
		( 1 −  initial_cpb_removal_delay[ SchedSelIdx − 1 ] * BitRate[ SchedSelIdx − 1 ]	(C-19)

o( r ) = α * initial_cpb_removal_delay_offset[ SchedSelIdx] * BitRate[ SchedSelIdx ] +
		( 1 − α ) * initial_cpb_removal_delay_offset[ SchedSelIdx − 1 ] * BitRate[ SchedSelIdx − 1 ]		(C-XX)

## In subclause D.1.1, make the following changes:

| sei_payload( payloadType, payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| if( payloadType = = 0 ) |  |  |
| buffering_period( payloadSize ) | 5 |  |
| … |  |  |
| else if( payloadType = = 202 ) |  |  |
| annotated_regions( payloadSize ) /* specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ [Ed. (YK/GJS): Note that the ISO text of AVC refers to VSEI for this SEI message, while the ITU text of AVC directly includes the specification of this SEI message. Consider changing the ITU text to also refers to VSEI for this SEI message and removing the text from the ITU text of AVC spec, for consistency between the two texts and for easier future maintenance.] | 5 |  |
| else if( payloadType = = 205 ) |  |  |
| shutter_interval_info( payloadSize ) [Ed. (YK/BB): Consider referring to VSEI for this SEI message and removing the text from the AVC spec, for easier future maintenance.] | 5 |  |
| else if( payloadType = = 210 ) |  |  |
| nn_post_filter_characteristics( payloadSize ) /* specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ | 5 |  |
| else if( payloadType = = 211 ) |  |  |
| nn_post_filter_activation( payloadSize ) /* specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ | 5 |  |
| else if( payloadType = = 212 ) |  |  |
| phase_indication( payloadSize ) /* specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ | 5 |  |
| else |  |  |
| reserved_sei_message( payloadSize ) | 5 |  |
| if( !byte_aligned( ) ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 5 | f(1) |
| while( !byte_aligned( ) ) |  |  |
| bit_equal_to_zero /* equal to 0 */ | 5 | f(1) |
| } |  |  |
| } |  |  |

## In subclause D.1.24, make the following changes:

| post_filter_hint( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| filter_hint_size_y | 5 | ue(v) |
| filter_hint_size_x | 5 | ue(v) |
| filter_hint_type | 5 | u(2) |
| for( colour_componentcIdx = 0; colour_componentcIdx < 3; colour_componentcIdx ++ ) |  |  |
| for( cy = 0; cy < filter_hint_size_y; cy++ ) |  |  |
| for( cx = 0; cx < filter_hint_size_x; cx++ ) |  |  |
| filter_hint_value[ colour_componentcIdx ][ cyx ][ cxy ] | 5 | se(v) |
| additional_extension_flag | 5 | u(1) |
| } |  |  |

In subclause D.2.1, add the following at the end:

PayloadBits is set equal to 8 * payloadSize − nPayloadAlignmentBits where nPayloadAlignmentBits is the number of bit_equal_to_one and bit_equal_to_zero syntax elements at the end of the sei_payload( ) syntax structure.

Delete subclause D.2.1.1:

      - Use of Rec. ITU-T H.274 | ISO/IEC 23002-7 SEI messages

When any SEI message for which the payload type value is specified in this document and the syntax and semantics are specified in Rec. ITU-T H.274 | ISO/IEC 23002-7 is included in a bitstream specified by this document, the SEI payload syntax shall be included into the sei_payload( ) syntax structure as specified in subclause D.1.1, shall use the payloadType value specified in subclause D.1.1, and, additionally, any SEI-message-specific constraints specified in this annex for that particular SEI message shall apply.

The value of PayloadBits as specified in subclause D.2.1.1 is passed to the parser of the SEI message syntax structures specified in Rec. ITU-T H.274 | ISO/IEC 23002-7.

When an annotated regions SEI message is included in a bitstream specified by this document, for purposes of interpretation of the annotated regions SEI message, the following variables are specified:

–	CroppedWidth and CroppedHeight, set as specified by Formulae 7-23 and 7-24, respectively.

–	A conformance cropping window left offset, ConfWinLeftOffset, set equal to CropUnitX * frame_crop_left_offset.

–	A conformance cropping window top offset, ConfWinTopOffset, set equal to CropUnitY * frame_crop_top_offset.

–	ChromaFormatIdc, set equal to chroma_format_idc.

In subclause D.2.21 (Film grain characteristics SEI message semantics), replace the first sentence with the following:

This SEI message provides the decoder with a parameterized model for a film grain synthesis process. The film grain synthesis process should be applied to the decoded pictures prior to their display.

In subclause D.2.21 (Film grain characteristics SEI message semantics) modify equation D-26 as follows:

G[ c ][ x ][ y ] = ( comp_model_value[ c ][ sj ][ 0 ] * n[ c ][ x ][ y ] +
		comp_model_value[ c ][ sj ][ 1 ] * ( G[ c ][ x − 1 ][ y ] +
		( ( comp_model_value[ c ][ s j ][ 4 ] * G[ c ][ x ][ y − 1 ] ) >> 
		log2_scale_factor ) ) + 	comp_model_value[ c ][ s j ][ 3 ] *
		( ( ( comp_model_value[ c ][ s j ][ 4 ] * ( G[ c ][ x − 1 ][ y − 1 ] + G[ c ][ x + 1 ][ y − 1 ] ) >>
		log2_scale_factor ) + G[ c ][ x + 1 ][ y − 1 ] )+
		comp_model_value[ c ][ s j ][ 5 ] * ( G[[ c ][ x − 2 ][ y ] +
		( ( comp_model_value[ c ][ s j ][ 4 ] * comp_model_value[ c ][ s j ][ 4 ] * G[ c ][ x ][ y − 2 ] ) >>
		( 2 * log2_scale_factor ) ) ) +
		comp_model_value[ c ][ s j ][ 2 ] * G[ c − 1 ][ x ][ y ] ) >> log2_scale_factor	(D-26)

In subclause D.2.24 (Post-filter hint SEI message semantics) make the following changes:

This SEI message provides the coefficients of a post-filter or correlation information for the design of a post-filter for potential use in post-processing of the output picturescurrent picture after it is decoded, cropped, and output, to obtain improved displayed quality.

...

filter_hint_type identifies the type of the transmitted filter hints as specified in Table D-7. The value of filter_hint_type shall be in the range of 0 to 2, inclusive. Decoders shall ignore post-filter hint SEI messages having filter_hint_type equal to the reserved value 3.

Table D-7 – filter_hint_type values

| Value | Description |
| --- | --- |
| 0 | coefficients of a 2D FIRfilter |
| 1 | coefficients of two 1D FIR filters |
| 2 | cross-correlation matrix |
| 3 | Reserved |

filter_hint[ colour_component ][ cy ][ cx ] specifies a filter coefficient or an element of a cross-correlation matrix between original and decoded signal with 16-bit precision. The value of filter_hint[ colour_component ][ cy ][ cx ] shall be in the range of −231 + 1 to 231 − 1, inclusive. colour_component specifies the related colour component. cy represents a counter in vertical direction, cx represents a counter in horizontal direction. Depending on filter_hint_type, the following applies:

–	If filter_hint_type is equal to 0, the coefficients of a 2-dimensional FIR filter with the size of filter_hint_size_y * filter_hint_size_x are transmitted.

–	Otherwise, if filter_hint_type is equal to 1, the filter coefficients of two 1-dimensional FIR filters are transmitted. In this case, filter_hint_size_y shall be equal to 2. The index cy = 0 specifies the filter coefficients of the horizontal filter and cy = 1 specifies the filter coefficients of the vertical filter. In the filtering process, the horizontal filter shall be applied first and the result shall be filtered by the vertical filter.

–	Otherwise (filter_hint_type is equal to 2), the transmitted hints specify a cross-correlation matrix between the original signal s and the decoded signal s′.

NOTE 1	The normalized cross-correlation matrix for a related colour component with the size of filter_hint_size_y * filter_hint_size_x is defined as follows:

(D-27)

where s denotes the original frame, s′ denotes the decoded frame, h denotes the vertical height of the related colour component, w denotes the horizontal width of the related colour component, bitDepth denotes the bit depth of the colour component, offset_y is equal to (filter_hint_size_y >> 1), offset_x is equal to (filter_hint_size_x >> 1), 0 <= cy < filter_hint_size_y and 0 <= cx < filter_hint_size_x.

NOTE 2	A decoder can derive a Wiener post-filter from the cross-correlation matrix of original and decoded signal and the auto-correlation matrix of the decoded signal.

filter_hint_type identifies the type of the filter hints as specified in Table D-7. The value of filter_hint_type shall be in the range of 0 to 2, inclusive. The value of filter_hint_type equal to 3 is reserved for future use by ITU T | ISO/IEC and shall not be present in bitstreams conforming to this version of this document. Decoders shall ignore post-filter hint SEI messages having filter_hint_type equal to 3.

Table D-7 – filter_hint_type values

| Value | Description |
| --- | --- |
| 0 | Coefficients of a 2D FIR filter |
| 1 | Coefficients of two 1D FIR filters |
| 2 | Cross-correlation matrix |

filter_hint_value[ cIdx ][ cx ][ cy ] specifies a filter coefficient or an element of a cross-correlation matrix between the original and the output signal with 16-bit precision. The value of filter_hint_value[ cIdx ][ cx ][ cy ] shall be in the range of −231 + 1 to 231 − 1, inclusive. cIdx specifies the related colour component. cx represents a counter in the horizontal direction, cy represents a counter in the vertical direction.

When filter_hint_type is equal to 1, filter_hint_size_y shall be equal to 2.

Depending on the value of filter_hint_type, the following applies:

–	If filter_hint_type is equal to 0, the coefficients of a two-dimensional FIR filter with the size of filter_hint_size_x * filter_hint_size_y are provided.

–	Otherwise, if filter_hint_type is equal to 1, the filter coefficients of two one-dimensional FIR filters are provided. In this case, filter_hint_size_y shall be equal to 2. In this case, the index cy equal to 0 specifies the filter coefficients of the horizontal filter and cy equal to 1 specifies the filter coefficients of the vertical filter. In the indicated filtering process, the horizontal filter is applied first and the result is filtered by the vertical filter.

–	Otherwise (filter_hint_type is equal to 2), the hint values specify a cross-correlation matrix between the original signal s and the output signal prior to the post-filtering operation s′.

When filter-hint_type is equal to 2, the normalized cross-correlation matrix values for a related colour component identified by cIdx with the size of filter_hint_size_x * filter_hint_size_y should be derived by the encoder as follows:

filter_hint_value[ cIdx ][ cx ][ cy ]=Round( 14( bitDepth - 8 ) * w * h *
	n = 0h - 1m = 0w - 1s m [ n ] * s'[ Clip3( 0, w-1,m+cx-midX ) ][ Clip3( 0, h-1, n+cy-midY ) ] )	(D-27)

where s denotes the array of samples of the colour component cIdx of the original picture, s′ denotes the corresponding array of the cropped output picture, h denotes the vertical height of the related colour component, w denotes the horizontal width of the related colour component, bitDepth denotes the bit depth of the colour component, midX is equal to (filter_hint_size_x >> 1), midY is equal to (filter_hint_size_y >> 1), 0 <= cx < filter_hint_size_x and 0 <= cy < filter_hint_size_y.

NOTE	A decoder can derive a Wiener post-filter from the cross-correlation matrix between the original signal s and the output signal s′ and the auto-correlation matrix of the the output signal s′ (which the decoder can compute from the output signal s′).

In subclause D.2.35.6.2, make the following changes:

–	If RegionWisePackingFlag is equal to 0 and erp_padding_flag is equal to 1, the following applies:

hPos′ = hPos − left_gb_erp_width		(D-45)
tmpPicWidthpictureWidth = pictureWidth − left_gb_erp_width − right_gb_erp_width

–	Otherwise, the following applies:

hPos′ = hPos		(D-46) 
tmpPicWidth = pictureWidth

–	The following applies:

ϕ = 180 − hPos′ * ( 360 ÷ tmpPicWidthpictureWidth )		(D-47)
θ = 90 − vPos * ( 180 ÷ pictureHeight )

In subclause D.2.35.6.4, make the following changes:

Inputs to this clause are:

- sample location (x, y) within the packed region, where x and y are in relative packed picture sample units, while the sample location is at an integer sample location within the packed picture,

- the width and the height (projRegWidth, projRegHeight) of the projected region, in relative projected picture sample units,

- the width and the height (packedRegWidth, packedRegHeight) of the packed region, in relative packed picture sample units,

- transform type (transformType), and

- offset values for the sampling position (offsetX, offsetY) in the range of 0, inclusive, to 1, exclusive, in horizontal and vertical relative packed picture sample units, respectively.

NOTE	offsetX and offsetY both equal to 0.5 indicate a sampling position that is in the centre point of a sample in packed picture sample units.

Outputs of this clause are:

- the centre point of the sample location (hPos, vPos) within the projected region in relative projected picture sample units, where hPos and vPos may have non-integer real values.

The outputs are derived as follows:

if( transformType  = =  0  | |  transformType  = =  1  | |  transformType  = =  2  | |  transformType  = =  3 ) {
	horRatio = projRegWidth ÷ packedRegWidth
	verRatio = projRegHeight ÷ packedRegHeight
} else if( transformType  = =  4  | |  transformType  = =  5  | |  transformType  = =  6  | |
	transformType  = =  7 ) {
	horRatio = projRegWidth ÷ packedRegHeight
	verRatio = projRegHeight ÷ packedRegWidth
}
if( transformType  = =  0 ) {
	hPos = horRatio * ( x + offsetX0.5 )
	vPos = verRatio * ( y + offsetY 0.5 )
} else if( transformType  = =  1 ) {
	hPos = horRatio * ( packedRegWidth − x − offsetX0.5 )
	vPos = verRatio * ( y + offsetY0.5 )
} else if( transformType  = =  2 ) {
	hPos = horRatio * ( packedRegWidth − x − offsetX0.5 )
	vPos = verRatio * ( packedRegHeight − y − offsetY0.5 )	(D-50)
} else if( transformType  = =  3 ) {
	hPos = horRatio * ( x + offsetX0.5 )
	vPos = verRatio * ( packedRegHeight − y − offsetY0.5 )
} else if( transformType  = =  4 ) {
	hPos = horRatio * ( y + offsetY0.5 )
	vPos = verRatio * ( x + offsetX0.5 )
} else if( transformType  = =  5 ) {
	hPos = horRatio * ( y + offsetY0.5 )
	vPos = verRatio * ( packedRegWidth − x − offsetX0.5 )
} else if( transformType  = = 6 ) {
	hPos = horRatio * ( packedRegHeight − y − offsetY0.5 )
	vPos = verRatio * ( packedRegWidth − x − offsetX0.5 )
} else if( transformType  = =  7 ) {
	hPos = horRatio * ( packedRegHeight − y − offsetY0.5 )
	vPos = verRatio * ( x+ offsetX0.5 )
}

In subclause D.2.35.6.5, make the following changes:

This clause specifies the semanticsmapping of luma sample locations within a cropped decoded picture to sphere coordinates relative to the global coordinate axes.

offsetX is set equal to 0.5 and offsetY is set equal to 0.5.

If RegionWisePackingFlag is equal to 1, the following applies for each packed region n in the range of 0 to NumPackedRegions − 1, inclusive:

- For each sample location (xPackedPicture, yPackedPicture) belonging to the n-th packed region, the following applies:

- The corresponding sample location (xProjPicture, yProjPicture) of the projected picture is derived as follows:

- x is set equal to xPackedPicture − PackedRegionLeft[ n ].

- y is set equal to yPackedPicture − PackedRegionTop[ n ].

- Subclause D.2.35.6.4 is invoked with x, y, PackedRegionWidth[ n ], PackedRegionHeight[ n ], ProjRegionWidth[ n ], ProjRegionHeight[ n ], and TransformType[ n ], offsetX, and offsetY as inputs, and the output is assigned to sample location (hPos, vPos).

- xProjPicture is set equal to ProjRegionLeft[ n ] + hPos.

- When StereoFlag is equal to 0 or TopBottomFlag is equal to 1, and when xProjPicture is greater than or equal to proj_picture_width, xProjPicture is set equal to xProjPicture − proj_picture_width.

- When SideBySideFlag is equal to 1, the following applies:

- When ProjRegionLeft[ n ] is less than proj_picture_width / 2 and xProjPicture is greater than or equal to proj_picture_width / 2, xProjPicture is set equal to xProjPicture − proj_picture_width / 2.

- When ProjRegionLeft[ n ] is greater than or equal to proj_picture_width / 2 and xProjPicture is greater than or equal to proj_picture_width, xProjPicture is set equal to xProjPicture − proj_picture_width / 2.

- yProjPicture is set equal to ProjRegionTop[ n ] + vPos.

- Subclause D.2.35.6.6 is invoked with xProjPicture, yProjPicture, ConstituentPicWidth, and ConstituentPicHeight as inputs, and the outputs indicating the sphere coordinates and the constituent picture index (for frame-packed stereoscopic video) for the luma sample location (xPackedPicture, yPackedPicture) belonging to the n-th packed region in the decoded picture.

Otherwise (RegionWisePackingFlag is equal 0), the following applies for each sample location (x, y) that is not an equirectangular projection padded sample within the cropped decoded picture, where a sample location (x, y) is an equirectangular projection padded sample when and only when ErpFlag is equal to 1, x is in the range of 0 to left_gb_erp_width − 1, inclusive, or ConstituentPicWidth − right_gb_erp_width to ConstituentPicWidth − 1, inclusive, and y is in the range of 0 to ConstituentPicHeight − 1, inclusive:

- xProjPicture is set equal to x + offsetX0.5.

- yProjPicture is set equal to y + offsetY0.5.

- If ErpFlag is equal to 0, projPicWidth is set equal to ConstituentPicWidth. Otherwise (ErpFlag is equal to 1), projPicWidth is set equal to ConstituentPicWidth − ( left_gb_erp_width + right_gb_erp_width ).

- Subclause D.2.35.6.6 is invoked with xProjPicture, yProjPicture, projPicWidth, and ConstituentPicHeight as inputs, and the outputs indicating the sphere coordinates and the constituent picture index (for frame-packed stereoscopic video) for the sample location (x, y) within the region-wise packed picture.

Add the following subclause D.3:

  - Use of SEI messages specified in other documents

    - General

The SEI messages having syntax structures identified in subclause D.1.1 that are specified in other documents including Rec. ITU-T H.274 | ISO/IEC 23002-7, ISO/IEC 23001-11, or ISO/IEC 23090-13 may be used together with bitstreams specified by this document.

When any SEI message for which the payload type value is specified in this document and the syntax and semantics are specified in Rec. ITU-T H.274 | ISO/IEC 23002-7 is included in a bitstream specified by this document, the SEI payload syntax shall be included into the sei_payload( ) syntax structure as specified in subclause D.1.1, shall use the payloadType value specified in subclause D.1.1, and, additionally, any SEI-message-specific constraints specified in this annex for that particular SEI message shall apply.

The value of PayloadBits as specified in subclause D.1.1 is passed to the parser of the SEI message syntax structures specified in Rec. ITU-T H.274 | ISO/IEC 23002-7.

    - Use of the annotated regions SEI message

When an annotated regions SEI message is included in a bitstream specified by this document, for purposes of interpretation of the annotated regions SEI message, the following variables are specified:

–	CroppedWidth and CroppedHeight, set as specified by Formulae 7-23 and 7-24, respectively.

–	A conformance cropping window left offset, ConfWinLeftOffset, set equal to CropUnitX * frame_crop_left_offset.

–	A conformance cropping window top offset, ConfWinTopOffset, set equal to CropUnitY * frame_crop_top_offset.

–	ChromaFormatIdc, set equal to chroma_format_idc.

    - Use of the neural network post-filter characteristics SEI message and the neural-network post-filter activation SEI message

Let currPic be the cropped decoded output picture for which the neural-network post-processing filter (NNPF) defined by the neural-network post-filter characteristics (NNPFC) SEI message is activated by a neural-network post-filter activation (NNPFA) SEI message and currLayerId be the nuh_layer_id value of currPic.

It is a requirement of bitstream conformance that when an NNPFA SEI message is associated with a picture, the value of output_flag, when present, of that picture shall be equal to 1.

NOTE – Since only cropped decoded output pictures are used as input pictures of the NNPF, the value of output_flag, when present, of the coded picture corresponding to each input picture of the NNPF is equal to 1.

The variable pictureRateUpsamplingFlag is set equal to ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1: 0.

The variable numInputPics is set equal to nnpfc_num_input_pics_minus1 + 1.

The variable numInferences is derived as follows:

–	If all of the following conditions are true, the variable numPostRoll is set equal to the value of i such that nnpfc_interpolated_pics[ i ] is greater than 0 and the variable numInferences is set equal to 1 + numPostRoll:

–	nnpfc_purpose is equal to 8 (i.e., the only purpose for the NNPF is picture rate upsampling).

–	The NNPFA SEI message that activated this NNPF has nnpfa_persistence_flag equal to 1.

–	nnpfc_interpolated_pics[ i ] is greater than 0 only for a single value of i that is greater than 0.

–	Either of the following conditions is true:

–	currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId.

–	currPic is the last picture in the coded video sequence in output order and nnpfa_no_foll_clvs_flag is equal to 1.

–	Otherwise, if all of the following conditions are true, the variable numPostRoll is set equal to InpIdx[ i ] for the value of i such that nnpfa_output_flag[ i ] is equal to 1, and the variable numInferences is set equal to 1 + numPostRoll:

–	pictureRateUpsamplingFlag is equal to 0.

–	numInputPics is greater than 1.

–	The NNPFA SEI message that activated this NNPF has nnpfa_persistence_flag equal to 1.

–	nnpfa_output_flag[ idx ] is equal to 1 for a single value of idx in the range of 0 to NumInpPicsInOutputTensor − 1, inclusive, and for that single value of idx, InpIdx[ idx ] is greater than 0.

–	Either of the following conditions is true:

–	currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId.

–	currPic is the last picture in the coded video sequence in output order and nnpfa_no_foll_clvs_flag is equal to 1.

–	Otherwise, the variable numInferences is set equal to 1.

For each value of j in the range of 0 to numInferences − 1, inclusive, the following applies:

–	The arrays inputPic[ i ] and inputPresentFlag[ i ] for i in the range of 0 to numInputPics − 1, inclusive, representing all the input pictures and the presence of input pictures, respectively, are specified as follows:

–	When j is greater than 0, for each value of k in the range of 0 to j − 1, inclusive, inputPic[ k ] is set to be currPic and inputPresentFlag[ k ] is set equal to 0.

–	The j-th input picture, inputPic[ j ], is set to be currPic and inputPresentFlag[ j ] is set equal to 1.

–	When numInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numInputPics − 1, inclusive, in increasing order of i:

–	If both of the following conditions are true, inputPic[ i ] is set to be prevPic and inputPresentFlag[ i ] is set equal to 1:

–	Either of the following conditions is true:

–	pictureRateUpsamplingFlag is equal to 1 and currPic is associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5 and a particular value of current_frame_is_frame0_flag, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId, precede inputPic[ i − 1 ] in output order, and are associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5 and the same value of current_frame_is_frame0_flag.

–	pictureRateUpsamplingFlag is equal to 0 or currPic is not associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId and precede inputPic[ i − 1 ] in output order.

–	nnpfa_no_prev_clvs_flag is equal to 0 or the coded picture corresponding to prevPic and the current picture are present in the same coded video sequence.

–	Otherwise, the following applies:

–	inputPic[ i ] is set to be the same picture as inputPic[ i − 1 ] and inputPresentFlag[ i ] is set equal to 0.

–	It is a requirement of bitstream conformance that, when pictureRateUpsamplingFlag is equal to 1, nnpfc_interpolated_pics[ i − 1 ] shall be equal to 0.

–	It is a requirement of bitstream conformance that when inputPresentFlag[ i ] is equal to 0 and nnpfc_input_pic_output_flag[ i ] is equal to 1, the value of nnpfa_output_flag[ idx ] shall be equal to 0 for the value of idx such that InpIdx[ idx ] is equal to i.

–	For purposes of interpretation of the NNPFC SEI message, the following variables are specified:

–	If numInputPics is greater than 1 and there is a second NNPF that is defined by at least one NNPFC SEI message, is activated by an NNPFA SEI message for currPic, and has nnpfc_purpose equal to 4, the following applies:

–	CroppedWidth is set equal to nnpfcOutputPicWidth defined for the second NNPF.

–	CroppedHeight is set equal to nnpfcOutputPicHeight defined for the second NNPF.

–	Otherwise, the following applies:

–	CroppedWidth is set as specified by Formulae 7-23 for currPic.

–	CroppedHeight is set as specified by Formulae 7-24 for currPic.

–	The luma sample arrays CroppedYPic[ i ] and the chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are derived as follows for each value of i in the range of 0 to numInputPics − 1, inclusive:

–	The variable sourcePic is derived as follows:

–	If inputPresentFlag[ i ] is equal to 1 or nnpfc_absent_input_pic_zero_flag is equal to 0, sourcePic is set to be inputPic[ i ].

–	Otherwise(inputPresentFlag[ i ] is equal to 0 and nnpfc_absent_input_pic_zero_flag is equal to 1), sourcePic is set to be a picture with a luma sample array of CroppedWidth × CroppedHeight samples equal to 0 and Cb and Cr sample arrays of ( CroppedWidth / SubWidthC ) × ( CroppedHeight / SubHeightC ) samples equal to 0.

–	If numInputPics is equal to 1, the following applies:

–	The luma sample array CroppedYPic[ i ] and the chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are set to be the 2-dimensional arrays of decoded sample values of the Y, Cb and Cr components, respectively, of sourcePic.

–	Otherwise (numInputPics is greater than 1), the following applies:

–	The variable sourceWidth is set equal to the value of CroppedWidth as specified by Formulae 7-23 for sourcePic.

–	The variable sourceHeight is set equal to the value of CroppedHeight as specified by Formulae 7-24 for sourcePic.

–	If sourceWidth is equal to CroppedWidth and sourceHeight is equal to CroppedHeight, resampledPic is set to be the same as sourcePic.

–	Otherwise (sourceWidth is not equal to CroppedWidth or sourceHeight is not equal to CroppedHeight), the following applies:

–	There shall be an NNPF, hereafter referred to as the super resolution NNPF, that is defined by at least one NNPFC SEI message, is activated by an NNPFA SEI message for sourcePic, and has nnpfc_purpose equal to 4, nnpfcOutputPicWidth equal to CroppedWidth and nnpfcOutputPicHeight equal to CroppedHeight.

–	resampledPic is set to be the output of the neural-network inference of the super resolution NNPF with sourcePic being an input.

–	The luma sample array CroppedYPic[ i ] and the chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are set to be the 2-dimensional arrays of decoded sample values of the Y, Cb and Cr components, respectively, of resampledPic.

–	BitDepthY and BitDepthC are set equal to BitDepthY and BitDepthC, respectively

–	ChromaFormatIdc is set equal to chroma_format_idc.

–	The array StrengthControlVal[ i ] for all values of i in the range of 0 to numInputPics − 1, inclusive, specifying the filtering strength control value for the input pictures for the NNPF, is derived as follows:

–	StrengthControlVal[ i ] is set equal to the value of ( firstSliceQpY + QpBdOffsetY ) ÷ ( 51 + QpBdOffsetY ), where firstSliceQpY is equal to SliceQpY of the first slice of inputPic[ i ].

There shall not be more than two NNPFC SEI messages present in a picture unit with the same value of nnpfc_id. When there are two NNPFC SEI messages present in a picture unit with the same value of nnpfc_id, these SEI messages shall have different content. When two NNPFC SEI messages with the same nnpfc_id and different content are present in the same picture unit, both of these NNPFC SEI messages shall be in the same SEI NAL unit.

    - Use of the phase indication SEI message

For purposes of interpretation of the phase indication SEI message, the following variables are specified:

–	CroppedWidth and CroppedHeight, set as specified by Formulae 7-23 and 7-24, respectively.

## In subclause E.2.1, starting with the sentence that says “The application of the transfer characteristics function is denoted by ( x )′ for an argument x.”, replace the remainder of the semantics of the matrix_coefficients syntax element with the following:

The application of the transfer characteristics function is denoted by ( x )′ for an argument x.

–	If matrix_coefficients is not equal to 14 or 15, the signals E′R, E′G, and E′B are determined by application of the transfer characteristics function as follows:

E′R = ( ER )′			(E-1)

E′G = ( EG )′			(E-2)

E′B = ( EB )′			(E-3)

In this case, the range of E′R, E′G, and E′B is specified as follows:

–	If transfer_characteristics is equal to 11 or 12, or transfer_characteristics is equal to 13 and matrix_coefficients is not equal to 0, E′R, E′G, and E′B are real numbers with values that have a larger range than the range of 0 to 1, and their range is not specified in this Specification.

–	Otherwise, E′R, E′G, and E′B are real numbers with values in the range of 0 to 1.

–	Otherwise (matrix_coefficients is equal to 14 or 15), the signals E′L, E′M, and E′S are determined by the following ordered steps:

a)	The "linear-domain" real-valued signals EL, EM, and ES are determined as follows:

–	If matrix_coefficients is equal to 14, the signals EL, EM, and ES are determined as follows:

EL = ( 1688 * ER + 2146 * EG + 262 * EB ) ÷ 4096      	(E-4)

EM = ( 683 * ER + 2951 * EG + 462 * EB ) ÷ 4096      	(E-5)

ES = ( 99 * ER + 309 * EG + 3688 * EB ) ÷ 4096        	(E-6)

–	Otherwise (matrix_coefficients is equal to 15), the signals EL, EM, and ES are determined as follows:

EL = ( 1747 * ER + 2169 * EG + 180 * EB ) ÷ 4096      	(E-7)

EM = ( 673 * ER + 3029 * EG + 394 * EB ) ÷ 4096      	(E-8)

ES = ( 50 * ER + 207 * EG + 3839 * EB ) ÷ 4096        	(E-9)

b)	The signals E′L, E′M, and E′S are determined by application of the transfer characteristics function as follows:

E′L = ( EL )′		(E-10)

E′M = ( EM )′		(E-11)

E′S = ( ES )′		(E-12)

When matrix_coefficients is equal to 0, 8, 16, or 17, the variables bitDepthRGB and maxValRGB are derived as follows:

a)	The variable bitDepthRGB is derived as follows:

–	If matrix_coefficients is equal to 0 or 8, the following applies:

bitDepthRGB = BitDepthY		(E-13)

–	Otherwise, if matrix_coefficients is equal to 16, the following applies:

bitDepthRGB = BitDepthY + 2		(E-14)

–	Otherwise (matrix_coefficients is equal to 17), the following applies:

bitDepthRGB = BitDepthY + 1		(E-15)

b)	The variable maxValRGB is derived as follows:

maxValRGB = ( 1 << bitDepthRGB ) − 1		(E-16)

The interpretation of matrix_coefficients is specified as follows:

–	If video_full_range_flag is equal to 0, the following applies:

–	If matrix_coefficients is equal to 1, 4, 5, 6, 7, 9, 10, 11, 12, 13, 14, or 15 the following equations apply:

Y = Clip1Y( Round( ( 1 << ( BitDepthY − 8 ) ) * ( 219 * E′Y + 16 ) ) )	(E-17)

Cb = Clip1C( Round( ( 1 << ( BitDepthC − 8 ) ) * ( 224 * E′PB + 128 ) ) )	(E-18)

Cr = Clip1C( Round( ( 1 << ( BitDepthC − 8 ) ) * ( 224 * E′PR + 128 ) ) )	(E-19)

–	Otherwise, if matrix_coefficients is equal to 0, 8, 16, or 17, the following equations apply:

R = Clip3( 0, maxValRGB, ( 1 << ( bitDepthRGB − 8 ) ) * ( 219 * E′R + 16 ) )	(E-20)

G = Clip3( 0, maxValRGB, ( 1 << ( bitDepthRGB − 8 ) ) * ( 219 * E′G + 16 ) )	(E-21)

B = Clip3( 0, maxValRGB, ( 1 << ( bitDepthRGB − 8 ) ) * ( 219 * E′B + 16 ) )	(E-22)

–	Otherwise, if matrix_coefficients is equal to 2, the interpretation of the matrix_coefficients syntax element is unknown or is determined by the application.

–	Otherwise (matrix_coefficients is not equal to 0, 1, 2, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, or 17), the interpretation of the matrix_coefficients syntax element is reserved for future definition by ITU-T | ISO/IEC.

–	Otherwise (video_full_range_flag is equal to 1), the following applies:

–	If matrix_coefficients is equal to 1, 4, 5, 6, 7, 9, 10, 11, 12, 13, 14, or 15, the following equations apply:

Y = Clip1Y( Round( ( ( 1 << BitDepthY ) − 1 ) * E′Y ) )	(E-23)

Cb = Clip1C( Round( ( ( 1 << BitDepthC ) − 1 ) * E′PB + ( 1 << ( BitDepthC − 1 ) ) ) )	(E-24)

Cr = Clip1C( Round( ( ( 1 << BitDepthC ) − 1 ) * E′PR + ( 1 << ( BitDepthC − 1 ) ) ) )	(E-25)

–	Otherwise, if matrix_coefficients is equal to 0, 8, 16, or 17, the following equations apply:

R = Clip3( 0, maxValRGB, maxValRGB * E′R )		(E-26)

G = Clip3( 0, maxValRGB, maxValRGB * E′G )		(E-27)

B = Clip3( 0, maxValRGB, maxValRGB * E′B )		(E-28)

–	Otherwise, if matrix_coefficients is equal to 2, the interpretation of the matrix_coefficients syntax element is unknown or is determined by the application.

–	Otherwise (matrix_coefficients is not equal to 0, 1, 2, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, or 17), the interpretation of the matrix_coefficients syntax element is reserved for future definition by ITU-T | ISO/IEC.

Reserved values for matrix_coefficients shall not be present in bitstreams conforming to this version of this Specification. Decoders shall interpret reserved values of matrix_coefficients as equivalent to the value 2.

It is a requirement of bitstream conformance to this version of this Specification that when colour_primaries is not equal to 1, 4, 5, 6, 7, 8, 9, 10, 11, 12, or 22, matrix_coefficients shall not be equal to 12 or 13.

When matrix_coefficients is equal to 1, 4, 5, 6, 7, 9, 10, 11, 12, or 13, the constants KR and KB are specified as follows:

–	If matrix_coefficients is not equal to 12 or 13, the constants KR and KB are specified in Table E-5.

–	Otherwise (matrix_coefficients is equal to 12 or 13), the constants KR and KB are computed as follows, using the chromaticity coordinates (xR, yR), (xG, yG), (xB, yB), and (xW, yW) specified by Table E-3 for the colour_primaries syntax element for the red, green, blue, and white colour primaries, respectively:

KR = 	(E-29)

KB = 	(E-30)

where the values of zR, zG, zB, and zW, are given by.

zR = 1 − ( xR + yR )		(E-31)

zG = 1 − ( xG + yG )		(E-32)

zB = 1 − ( xB + yB )		(E-33)

zW = 1 − ( xW + yW )		(E-34)

The variables E′Y, E′PB, and E′PR (for matrix_coefficients not equal to 0, 8, 16, or 17) or Y, Cb, and Cr (for matrix_coefficients equal to 0, 8, 16, or 17) are specified as follows:

–	If matrix_coefficients is not equal to 0, 8, 10, 11, 13, 14, 15, 16, or 17, the following equations apply:

E′Y = KR * E′R + ( 1 − KR − KB ) * E′G + KB * E′B		(E-35)

E′PB = 0.5 * ( E′B − E′Y ) ÷ ( 1 − KB )		(E-36)

E′PR = 0.5 * ( E′R − E′Y ) ÷ ( 1 − KR )		(E-37)

E′Y is a real number with the value 0 associated with nominal black and the value 1 associated with nominal white. E′PB and E′PR are real numbers with the value 0 associated with both nominal black and nominal white. When transfer_characteristics is not equal to 11 or 12, E′Y is a real number with values in the range of 0 to 1. When transfer_characteristics is not equal to 11 or 12, E′PB and E′PR are real numbers with values in the range of −0.5 to 0.5. When transfer_characteristics is equal to 11, or 12, E′Y, E′PB and E′PR are real numbers with a larger range not specified in this Specification.

–	Otherwise, if matrix_coefficients is equal to 0, the following equations apply:

Y   = Round( G )		(E-38)

Cb = Round( B )		(E-39)

Cr = Round( R )		(E-40)

–	Otherwise, if matrix_coefficients is equal to 8 and BitDepthC is equal to BitDepthY, the following applies:

Y   = Round( 0.5 * G + 0.25 * ( R + B ) )		(E-41)

Cb = Round( 0.5 * G − 0.25 * ( R + B ) ) + ( 1 << ( BitDepthC − 1 ) )	(E-42)

Cr = Round( 0.5 * ( R − B ) ) + ( 1 << ( BitDepthC − 1 ) )	(E-43)

NOTE 6 – In this case, for purposes of the YCgCo nomenclature used in Table E-5, Cb and Cr of Equations E-42 and E-43 would be referred to as Cg and Co, respectively. An appropriate inverse conversion for Equations E-41 to E-43 is as follows:

t   = Y − ( Cb − ( 1 << ( BitDepthC − 1 ) ) )		(E-44)

G = Clip1Y( Y + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-45)

B = Clip1Y( t − ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )		(E-46)

R = Clip1Y( t + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )		(E-47)

–	Otherwise, if matrix_coefficients is equal to 8, 16, or 17, the following equations apply:

Cr = Round( R ) − Round( B ) + ( 1 << ( BitDepthC − 1 ) )	(E-48)

t = Round( B ) + ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(E-49)

Cb = Round( G ) − t + ( 1 << ( BitDepthC − 1 ) )		(E-50)

Y = t + ( ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )		(E-51)

NOTE 7 – In this case, for purposes of the YCgCo nomenclature used in Table E-5, Cb and Cr of Equations E-50 and E-48 would be referred to as Cg and Co, respectively. An appropriate inverse conversion for Equations E-48 to E-51 is as follows:

t   = Y − ( ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(E-52)

G = Clip3( 0, maxValRGB, t + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-53)

B = Clip3( 0, maxValRGB, t − ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) )	(E-54)

R = Clip3( 0, maxValRGB, B + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-55)

–	Otherwise, if matrix_coefficients is equal to 10 or 13, the signal E′Y is determined by application of the transfer characteristics function as follows:

EY = KR * ER + ( 1 − KR − KB ) * EG + KB * EB		(E-56)

E′Y = ( EY )′				(E-57)

In this case, EY is defined from the "linear-domain" signals for ER, EG, and EB, prior to application of the transfer characteristics function, which is then applied to produce the signal E′Y. EY and E′Y are real values with the value 0 associated with nominal black and the value 1 associated with nominal white.

In this case, the signals E′PB and E′PR are determined as follows:

E′PB = ( E′B − E′Y ) ÷ ( 2 * NB )      for −NB  <=  E′B − E′Y  <=  0	(E-58)

E′PB = ( E′B − E′Y ) ÷ ( 2 * PB )      for 0  <  E′B − E′Y  <= PB	(E-59)

E′PR = ( E′R − E′Y ) ÷ ( 2 * NR )      for −NR  <=  E′R − E′Y  <=  0	(E-60)

E′PR = ( E′R − E′Y ) ÷ ( 2 * PR )      for 0  <  E′R − E′Y  <=  PR	(E-61)

where the constants NB, PB, NR, and PR are determined by application of the transfer characteristics function to expressions involving the constants KB and KR as follows:

NB = ( 1 − KB )′		(E-62)

PB = 1 − ( KB )′ 		(E-63)

NR = ( 1 − KR )′		(E-64)

PR = 1 − ( KR )′			(E-65)

–	Otherwise, if matrix_coefficients is equal to 11, the following equations apply:

E′Y = E′G		(E-66)

E′PB = ( 0.986566 * E′B − E′Y ) ÷ 2	 	(E-67)

E′PR = ( E′R − 0.991902 * E′Y ) ÷ 2		(E-68)

NOTE 8 – In this case, for purposes of the Y′D′ZD′X nomenclature used in Table E-5, E′PB and E′PR of Equations E-67 and E-68 would be referred to as D′Z and D′X, respectively.

–	Otherwise, if matrix_coefficients is equal to 14, the following applies:

–	If transfer_characteristics is not equal to 18, the following equations apply:

E′Y = 0.5 * ( E′L + E′M )							(E-69)

E′PB = ( 6610 * E′L − 13613 * E′M + 7003 * E′S ) ÷ 4096	(E-70)

E′PR = ( 17933 * E′L − 17390 * E′M − 543 * E′S ) ÷ 4096	(E-71)

–	Otherwise, the following equations apply:

E′Y = 0.5 * ( E′L + E′M )							(E-72)

E′PB = ( 3625 * E′L − 7465 * E′M + 3840 * E′S ) ÷ 4096	(E-73)

E′PR = ( 9500 * E′L − 9212 * E′M − 288 * E′S ) ÷ 4096	(E-74)

NOTE 9 – In this case, for purposes of the ICTCP nomenclature used in Table E-5, E′Y, E′PB, and E′PR of Equations E-69, E-70, and E-71 may be referred to as I, CT, and CP, respectively. Equations E-69 through E-71 were designed specifically for use with transfer_characteristics equal to 16 (PQ), and Equations E-72 through E-74 were designed specifically for use with transfer_characteristics equal to 18 (HLG).

–	Otherwise (matrix_coefficients is equal to 15), the following applies:

E′Y = ( 1638 * E′L + 1638 * E′M +  820 * E′S ) ÷ 4096		(E-75)

E′PB = ( 18248 * E′L − 19870 * E′M + 1622 * E′S ) ÷ 4096	(E-76)

E′PR = ( 3300 * E′L + 1463 * E′M − 4763 * E′S ) ÷ 4096	(E-77)

NOTE 10 – In this case, for purposes of the IPT-C2 nomenclature used in Table E-5, E′Y, E′PB, and E′PR of Equations E-75, E-76, and E-77 may be referred to as I, P, and T, respectively.

Table E-5 – Matrix coefficients interpretation using the matrix_coefficients syntax element

| Value | Matrix | Informative remark |
| --- | --- | --- |
| 0 | GBR | The identity matrix. Typically used for GBR (often referred to as RGB); however, may also be used for YZX (often referred to as XYZ) IEC 61966-2-1 (sRGB) Society of Motion Picture and Television Engineers ST 428-1 XYZ (2006) See Equations E -1 to E-3 |
| 1 | KR = 0.2126; KB = 0.0722 | Rec. ITU-R BT.709-6 Rec. ITU-R BT.1361-0 conventional colour gamut system and extended colour gamut system (historical) IEC 61966-2-4 xvYCC709 Society of Motion Picture and Television Engineers RP 177 (1993) Annex B See Equations E-35 to E-37 |
| 2 | Unspecified | Image characteristics are unknown or are determined by the application. |
| 3 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 4 | KR = 0.30; KB = 0.11 | United States Federal Communications Commission Title 47 Code of Federal Regulations (2016) 73.682 (a) (20) See Equations E-35 to E-37 |
| 5 | KR = 0.299; KB = 0.114 | Rec. ITU-R BT.470-7 System B, G (historical) Rec. ITU-R BT.601-7 625 Rec. ITU-R BT.1358-0 625 (historical) Rec. ITU-R BT.1700-0 625 PAL and 625 SECAM IEC 61966-2-1 (sYCC) IEC 61966-2-4 xvYCC601 (functionally the same as the value 6) See Equations E-35 to E-37 |
| 6 | KR = 0.299; KB = 0.114 | Rec. ITU-R BT.601-7 525 Rec. ITU-R BT.1358-1 525 or 625 Rec. ITU-R BT.1700-0 NTSC Society of Motion Picture and Television Engineers 170M (2004) (functionally the same as the value 5) See Equations E-35 to E-37 |
| 7 | KR = 0.212; KB = 0.087 | Society of Motion Picture and Television Engineers 240M (1999, historical) See Equations E-35 to E-37 |
| 8 | YCgCo or YCgCo-R | See Equations E-41 to E-47 for YCgCo (when BitDepthC is equal to BitDepthY) See Equations E-48 to E-55 for YCgCo-R (when BitDepthC is equal to BitDepthY + 1) |
| 9 | KR = 0.2627; KB = 0.0593 | Rec. ITU-R BT.2020-2 non-constant luminance system Rec. ITU-R BT.2100-2 Y′CbCr See Equations E-35 to E-37 |
| 10 | KR = 0.2627; KB = 0.0593 | Rec. ITU-R BT.2020-2 constant luminance system See Equations E-56 to E-65 |
| 11 | Y′D′ZD′X | Society of Motion Picture and Television Engineers ST 2085 (2015) See Equations E-66 to E-68 |
| 12 | See Equations E-29 to E-34. | Chromaticity-derived non-constant luminance system See Equations E-35 to E-37. |
| 13 | See Equations E-29 to E-34. | Chromaticity-derived constant luminance system See Equations E-56 to E-65. |
| 14 | ICTCP | Rec. ITU-R BT.2100-2 ICTCP See Equations E-69 to E-71. for transfer_characteristics value 16 (PQ) See Equations E-72 to E-74 for transfer_characteristics value 18 (HLG) |
| 15 | IPT-C2 | Society of Motion Picture and Television Engineers ST 2128 See Equations E-75 to E-77 |
| 16 | YCgCo-Re | See Equations E-48 to E-55 |
| 17 | YCgCo-Ro | See Equations E-48 to E-55 |
| 18..255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

In subclause E.2.1, make the following changes:

log2_max_mv_length_horizontal and log2_max_mv_length_vertical indicate the maximum absolute value range of a decoded horizontal and vertical motion vector component, respectively, in ¼ luma sample units, for all pictures in the coded video sequence. A value of n asserts that no value of a motion vector component shall exceed the range from −2n to 2n − 1, inclusive, in units of ¼ luma sample displacement. The value of log2_max_mv_length_horizontal shall be in the range of 0 to 15, inclusive. The value of log2_max_mv_length_vertical shall be in the range of 0 to 15, inclusive. When log2_max_mv_length_horizontal is not present, the values of log2_max_mv_length_horizontal and log2_max_mv_length_vertical shall be inferred to be equal to 15.

NOTE 15 – The maximum absolute value range of a decoded vertical or horizontal motion vector component is also constrained by profile and level limits as specified in Annex ‎A and clauses ‎G.10 and ‎H.10.

For bitstreams of levels 6, 6.1, and 6.2 (see Annex A), it has been reported that some decoder implementations do not properly decode bitstreams that have horizontal motion vector component values that exceed the range from −2048 to 2047, inclusive, or that have vertical motion vector component values that exceed the range from −512 to 511, inclusive, in units of ¼ luma sample displacement. It is therefore suggested that bitstreams should not contain motion vector component values that exceed these ranges. This constraint can be indicated by using values of log2_max_mv_length_horizontal less than or equal to 11 and values of log2_max_mv_length_vertical less than or equal to 9.

In subclause F.9.2 (SEI payload semantics), make the following changes:

The semantics of the SEI messages with payloadType in the range of 0 to 23, inclusive, or equal to 45, 47, 137, 142, 144, 147, 148, 149, 150, 151, 154, 155, 156, 200, 201, 202, or 205, 210, 211, or 212, which are specified in Clause D.2, are extended as follows:

…

–	Otherwise, if payloadType is equal to 2, 6, 7, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 21, 23, 45, 47, 137, 142, 144, 147, 148, 149, 150, 151, 154, 155, 156, 200, 201, 202, or 205, 210, 211, or 212, the following applies:

…

In subclause G.13.2.1, make the following changes:

log2_max_mv_length_horizontal[ i ] and log2_max_mv_length_vertical[ i ] indicate the values of log2_max_mv_length_horizontal and log2_max_mv_length_vertical, as specified in clause ‎E.2.1, that apply to the current scalable layer representation.

NOTE 17 – The maximum absolute value range of a decoded vertical or horizontal motion vector component is also constrained by profile and level limits as specified in Annex ‎A and clause ‎G.10.

In subclause H.13.2.3, make the following changes:

log2_max_mv_length_horizontal[ i ] and log2_max_mv_length_vertical[ i ] specify the values of log2_max_mv_length_horizontal and log2_max_mv_length_vertical, as specified in clause ‎E.2.1, for the current operation point representation. When log2_max_mv_length_horizontal[ i ] is not present, the values of log2_max_mv_length_horizontal and log2_max_mv_length_vertical for the current operation point representation shall be inferred to be equal to 16. The value of log2_max_mv_length_horizontal[ i ] shall be in the range of 0 to 16, inclusive. The value of log2_max_mv_length_vertical[ i ] shall be in the range of 0 to 16, inclusive.

NOTE 3 – The maximum absolute value range of a decoded vertical or horizontal motion vector component is also constrained by profile and level limits as specified in Annex ‎A or clause ‎H.10.2.