| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0006-v1 |
| --- | --- |

| Title: | JVET AHG report: ECM software development (AHG6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Roman Chernyak Fabrice Le Léannec Kai Zhang | Tel: Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com chernyak@global.tencent.com Fabrice.Leleannec@InterDigital.com zhangkai.video@bytedance.com |
| Source: | AHG6 on ECM software development |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AHG6 on ECM software development that has taken place between the 31st and 32nd JVET meetings.

# Introduction

The mandates for the AHG are as follows:

- Coordinate development of the ECM software and associated configuration files.

- Produce documentation of software usage for distribution with the software.

- Prepare and deliver ECM-10.0 software version and the reference configuration encodings according to the ECM common test conditions.

- Investigate encoder speedup and other encoder software optimization such as reduction of memory consumption.

- Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate.

# Software development

ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

VTM-11.0ecm anchor https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC is used for ECM performance evaluation.

The following adopted aspects were integrated into ECM-10.0:

JVET-AE0091: High-Accuracy template matching (Test 3.3e) (MR 449)

JVET-AE0174: Add non-inter TM SPS flag for tool control (MR 452)

JVET-AE0139(Test 5.2c): Improved ALF fixed filters (MR 450)

JVET-AE0059(Test3.1b): Convolutional cross-component model for inter prediction (MR 453)

JVET-AE0169 (Test 2.2c/2.3b): Bi-predictive IBC (MR 448)

JVET-AE0100 (Test 2.1b): Block vector guided CCCM (MR 455)

JVET-AE0125 (Test 4.1) Shifting Quantization Center (MR 461)

JVET-AE0150: RPR filters and thresholds for scale factors 1.1x to 1.35x (MR 468)

JVET-AE0077(Test-2.9): Extended search areas for IntraTMP mode (MR 458)

JVET-AE0094(Test 2.6c): IBC with non-adjacent spatial candidates (MR 457)

JVET-AE0086(Test-4.2): Large NSPT (MR 466)

JVET-AE0091: Iterative BDOF Part (Test 3.5) (MR 460)

JVET-AE0084(Test-2.11a): Harmonization of IBC HMVP and IBC-LIC (MR 459)

JVET-AE0046(Test 3.2): Bi-predictive GPM (MR 465)

JVET-AE0159(Test 2.5c): Filtered Intra Block Copy (MR 462)

JVET-AE0169: IBC MBVD list construction (Test 2.4a) (MR 469)

JVET-AE0097: Remove offset update process in CCP merge (MR 463)

JVET-AE0043 (Test 2.7): Cross-component merge mode with temporal candidates (MR 467)

JVET-AE0102: LFNST/NSPT coefficients context (Test 4.3) (MR 478)

JVET-AE0151 (Test5.1b): CCSAO with history offsets and extended edge classifiers (MR 472)

JVET-AE0278: ALF, CCALF memory reduction (lossless change) (MR 480)

JVET-AE0078: IBC-LIC extension (Test 2.10a) (MR 482)

JVET-AE0136: Fix on TM-based reordering for affine MMVD (MR 456)

JVET-AE0162: fix MS-SSIM calculation for RPR/SR (MR 475)

JVET-AE0055: Add NumSignPredOverrideByQP to LD configurations (MR 454)

Bug fixes:

Fix: memory release for JVET_AB0155_SGPM (MR 452)

Fixes for DebugStream tool (MRs 488, 489, 490, 502)

Fix: buffer copy for AC0119 LM chroma fusion (MR 484)

Fix condition for checking buffer overflow (MR 494)

Fix: add initialization for BIF overhead parsing and copy (MR 504)

Fix: Add checks for scaled collocated picture, inter check for GPM (MR 508)

Fix: Valgrind error in spanGeoMMVDMotionInfo (MR 514)

Fix: use picture level reconstructed buffer for SBT sub-TUs in BIF (MR 512)

The following adopted aspects were integrated into VTM-11ecm anchor:

JVET-AE0057: MTT split modes early termination (MR 474)

JVET-AE0162: fix MS-SSIM calculation for RPR/SR (MR 476)

ECM-10.0 and VTM-11ecm10.0 were tagged on August 29, 2023.

## CTC Performance

In this section, ECM test results following ECM CTC configuration descried in JVET-AE2017 are summarized.

Next tables show ECM-10.0 performance over ECM-9.1 anchor.

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.93% | -0.63% | -1.17% | 108.3% | 102.2% | 74.2% |
| Class A2 | -1.02% | -1.59% | -1.37% | 106.5% | 100.6% | 74.2% |
| Class B | -1.06% | -1.10% | -1.31% | 106.0% | 102.1% | 70.5% |
| Class C | -0.89% | -0.66% | -0.80% | 110.1% | 103.5% | 87.7% |
| Class E | -1.06% | -1.05% | -0.98% | 105.8% | 102.1% | 79.9% |
| Overall | -1.00% | -1.00% | -1.13% | 107.3% | 102.2% | 76.9% |
| Class D | -0.62% | -0.79% | -0.72% | 108.0% | 99.2% | 95.1% |
| Class F | -2.56% | -2.81% | -3.05% | 106.7% | 103.7% | 80.1% |
| Class TGM | -3.21% | -3.53% | -3.50% | 104.9% | 108.7% | 71.4% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-9.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -1.74% | -3.59% | -6.37% | 108.4% | 97.7% | 79.4% |
| Class A2 | -2.07% | -4.14% | -5.39% | 106.1% | 96.0% | 79.6% |
| Class B | -1.24% | -3.84% | -3.50% | 103.1% | 95.7% | 72.3% |
| Class C | -1.25% | -2.08% | -1.93% | 105.3% | 99.5% | 85.5% |
| Class E |  |  |  |  |  |  |
| Overall | -1.51% | -3.38% | -4.04% | 105.3% | 97.2% | 78.5% |
| Class D | -1.27% | -2.12% | -2.15% | 105.8% | 100.2% | 92.8% |
| Class F | -2.13% | -2.98% | -3.03% | 107.9% | 97.6% | 80.2% |
| Class TGM | -2.23% | -2.82% | -2.95% | 102.1% | 90.8% | 76.0% |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over ECM-9.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.69% | -8.86% | -7.04% | 94.0% | 98.5% | 69.8% |
| Class C | -0.97% | -3.14% | -3.09% | 106.7% | 101.6% | 86.2% |
| Class E | -0.98% | -3.47% | -3.09% | 105.8% | 98.2% | 78.5% |
| Overall | -0.86% | -5.61% | -4.74% | 101.0% | 99.4% | 77.1% |
| Class D | -0.87% | -4.16% | -3.75% | 105.0% | 99.7% | 94.0% |
| Class F | -1.59% | -6.03% | -7.65% | 106.9% | 95.9% | 79.7% |
| Class TGM | -1.98% | -4.41% | -5.46% | 103.8% | 94.5% | 74.6% |
|  | Low delay P Main 10 |  |  |  |  |  |
|  | Over ECM-9.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.42% | -8.57% | -7.36% | 93.1% | 95.5% | 69.6% |
| Class C | -0.84% | -3.48% | -3.68% | 105.2% | 94.6% | 86.3% |
| Class E | -0.59% | -2.68% | -2.66% | 107.0% | 98.0% | 78.5% |
| Overall | -0.60% | -5.40% | -4.96% | 100.4% | 95.8% | 77.1% |
| Class D | -1.01% | -2.98% | -4.80% | 105.1% | 95.4% | 94.2% |
| Class F | -2.00% | -6.37% | -6.84% | 109.5% | 96.1% | 79.5% |
| Class TGM | -2.71% | -5.41% | -6.28% | 105.5% | 98.0% | 73.5% |

The below tables show ECM-10.0 performance comparing to VTM-11.0ecm10.0 anchor.

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm10 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -10.55% | -21.81% | -29.24% | 915.8% | 439.3% | #DIV/0! |
| Class A2 | -16.68% | -28.60% | -31.54% | 916.2% | 458.0% | #DIV/0! |
| Class B | -11.02% | -26.58% | -24.82% | 826.4% | 455.0% | #DIV/0! |
| Class C | -11.07% | -16.52% | -17.06% | 898.0% | 452.7% | #DIV/0! |
| Class E | -14.59% | -24.40% | -22.89% | 804.1% | 467.8% | #DIV/0! |
| Overall | -12.49% | -23.52% | -24.63% | 867.2% | 454.4% | #DIV/0! |
| Class D | -9.17% | -14.38% | -14.35% | 851.0% | 445.1% | #DIV/0! |
| Class F | -26.21% | -35.26% | -35.17% | 541.1% | 452.7% | #DIV/0! |
| Class TGM | -39.28% | -47.12% | -46.45% | 457.3% | 496.2% | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over VTM-11.0ecm10 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -22.25% | -27.01% | -36.07% | 882.3% | 784.9% | #DIV/0! |
| Class A2 | -25.50% | -35.13% | -38.75% | 791.3% | 889.0% | #DIV/0! |
| Class B | -20.50% | -34.42% | -32.13% | 673.3% | 731.6% | #DIV/0! |
| Class C | -21.91% | -26.29% | -26.07% | 708.2% | 777.7% | #DIV/0! |
| Class E |  |  |  |  |  |  |
| Overall | -22.23% | -30.91% | -32.63% | 744.0% | 784.1% | #DIV/0! |
| Class D | -22.87% | -27.80% | -28.10% | 738.6% | 851.2% | #DIV/0! |
| Class F | -28.30% | -36.71% | -37.10% | 602.2% | 482.5% | #DIV/0! |
| Class TGM | -36.63% | -43.02% | -43.08% | 570.5% | 415.0% | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over VTM-11.0ecm10 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -17.55% | -38.06% | -35.34% | 589.9% | 668.2% | #DIV/0! |
| Class C | -19.29% | -28.96% | -29.31% | 648.1% | 710.0% | #DIV/0! |
| Class E | -16.12% | -28.03% | -27.57% | 642.1% | 459.1% | #DIV/0! |
| Overall | -17.77% | -32.52% | -31.39% | 621.7% | 620.8% | #DIV/0! |
| Class D | -21.17% | -30.48% | -30.27% | 656.7% | 758.7% | #DIV/0! |
| Class F | -24.60% | -37.20% | -36.64% | 588.5% | 498.3% | #DIV/0! |
| Class TGM | -35.27% | -43.12% | -43.29% | 560.3% | 418.2% | #DIV/0! |
|  | Low delay P Main 10 |  |  |  |  |  |
|  | Over VTM-11.0ecm10 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -16.23% | -37.00% | -34.66% | 529.3% | 651.6% | #DIV/0! |
| Class C | -18.30% | -28.47% | -28.23% | 561.3% | 660.2% | #DIV/0! |
| Class E | -15.42% | -26.79% | -27.20% | 626.8% | 468.8% | #DIV/0! |
| Overall | -16.72% | -31.60% | -30.65% | 563.0% | 602.8% | #DIV/0! |
| Class D | -21.10% | -29.72% | -30.38% | 592.5% | 707.9% | #DIV/0! |
| Class F | -23.63% | -37.06% | -36.33% | 602.7% | 467.7% | #DIV/0! |
| Class TGM | -32.74% | -41.78% | -41.94% | 624.2% | 417.1% | #DIV/0! |

# ECM memory consumption

ECM memory consumption (VmPeak, GiB) is provided in ECM encoder log files and is summarized in the table below.

|  | AI | RA | LB |
| --- | --- | --- | --- |
| class A (A1& A2) | 9 | 20 |  |
| class B | 7 | 10 | 8 |
| class C | 5 | 6 | 5 |
| class D | 5 | 5 | 5 |
| class E | 6 |  | 6 |
| class F | 8 | 10 | 8 |
| class TGM | 8 | 12 | 11 |

It is encouraged to care about memory allocation when developing and integrating tools into ECM. In particular, it is strongly recommended to re-use already existed memory wherever possible, rather than systematically allocating new memory, and allocate only the required amount of memory if new memory is needed.

# Recommendations

The AHG recommends to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.

- Encourage people to continue working on ECM memory consumption reduction.

- Encourage people to continue working on speeding up ECM encoder to reduce the simulation time.