| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0011-v1 |
| --- | --- |

| Title: | JVET AHG report: Neural network-based video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Shan Liu Andrew Segall Franck Galpin Junru Li Ru-Ling Liao Dmytro Rusanovskyy Tong Shao Mathias Wien Ping Wu | Email: | elena.alshina@huawei.com shanl@global.tencent.com asegall@amazon.com Franck.Galpin@InterDigital.com lijunru@bytedance.com ruling.lrl@alibaba-inc.com dmytror@qti.qualcomm.com tshao@dolby.com wien@lfb.rwth-aachen.de pwuzte@gmail.com |
| Source: | AHG11 coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities of AHG11: Neural network-based video coding between the 31st meeting (11 – 19 July 2023) held in Geneva, CH and the 32nd meeting (13 – 20 October 2023) held in Hannover, DE.

# Introduction

The AHG was established with the following mandates:

- Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools and novel architectures.

- Discuss potential refinements of the test conditions for NN-based video coding in JVET-AD2016. Generate and distribute anchor encoding, and develop supporting software as needed.

- Study the impact of training (including the impact of loss functions) on the performance of candidate technologies, and identify suitable material for testing and training.

- Analyse complexity characteristics, perform complexity analysis, and develop complexity reductions of candidate technology.

- Finalize and discuss the EE on neural network-based video coding.

- Coordinate with other relevant groups, including SC29/AG5 on the evaluation and assessment of visual quality, and AHG12 on the interaction with ECM coding tools. If possible, prepare encodings with combinations of tools included in the NNVC software for visual quality assessment at the next meeting.

- Coordinate with AHG14 on items related to NNVC software development.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, for email. Nine emails were exchanged on the reflector related to the AHG mandates.

## Common Test Conditions

### Document

The AHG released revised common test conditions as decided at the 31st meeting, including the following changes:

- Define the software anchor to be either NNVC-6.0 or the latest version of NNVC.

- Section 2 (Editorial): Change “a set” to “the set”.

- Section 3.3 (Editorial): Clarify how decoder runtime should be calculated when parallel processing is used.  Specifically, clarify that the decoding run-time can be calculated in the same way as what’s defined for PSNR and MS-SSIM. That is, it can be calculated by either concatenated the bit-streams and performing a single decode or summing up the parallel decoding runtimes across segments.

- Other editorial improvements.

### Anchor Encoding

Anchors for the NN-based video coding activity made available on the Git repository used for the AHG activity: https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master.

## EE Coordination

The AHG finalized, conducted, and discussed the EE on NN based video coding.  A summary report for the EE is available at this meeting as:

| JVET-AF0023 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang, Z. Xie (EE coordinators) |
| --- | --- | --- |

## Teleconferences

The AHG conducted a joint teleconference with AHG14 and EE1 during the interim period. The teleconference was held on August 9, 2023. In this teleconference the progress of High Operation Point (HOP) NN-based filter training was discussed. Authors of JVET-AE0191 presented results of the third (last) stage training. Final results were reported in JVET-AF0041.

Low Operation Point (LOP) unified filter training status was also discussed.

On August 30, 2023, a second joint teleconference with AHG 14 was held to discuss further LOP training status. The training process and test results for each stage of training and inference testing are reported in JVET-AF0043.The meeting minutes are reported in JVET-AF0042. :

## Performance Evaluation

The performance of the NNVC-6.0 anchor compared to VTM anchor is reported below. By default, NN-Intra and Low Operation Point (LOP.1) filter are enabled in NNVC-6.0.

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | -7.2% | -4.9% | -6.0% | -8.7% | -4.1% | -5.1% | 134% | 7667% | 96% |
| Class A2 | -6.6% | -5.8% | -5.1% | -6.5% | -4.1% | -2.9% | 130% | 7183% | 97% |
| Class B | -6.3% | -8.2% | -7.5% | -6.0% | -6.1% | -5.2% | 132% | 7834% | 98% |
| Class C | -6.6% | -10.5% | -9.6% | -6.3% | -7.9% | -5.6% | 132% | 7422% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -6.6% | -7.7% | -7.3% | -6.7% | -5.8% | -4.8% | 132% | 7557% | 97% |
| Class D | -7.6% | -9.4% | -9.7% | -5.8% | -6.8% | -5.9% | 129% | 8193% | 98% |
| Class F | -3.1% | -5.9% | -4.9% | -3.0% | -4.8% | -3.5% | 148% | 9739% | 99% |
| Class H |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -4.8% | -8.1% | -7.8% | -5.1% | -6.2% | -6.6% | 134% | 6812% | 99% |
| Class C | -5.2% | -11.1% | -9.0% | -5.6% | -10.7% | -4.0% | 126% | 6233% | 99% |
| Class E | -4.9% | -4.1% | -5.2% | -5.6% | -2.2% | -3.4% | 162% | 9877% | 98% |
| Overall | -5.0% | -8.1% | -7.6% | -5.4% | -6.7% | -4.9% | 138% | 7257% | 99% |
| Class D | -6.5% | -10.8% | -9.8% | -5.8% | -9.5% | -6.8% | 125% | 6400% | 98% |
| Class F | -2.0% | -4.9% | -4.5% | -2.3% | -3.1% | -4.1% | 148% | 8941% | 99% |
| Class H |  |  |  |  |  |  |  |  |  |
|  | All Intra Main10 |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | -8.5% | -7.2% | -8.7% | -10.0% | -8.0% | -7.5% | 210% | 5134% | 97% |
| Class A2 | -6.6% | -8.0% | -7.9% | -6.9% | -6.4% | -5.4% | 204% | 4416% | 98% |
| Class B | -7.0% | -8.6% | -8.7% | -7.0% | -7.6% | -7.0% | 203% | 4493% | 98% |
| Class C | -7.3% | -9.8% | -9.9% | -7.6% | -8.4% | -7.6% | 190% | 3770% | 98% |
| Class E | -10.3% | -9.0% | -8.8% | -10.1% | -7.5% | -6.4% | 199% | 5149% | 96% |
| Overall | -7.8% | -8.6% | -8.9% | -8.2% | -7.6% | -6.9% | 201% | 4507% | 97% |
| Class D | -7.6% | -8.1% | -9.3% | -7.2% | -6.8% | -7.6% | 180% | 3760% | 98% |
| Class F | -4.6% | -6.0% | -5.6% | -4.2% | -4.8% | -4.4% | 155% | 3961% | 98% |
| Class H |  |  |  |  |  |  |  |  |  |

Compression Performance of other tools available in NNVC depending on two complexity metrics (kMAC/pxl and total member for parameters) is shown on plots below. Performance of tools under study in EE1 can be found in EE1 report.

# Input contributions

There are 35 input contributions related to the AHG mandates. The list of input contributions is provided below.

## EE and Related Input Contributions

| Reporting |  |  |
| --- | --- | --- |
| JVET-AF0023 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang, Z. Xie |
| EE Technology |  |  |
| JVET-AF0043 | AhG11/EE1: Status of the joint EE1-0 (LOP.2) training | D. Rusanovskyy, Y. Li (Qualcomm), T. Shao, P. Yin, S. McCarthy (Dolby), J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya (Ittiam), J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang (Bytedance), R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AF0056 | EE1-1.2.2: Content-adaptive LOP filter | R. Yang, M. Santamaria, F. Cricri, H. Zhang, J. Lainema, M. M. Hannuksela, A. Hallapuro (Nokia) |
| JVET-AF0071 | EE1-related: Further complexity reduction on the joint EE1-0 (LOP.2) unified filter | T. Shao, P. Yin, S. McCarthy (Dolby), J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya (Ittiam) |
| JVET-AF0085 | EE1-1.2.1: On residual adjustments for NNLF | Z. Dai, Y. Yu, H. Yu, D. Wang (OPPO) |
| JVET-AF0086 | EE1-1.2.6: On flipping of input and output of model in NNVC HOP filter | Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO) |
| JVET-AF0102 | EE1-1.1.2 Complexity-performance tradeoff of decomposition | D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm) |
| JVET-AF0103 | EE1-1.1.3 Study on input feature set optimization | D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm) |
| JVET-AF0139 | EE1-3.1: Neural network-based intra prediction with reduced complexity | T. Dumas, F. Galpin, P. Bordes (InterDigital) |
| JVET-AF0153 | EE1-1.1.1: Optimization for complexity-performance trade-off of HOP network | R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AF0154 | EE1-1.2.3: Input and output rotation of model for NNVC in-loop filter | R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AF0181 | EE1-1.1.4.a: Simplified feature extraction for HOP | Y. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AF0182 | EE1-1.1.4.b: Group convolution for HOP | Y. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AF0183 | EE1-1.1.4.c/d: Separate models for HOP | Y. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AF0192 | EE1-1.1.5: Combination test of EE1-1.1.1 and EE1-1.1.2 | R. Chang, L. Wang, X. Xu, S. Liu (Tencent), D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm) |
| JVET-AF0205 | EE1-1.2.4: An improved inference design of NN-based loop-filters at high operation point | J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang (Bytedance) |
| JVET-AF0208 | EE1-2.1: Deep Reference Frame Generation for Inter Prediction Enhancement | W. Bao, X. Chen, J. Jia, Y. Zhang, Z. Chen (Wuhan Univ.), Z. Liu, X. Xu, S. Liu (Tencent) |
| Cross Checks |  |  |
| JVET-AF0119 | Crosscheck of JVET-AF0056 (EE1-1.2.2: Content-adaptive LOP filter) | J. N. Shingala, A. Shyam, S. P. Badya (Ittiam) |
| JVET-AF0216 | Crosscheck of JVET-AF0056 (EE1-1.2.2: Content-adaptive LOP filter) | Z. Dai (OPPO) |
| JVET-AF0217 | Crosscheck of JVET-AF0139 (EE1-3.1: neural network-based intra prediction with reduced complexity) | Z. Dai (OPPO) |
| JVET-AF0254 | Cross-check of JVET-AF0085 (EE1-1.2.1: on residual adjustments of NNLF) | T. Dumas (Interdigital) |
| JVET-AF0257 | Crosscheck of JVET-AF0154 (EE1-1.2.3: Input and output rotation of model for NNVC in-loop filter)) | D. Liu (Ericsson) |
| JVET-AF0258 | Crosscheck of JVET-AF0103 (EE1-1.1.3 Study on input feature set optimization) | F. Galpin (InterDigital) |
| JVET-AF0260 | Crosscheck of JVET-AF0183 (EE1-1.1.4.c/d: Separate models for HOP) | F. Galpin (InterDigital) |

## Non-EE Input Contributions

| Reporting |  |  |
| --- | --- | --- |
| JVET-AF0011 | JVET AHG report: Neural network-based video coding (AHG11) | E. Alshina, F. Galpin, S. Liu, A. Segall (co chairs), J. Li, R.-L. Liao, D. Rusanovskyy, T. Shao, M. Wien, P. Wu (vice chairs) |
| JVET-AF0041 | AhG11: HOP full results | F. Galpin (InterDigital), D. Rusanovskyy, Y. Li (Qualcomm), Y. Li, J. Li (ByteDance), L. Wang, R. Chang (Tencent), Z. Xie (OPPO), E. Alshina (Huawei) |
| JVET-AF0042 | AhG11/AhG14 teleconference | E. Alshina, F. Galpin |
| Proposal |  |  |
| JVET-AF0143 | AHG11: Unified CNN super resolution for resampling-based video coding | C. Lin, Y. Li, J. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AF0150 | AhG11: on HOP batch size | F. Galpin, T. Dumas, P. Bordes (InterDigital) |
| JVET-AF0155 | AhG11: on HOP luma/chroma balance | F. Galpin, T. Dumas, P. Bordes (InterDigital) |
| JVET-AF0158 | AHG11: HOP In-loop filter with transformer blocks | Y. Li, D. Rusanovskyy, T. Ryder, S. Eadie, M. Karczewicz (Qualcomm) |
| JVET-AF0180 | AHG11: HOP training adjustment to improve luma/chroma coding gain balance | D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm) |
| JVET-AF0193 | AHG11: Decoder complexity optimization for NNVC in-loop filter | R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AF0206 | AHG11: Complexity reduction of NN-based loop-filters | J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang (Bytedance) |
| Cross Checks |  |  |
| JVET-AF0256 | Crosscheck of JVET-AF0193 (AHG11: Decoder complexity optimization for NNVC in-loop filter) | D. Liu (Ericsson) |

# Recommendations

The AHG recommends:

- Review all input contributions.

- Continue investigating neural network-based video coding tools, including coding performance and complexity.