| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AE0043-v2 |
| --- | --- |

| Title: | AHG11: Status of the joint EE1-0 (LOP.2) unified filter training |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Informational |  |  |
| Author(s) or Contact(s): | D. Rusanovskyy, Y.Li (Qualcomm) J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya(Ittiam) T. Shao, P. Yin (Dolby) | Email: | dmytror@qti.qualcomm.com yli30@qti.qualcomm.com jay.shingala@ittiam.com ajayshyam@ittiam.com ajat.suneja@ittiam.com siddarth.badya@ittiam.com Tong.Shao@dolby.com pyin@dolby.com |
| Source: | EE1-0 participants (Dolby, Ittiam, Qualcomm) |  |  |

_____________________________

# Abstract

A Unified Filter Architecture for the Low-performance Operation Point (LOP) was proposed in JVET-AE0281 to meet LOP complexity constraints (LOP.2), following HOP based training process. This contribution reports on the progress of LOP.2 joint training and presents full set of LOP2.0 results. Information presented in this document was partly presented during the AhG11/14 Telco on 08/09/2023.

# Introduction

In JVET-AE0281 a Unified Filter Architecture for the Low-performance Operation Point (LOP) was proposed and agreed for a joint training targeting LOP.2.

The filter architecture for LOP.2 was a joint design from EE1-4.1 (JVET-AE0067), EE1-4.4 (JVET-AE0165), and JVET-AE0238. The training process are aligned to the HOP filter training. The unified LOP model uses the simplified head block, simplified backbone and the split luma-chroma architecture which filter luma and chroma in separate branches using the simplified backbone and simplified tail blocks, as shown in Figure 1.

The following joint training procedure was defined for LOP.2 in JVET-AE2023.

For the stages I and II the following sub-tests are planned:

Subtest EE1-0.1: Test LOP filter architecture defined in JVET-AE0281.

Subtest EE1-0.2: Optimize the headblock design of JVET-AE0281 toward improving hardware friendliness (e.g. multiple of 16).

Subtest EE1-0.3: Based on of JVET-AE0281, different combinations of model parameters in luma and chroma branches toward better complexity-performance trade-off:

  - Number of hidden layers: NY, NC

  - Number of channels: CY, CUV, CY21, CUV21

  - SIMD friendly sizes: number of channels preferably multiple of 16

After Stage II training, a single model architecture to be selected for unified LOP test using HOP based filter usage and tested in NNVC-6.0 using SADL:

Sub-test EE1-0.4: Filter usage:

For the final trained LOP test mode, study filter usage aspects such as combine interfaces for usage of HOP and LOP filters, adaptive block sizes, on/off control at low block level.

Figure 1. LOP2.0 filter architecture of JVET-AE0281

The document below describes the progress of the LOP2.0 filter training.

# Reuse of HOP training process

## Overview

Figure 2 Training steps for HOP model creation

For training of the LOP.2, a modified HOP training, shown in Figure 2, was utilized. The following stages were implemented:

- Stage I:

  - Reuse HOP stage 1 dataset, md5: 1862a271133e3fabdf78e19d0d2c608f.

  - Train LOP.2 model(s) on the Stage 1 dataset: resulting model is lop2.x_st1.

- Stage II:

  - Encode BVI and TVD set data with nnvc6.0 and model of the stage 1, lop2.x_st1.

  - Produce the Stage 2 dataset from.

  - Train LOP.2 model (s) on the Stage 2 dataset: resulting model is lop2.x_st2.

- Stage III:

  - Encode BVI and TVD set data with nnvc6.0 and model of the stage 2, lop2.x_st2.

  - Produce the Stage 3 dataset from.

  - Train LOP.2 model on the Stage 3 dataset: resulting model is lop2.0_st3 (final model).

# Stage 1 training

Joined training of the Stage 1 was conducted by Ittiam/Dolby and Qualcomm independently with multiple LOP2.x filter architectures, following sub-tests of EE1-0.1, 0.2 and 0.3. The results of training, such as validation loss, CTC inference results and model have been exchanged and cross-checked.

The following architectures have been tested in the Stage 1:

- EE1-0.1: Default architecture

Default architecture have been implemented by Ittiam/Dolby and Qualcomm independently. Identified incomplete SADL support for luma/chroma split functionality was addressed differently, resulting in variance of the split implementation.

Ittiam/Dolby implemented split explicitly using slicing of the input tensor into luma and chroma tensors, as shown in Figure 3.a. ONNX to SADL conversion script had issues in ‘slice’ layer parsing due to incorrect assumptions. In addition to this, ‘slice’ layer implementation inside SADL library had newly introduced bugs since SADL_v5.1.  Both these issues have been fixed and merged into SADL dev_v7 branch (Commit id #6cce76e37cda77504d9d7fd978e0c0840b2db5ef).

Qualcomm implemented split through two separate convolutions, output of which produces luma and chroma associated out channels, as shown in Figure 3.b. No changes to the ONNX/SADL conversion or quantization is required.

Both designs result in near identical normative implementation, and are within the scope of EE1 testing.

- b.

Figure 3. Tested LOP2.0 filter split implementation

Additionally, the following training configurations have been tested:

Schedulers:

Scheduler 1:  "lr_scheduler" : { "milestones" : [110,  115, 120] } , "mse_epoch" : 120

Scheduler 2:  "lr_scheduler" : { "milestones" : [120,  123, 126] }, "mse_epoch" : 126

Weights on training losses (Y,U,V):

Weights 1: “component_loss_weightings” : [6,2]

Weights 2: “component_loss_weightings” : [12,2]

Summary of the inference CTC results of independently trained Stage 1 model for sub-test EE1-0.1, are shown in Table 1 and Table 2. Complete simulation test results are provided with this document.

Table 1: Inference results of default LOP2.0 architecture (source: Qualcomm (Scheduler 2 and Weights 1) and Ittiam/Dolby (Schedule 1 and Weights 1)

|  | lop2.0_st1_float (QC) |  |  | lop2.0_st1_float (Ittiam/Dolby) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -3.82% | -9.35% | -10.73% | -3.68% | -9.00% | -10.12% |
| Class A2 | -4.13% | -11.69% | -9.39% | -4.08% | -11.48% | -9.56% |
| Class B | -3.92% | -11.21% | -9.85% | -3.96% | -10.70% | -10.32% |
| Class C | -4.34% | -12.04% | -12.07% | -4.29% | -11.49% | -12.42% |
| Class E | -5.83% | -11.00% | -11.60% | -5.86% | -11.00% | -12.43% |
| Overall | -4.35% | -11.13% | -10.71% | -4.32% | -10.77% | -10.98% |
| Class D | -4.60% | -10.96% | -11.69% | -4.64% | -10.64% | -12.77% |

Table 2: Inference results of default LOP2.0 architecture and Scheduler 2 (source: Ittiam/Dolby, Qualcomm, Schedule 2).

|  | Lop2.0_st1_float, weights1 |  |  | lop2.0_st1_float, weights2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -3.82% | -9.35% | -10.73% | -3.99% | -9.83% | -11.36% |
| Class A2 | -4.13% | -11.69% | -9.39% | -4.11% | -11.88% | -9.67% |
| Class B | -3.92% | -11.21% | -9.85% | -4.01% | -11.44% | -10.61% |
| Class C | -4.34% | -12.04% | -12.07% | -4.38% | -12.32% | -13.08% |
| Class E | -5.83% | -11.00% | -11.60% | -5.98% | -11.67% | -13.49% |
| Overall | -4.35% | -11.13% | -10.71% | -4.43% | -11.48% | -11.61% |
| Class D | -4.60% | -10.96% | -11.69% | -4.70% | -11.12% | -13.39% |

Based on the observed inference results, st1_lop2.0_float model, resulted from the default architecture of LOP2.0, as defined in JVET-AE0281, trained with Scheduler 2 and Weights 2 (12:1:1) was selected for producing dataset for Stage 2. Since Stage 2 dataset is temporal, and performance of quantization on the model is not fully assessed at this stage, SADL model with float32 implemented was selected for Stage2 dataset production.

Resulting training configuration is provided below:

"stage1" : {
        "training" : {
            "mse_epoch" : 126,
            "max_epochs" : 130,
            "component_loss_weightings" : [12, 2],
            "lr_scheduler" : {"milestones" : [120, 123, 126]
            }
        }
    }
Training progress and simulation results for the LOP2.0 Stage 1 have been communicated on the AhG11/14 Telco, held on August 09th, 2023. Presentation was prepared by EE1-0 proponents and provided with this contribution.

Resulting Stage 1 model, training scripts, configurations files and model have been shared on the EE1 SW repository:
https://vcgit.hhi.fraunhofer.de/jvet-ae-ee11/VVCSoftware_VTM/-/tree/EE1-0

# Stage 2 training:

Stage 1 model selected to produce Stage 2 data set was shared through EE1-0 SW repository. Originally the model was produced and tested with SADL0003, latter converted to SADL004. Both versions of the model file have been tested and provided over the SW repository. NNVC6.0rc1 SW with SADL bug-fix was selected to be used for Stage 2 dataset production and as a codebase for EE1-0 SW.

## Stage 2 dataset:

Stage 2 dataset production followed the respective HOP guidelines. EE1-0 SW was used to dump training data with following encoder parameters:

--NnlfHopDebugOption=1 --NnlfOption=4 --NnlfHop=1 –NnlfHopModelName = “model_st1_lop2.0_float.sadl”

SADL model “model_st1_lop2.0_float.sadl” was made available on the EE1-0 branch:

./training/training_scripts/NN_Filtering_LOP/stage1/sadl_model/model_st1_lop2.0_float.sadl

Dataset was produced independently by participants (Dolby/Ittiam and Qualcomm) and resulting encoder log files have been shared between proponents for cross-check. Due to use of SADL float model, md5 sum could not be matched. Cross-check was accomplished by computing BD-rate difference between coding results of stage2 dataset produced by the proponents. The observed average BD-rate difference between datasets is shown in Table 3 below.

Table 3: BD-rate difference between encoding results for dataset produced independently by Dolby/Ittiam and Qualcomm.

| Stage 2 dataset, QC vs Dolby/Ittam | Average BD-Rate difference |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| BVI-DVC | -0.04% | 0.70% | 0.22% |
| BVI-DVC valid | 0.01% | 0.45% | 0.54% |
| TVD | -0.02% | 0.49% | 0.86% |

Average BD-rate difference results were found to be within expected range for floating point model and SADL implementation and thus successful cross-check of produced datasets was agreed.

## Training procedure:

LOP.2 Stage 2 training in general follows the HOP training guidelines. Limited changes to the HOP process were announced and agreed during AhG11/14 Telco (09 Aug.2023).

Agreed changes to the Stage 2 schedule are shown below and available from EE1-0 SW repository:

- ./training/training_scripts/NN_Filtering_LOP/stage2/stage2_lop2.0_training.json.

Training strategy 1:

"stage2" : {
        "training" : {
            "mse_epoch" : 58,
            "max_epochs" : 60,
            "component_loss_weightings" : [12, 2],
            "lr_scheduler" : {"milestones" : [55, 57, 58]
            }
        }
    }

Additional training strategy, of increasing the batch size and learning rate, was tested by proponents as an adaptation of the LOP filter architecture to the available computation resources. Resulting training configuration is shown below, with changes marked by yellow highlight and are available from EE1-0 SW repository:

- ./training/training_scripts/NN_Filtering_LOP/stage2/stage2_lop2.0_bs128_training.json.

Training strategy 2:

"stage2" : {
        "training" : {
            "mse_epoch" : 58,
            "max_epochs" : 60,
            "component_loss_weightings" : [12, 2],
            "dataloader": {"batch_size": 128, },
            "optimizer": {"lr": 0.0004, },
            "lr_scheduler" : {"milestones" : [55, 57, 58]
            }
        }
    }

Additional training for independent change of the batch size and learning rates are being conducted as well.

## Tested LOP.2 architecture:

To optimize luma/chroma performance balance of newly introduced split technology of JVET-AE0281, proponents tested different parameters of the filter architectures under the EE1-0.3 mandate, shown below:

Subtest EE1-0.3: Based on of JVET-AE0281, different combinations of model parameters in luma and chroma branches toward better complexity-performance trade-off:

  - Number of hidden layers: NY, NC

  - Number of channels: CY, CUV, CY21, CUV21

Based on the initial study in the Stage 1 training, presented during telco of 9th of August 2023, and the fast Stage 3 training (to be described below), two parameter sets of LOP.2 filter architecture were trained to be considered as candidates for Stage 3 training:

LOP2.0:  (17.6 kMAC/pixel)

LOP2.3:  (17.2 kMAC/pixel):  “N_Y”: 14, “N_UV”: 4, “C1_UV”: 32

## Stage 2 training result:

Stage 2 training was conducted independently, and participants have shared for cross-check following training results during the process:

- Validation loss function,

- Resulting model in SADL format,

- Results model in ckpt format.

Figure 4 show Stage 2 L1 training loss results produced by independent training for two strategies and two filter architectures described above.  Figure 5 show Stage 2 L2 training loss results produced by independent training for two strategies and two filter architectures described above.

At of Telco 08/30/2023, only Dolby/Ittiam training of LOP2.0 was completed, with final L2 losses at 60th epoch, equal to {2.27 e-4, 5.67 e-5} for Y and UV component, respectively.

However, analysis of the training losses suggests that Training Strategy 2 provides better training performance. A “short scheduler” was tested with LOP2.3_bs128 test, and L2 validation loss of {2.27 e-4, 5.74 e-05} for Y and UV components, was achieved at the epoch 22, respectively. Short scheduler:  “mse_epoch”: 21,  “max_epochs”: 23, “milestones”: [18,20,21].

Figure 4. Stage 2 L1 training loss produced for LOP2.0 and LOP2.3 filters and two training strategies, with “bs128“ notation marking the test with Training Strategy 2

Figure 5. Stage 2 L2 training loss for LOP2.0, LOP2.3 and two training strategies.

## Stage 2 inference test results:

Stage 2 LOP2.0 inference testing was conducted by Dolby/Ittiam and cross-checked by Qualcomm for model trained for 60 epochs with training Strategy 1. Comparative results of float model comparing to VTM anchor is shown in Table 4.

Table 4. Comparative results of LOP2.0 float vs VTM (Source: Dolby/Ittiam, cross-checked by Qualcomm)

|  | st2.lop2.0_fl vs vtm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -4.01% | -9.78% | -9.70% | -5.18% | -8.56% | -9.23% |
| Class A2 | -3.91% | -11.01% | -9.00% | -5.00% | -12.15% | -8.17% |
| Class B | -3.92% | -10.63% | -11.50% | -4.16% | -12.59% | -11.65% |
| Class C | -4.04% | -11.22% | -12.08% | -4.24% | -12.05% | -10.97% |
| Class E | -5.65% | -10.68% | -11.36% |  |  |  |
| Overall | -4.25% | -10.69% | -10.89% | -4.55% | -11.55% | -10.29% |
| Class D | -4.36% | -10.50% | -12.61% | -5.04% | -12.17% | -12.10% |

Preliminary inference testing was conducted by Qualcomm for LOP2.3_bs128 model trained for 22 epochs, with “short scheduler”: "mse_epoch": 21,  "max_epochs": 23, "milestones": [18,20,21]. Comparative results of float model comparing to VTM anchor is shown in Table 4.

Table 5. Comparative results of LOP2.3_bs128 float vs VTM (Source: Qualcomm)

|  | st2.lop2.3_bs128_fl_e22 vs vtm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -4.14% | -8.50% | -8.57% | -5.18% | -8.32% | -8.83% |
| Class A2 | -4.02% | -10.02% | -8.07% | -5.09% | -10.73% | -7.06% |
| Class B | -4.01% | -9.98% | -10.22% | -4.35% | -10.94% | -9.94% |
| Class C | -4.11% | -10.72% | -10.70% | -4.42% | -10.80% | -8.91% |
| Class E | -5.84% | -8.98% | -10.90% |  |  |  |
| Overall | -4.36% | -9.74% | -9.81% | -4.68% | -10.34% | -8.87% |
| Class D | -4.39% | -9.71% | -10.71% | -5.12% | -9.85% | -8.43% |

### Impact of quantization:

Tests on quantization of resulting Stage 2 models have been conducted. Inference test results for int16 models of LOP2.0 (quantized with Q11) and “short training” LOP2.3_bs128 (quantized with Q10) comparing to VTM anchor are shown in Table 6A and 7 respectively.

Performance of the LOP2.0 int16 model is further improved by using Q10 quantization for luma branch (except final luma layer) and Q11 quantization for chroma branch. The result is shown in Table 6B.

Similarly, performance of “short scheduler” LOP2.3_bs128 int16 model was improved by using Q10 quantization for layers featuring dynamic range overflow and Q11 for the rest of layers. The result is shown in Table 7B.

Table 6A. Comparative results of LOP2.0 int16 vs VTM (Source: Dolby/Ittiam, cross-checked by Qualcomm)

|  | st2.lop2.0_int16A vs vtm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -4.01% | -9.98% | -10.11% | -4.10% | -9.20% | -9.11% |
| Class A2 | -3.88% | -10.97% | -8.88% | -4.60% | -12.08% | -8.29% |
| Class B | -3.81% | -10.51% | -11.37% | -3.65% | -12.47% | -11.42% |
| Class C | -3.70% | -11.34% | -12.07% | -3.73% | -12.09% | -11.13% |
| Class E | -5.43% | -10.56% | -11.30% |  |  |  |
| Overall | -4.10% | -10.69% | -10.89% | -3.95% | -11.63% | -10.25% |
| Class D | -3.83% | -10.49% | -12.55% | -4.38% | -12.53% | -12.20% |

Table 6B. Comparative results of LOP2.0 int16B vs VTM (Source: Dolby/Ittiam)

|  | All Intra Main10 |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -3.91% | -9.80% | -10.05% | -4.72% | -9.24% | -9.82% |
| Class A2 | -3.74% | -10.86% | -8.79% | -4.69% | -12.21% | -8.53% |
| Class B | -3.77% | -10.43% | -11.22% | -3.94% | -12.40% | -11.62% |
| Class C | -3.92% | -11.33% | -12.10% | -4.04% | -12.06% | -11.07% |
| Class E | -5.42% | -10.58% | -11.21% |  |  |  |
| Overall | -4.10% | -10.62% | -10.81% | -4.27% | -11.64% | -10.50% |
| Class D | -4.23% | -10.59% | -12.65% | -4.74% | -12.45% | -12.19% |

Table 7A. Comparative results of LOP2.3_bs128 int16A vs VTM (Source: Qualcomm)

|  | st2.lop2.3_bs128_int16 vs vtm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -3.94% | -7.58% | -9.26% | -4.86% | -4.56% | -8.69% |
| Class A2 | -3.92% | -9.32% | -7.81% | -4.92% | -8.03% | -7.17% |
| Class B | -3.93% | -9.33% | -9.93% | -4.23% | -8.05% | -9.56% |
| Class C | -3.99% | -10.40% | -10.63% | -4.31% | -9.50% | -8.83% |
| Class E | -5.71% | -8.41% | -10.94% |  |  |  |
| Overall | -4.24% | -9.12% | -9.79% | -4.51% | -7.74% | -8.71% |
| Class D | -4.12% | -9.51% | -10.65% | -4.88% | -7.01% | -8.55% |

Table 7B. Comparative results of LOP2.3_bs128 int16B vs VTM (Source: Qualcomm)

|  | st2.lop2.3_bs128_int16 vs vtm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -4.19% | -8.83% | -9.36% | -5.10% | -8.10% | -7.42% |
| Class A2 | -4.04% | -9.94% | -7.93% | -5.06% | -10.51% | -6.06% |
| Class B | -4.01% | -9.89% | -9.92% | -4.30% | -10.55% | -8.14% |
| Class C | -4.01% | -10.70% | -10.64% | -4.34% | -10.55% | -8.14% |
| Class E | -5.80% | -8.97% | -10.81% |  |  |  |
| Overall | -4.34% | -9.75% | -9.80% | -4.62% | -10.05% | -7.58% |
| Class D | -4.14% | -9.77% | -10.55% | -4.91% | -9.98% | -7.86% |

## Conclusions on Stage 2:

Based on observed results, it is proposed to adopt filter LOP2.3 as unified LOP filter architecture for joint Stage 3 training.

LOP2.3:  (17.2 kMAC/pixel):  "N_Y": 14, "N_UV": 4, "C1_UV": 32

It is also proposed to adopt training strategy 2 (increased batch size to 128 and increase lr to 0.0004) for joint training of Stage 3.

# Stage 3 training:

Stage 2 model selected to produce Stage 3 data set was shared through EE1-0 SW repository.

## Stage 3 dataset:

Stage 3 dataset production followed the respective HOP guidelines for the stage 3 dataset production.

## Training procedure:

LOP.2 Stage 3 training in general follows the HOP training guidelines. Limited changes to the HOP process were announced and agreed during AhG11/14 Telco (30 Aug.2023).

Training strategy:

"stage3" : {
        "training" : {
            "mse_epoch" : 58,
            "max_epochs" : 60,
            "component_loss_weightings" : [12, 2],
            "dataloader": {"batch_size": 128, },
            "optimizer": {"lr": 0.0004, },
            "lr_scheduler" : {"milestones" : [55, 57, 58]
            }
        }
    }

## Tested LOP.2 architecture:

Based on the initial study in the Stage 1 and 2 training, presented during telco of 9th and 30th of August 2023, and the fast Stage 3 training, LOP2.3 filter architecture was selected for joined training in Stage 3:

LOP2.3:  (17.2 kMAC/pixel):  "N_Y": 14, "N_UV": 4, "C1_UV": 32

# “Fast” Stage 3 training:

To assess feasibility of the reusing HOP dataset for training of LOP2.x architecture, proponents conducted training of selected LOP2.x architectures with LOP2.x training strategies with HOP stage 3 dataset.

## Stage 3 dataset:

HOP Stage 3 dataset was used to train LOP2.x architectures.

## Training procedure:

LOP.2 Stage 3 training in general follows the HOP training guidelines, with changes agreed for LOP Stage 2 and described in section 4.2. The HOP stage 3 dataset was used for training.

Two training strategies have been tested:

Training strategy 1:

"stage2" : {
        "training" : {
            "mse_epoch" : 58,
            "max_epochs" : 60,
            "component_loss_weightings" : [12, 2],
            "lr_scheduler" : {"milestones" : [55, 57, 58]
            }
        }
    }

Training strategy 2:

"stage3" : {
        "training" : {
            "mse_epoch" : 58,
            "max_epochs" : 60,
            "component_loss_weightings" : [12, 2],
            "dataloader": {"batch_size": 128, },
            "optimizer": {"lr": 0.0004, },
            "lr_scheduler" : {"milestones" : [55, 57, 58]
            }
        }
    }

## Tested LOP.2 architecture:

The following LOP2.0 configuration have been tested:

LOP2.1 (17.3 kMAC/pixel): "N_Y": 13, "N_UV": 6, "C1_UV": 32,

LOP2.3: (17.2 kMAC/pixel):  "N_Y": 14, "N_UV": 4, "C1_UV": 32,

## Fast Stage 3 inference test results:

LOP2.1 was trained with HOP Stage 3 dataset. Training was conducted by Qualcomm for 60 epochs with training Strategy 2. Results of SADL float model compared to VTM anchor are shown in Table 8.

Table 8. Comparative results of LOP2.1_bs128 float vs VTM (Source: Qualcomm, cross-checked by Dolby/Ittiam)

|  | fastSt3.lop2.1_bs128_fl vs vtm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -4.26% | -9.93% | -10.69% | -5.63% | -9.68% | -10.86% |
| Class A2 | -4.14% | -11.24% | -9.27% | -5.24% | -12.98% | -9.02% |
| Class B | -4.12% | -10.97% | -11.88% | -4.64% | -13.11% | -12.63% |
| Class C | -4.35% | -11.76% | -12.66% | -4.88% | -13.98% | -13.18% |
| Class E | -5.94% | -10.61% | -12.30% |  |  |  |
| Overall | -4.50% | -10.96% | -11.49% | -5.02% | -12.63% | -11.70% |
| Class D | -4.57% | -11.15% | -12.89% | -5.74% | -14.27% | -14.49% |

Impact of the batch size and lr increase for given filter configuration can be assessed from Table 9, where LOP2.1 trained with training strategy 2 is compared against the LOP2.1 trained with training strategy 1.

Table 9. Comparative results of LOP2.1_bs128 float vs LOP2.1 float (Source: Qualcomm)

|  | fastSt3.lop2.1_bs128_fl vs fastSt3.lop2.1_fl |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.28% | -1.29% | -2.78% | -0.77% | -2.12% | -3.54% |
| Class A2 | -0.13% | -1.38% | -1.15% | -0.50% | -2.98% | -2.07% |
| Class B | -0.20% | -1.68% | -2.40% | -0.36% | -3.08% | -4.22% |
| Class C | -0.28% | -2.06% | -2.76% | -0.40% | -4.07% | -4.58% |
| Class E | -0.20% | -1.68% | -3.08% |  |  |  |
| Overall | -0.22% | -1.65% | -2.45% | -0.48% | -3.13% | -3.75% |
| Class D | -0.16% | -1.97% | -2.61% | -0.37% | -5.18% | -6.59% |

Table  10. Comparative results of LOP2.3vs LOP2.1 float as anchor

|  | fastSt3.lop2.3_fl vs fastSt3.lop2.1_fl |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.01% | 1.49% | -0.83% | -0.19% | 2.41% | -0.92% |
| Class A2 | -0.01% | 0.93% | -0.07% | -0.29% | 1.42% | 0.01% |
| Class B | -0.03% | 0.32% | -0.21% | -0.12% | 0.99% | -0.73% |
| Class C | -0.05% | 0.60% | -0.20% | -0.22% | 0.30% | -1.18% |
| Class E | 0.05% | 1.47% | -1.36% |  |  |  |
| Overall | -0.01% | 0.87% | -0.48% | -0.19% | 1.18% | -0.74% |
| Class D | -0.02% | 2.13% | 0.36% | -0.08% | 4.09% | -1.75% |

## Conclusions on Fast Stage 3:

Based on the observed results, it is assessed that in filter configuration LOP2.1, increase of the batch size to 128 and lr to 0.0004 provides {0.5%, 3.1%, 3.8%} of BD-rate gain for Y,Cb, Cr components compared to batch size 64 and lr 0.0001.

It is also observed, that LOP2.3 provides acceptable luma/chroma gain rebalancing and can be recommended for Stage 3 testing.

# Communication:

Training progress and simulation results for the LOP2.0 Stage 1 have been communicated on the AhG11/14 Telcos, held on August 09th and August 30th, 2023. Presentation was prepared by EE1-0 proponents and provided with this contribution.

# Software availability and complexity:

SW branch for joint LOP2.0 training

https://vcgit.hhi.fraunhofer.de/jvet-ae-ee11/VVCSoftware_VTM/-/tree/EE1-0

Training scripts and configuration files can be found in folder:

training/training_scripts/NN_Filtering_LOP

Computational resources and storage requirements are identical to those of HOP training. DIV2K dataset of HOP is used for LOP training in stage 1.