| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0049-v1 |
| --- | --- |

| Title: | AHG9: On the SEI processing order SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes the following changes related to the SEI processing order SEI message:

- Removal of the capability of wrapping SEI messages within the SEI processing order SEI message and addition of a processing order nesting SEI message that includes one or more SEI messages having the same persistence and are used in an indicated position within the processing order.

- Removal of the following constraints, which are asserted to be imposed elsewhere: "It is a requirement of bitstream conformance that, within an SEI processing order SEI message, there shall be at least two pairs of the syntax elements po_sei_payload_type[ i ] and po_sei_processing_order[ i ], and there shall be at least two values of po_sei_processing_order[ i ] that are not equal."

- Indication of the increment po_sei_processing_order_incr_flag[ i ] relative to the previous derived processing order value, instead of an absolute processing order value po_sei_processing_order[ i ].

- Indication of the number of bits instead of bytes for SEI message prefixes included in the SEI processing order SEI message.

# Processing order nesting

## Introduction

Asserted downsides or issues related to the SEI message wrapping design for the SEI processing order SEI message include:

- The persistence of the wrapped SEI messages may differ from each other and from the persistence of the SEI processing order SEI message. The SEI processing order SEI message has been intended to persist for the entire CLVS, whereas the wrapped SEI messages may have shorter persistence.

- The size of the SEI processing order SEI message may become relatively large, which may hinder its usage in session setup or negotiation.

Rather than wrapping SEI messages within the SEI processing order SEI message, it is proposed to define a processing order nesting SEI message that includes one or more SEI messages having the same persistence and are used in an indicated position within the processing order.

## Proposed syntax of the processing order nesting SEI message

| processing_order_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| pon_num_seis_minus1 | u(8) |
| for( i = 0; i <= pon_num_seis_minus1; i++ ) { |  |
| pon_processing_order[ i ] | u(8) |
| sei_message( ) |  |
| } |  |
| } |  |

## Proposed semantics the processing order nesting SEI message

The processing order nesting SEI message includes one or more SEI messages that should be applied only as parts of the processing chain implied by the SEI processing order SEI message and should not be applied in a manner that would contradict with the processing chain implied by the SEI processing order SEI message.

The persistence of all the SEI messages included in the same processing order nesting SEI message shall be the same.

pon_num_seis_minus1 specifies the number of the processing-order-nested SEI messages that are included in this SEI message. The value of pon_num_seis_minus1 shall be in the range of 0 to 255, inclusive.

pon_processing_order[ i ] specifies the position of the i-th processing-order-nested SEI message within the processing order. When i is greater than 0, pon_processing_order[ i ] shall be greater than or equal to pon_processing_order[ i − 1 ]. It is a requirement of bitstream conformance that the SEI processing order SEI message for this CLVS shall have an entry idx for which all of the following are true:

–	po_sei_processing_order[ idx ] is equal to pon_processing_order[ i ]

–	po_sei_payload_type[ idx ] is equal to the payloadType value of the i-th processing-order-nested SEI message.

–	When po_sei_prefix_flag[ idx ] is equal to 1, po_prefix_byte[ idx ][ j ] for j in the range of 0 to po_num_prefix_bytes[ idx ] − 1, inclusive, contain the same content as the po_num_prefix_bytes[ idx ] initial bytes of the SEI message payload of the i-th processing-order-nested SEI message.

## Proposed changes in the syntax of the SEI processing order SEI message

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| if( po_sei_wrapping_flag[ i ] ) { |  |
| reserved_alignment_6bits | u(6) |
| sei_message( ) |  |
| } else { |  |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| if( po_sei_prefix_flag[ i ]) { |  |
| po_num_prefix_bytes[ i ] | b(8) |
| for( j = 0; j < po_num_prefix_bytes[ i ]; j++ ) |  |
| po_prefix_byte[ i ][ j ] | b(8) |
| } |  |
| } |  |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| } |  |

## Proposed changes in the SEI processing order SEI message semantics

po_sei_wrapping_flag[ i ] equal to 1 specifies that one or more processing order nesting SEI messages with both of the following constraints should be present:

- The payloadType of the nested SEI message is equal to po_sei_payload_type[ i ].

- pon_processing_order[ j ] with any value of j is equal to po_sei_processing_order[ i ].

If po_sei_wrapping_flag[ i ] is equal to 0, an SEI message with payloadType equal to po_sei_payload_type[ i ] should be present outside of the SEI processing order processing order nesting SEI message with payloadType equal to po_sei_payload_type[ i ]. However, if po_sei_wrapping_flag[ i ] is equal to 0 and no SEI message is present with payloadType equal to po_sei_payload_type[ i ], the following applies:

- If po_sei_importance_flag[ i ] is equal to 1, the decoder should ignore the entire SEI processing order SEI message.

- Otherwise, the decoder should ignore all data associated with the loop variable value of i.

NOTE – po_sei_wrapping_flag[ i ] equal to 1 enables SEI messages to be carried within the SEI processing order processing order nesting SEI message to prevent such SEI messages from being incorrectly interpreted by decoders that do not process the SEI processing order SEI message. Thus, po_sei_wrapping_flag[ i ] equal to 1 is intended to be used when po_sei_wrapping_flag[ i ] equal to 0 can lead to unintended results being produced by such decoders.

# Removal of constraints imposed elsewhere

## Introduction

It is asserted that the constraints in the following paragraph are redundant:

It is a requirement of bitstream conformance that, within an SEI processing order SEI message, there shall be at least two pairs of the syntax elements po_sei_payload_type[ i ] and po_sei_processing_order[ i ], and there shall be at least two values of po_sei_processing_order[ i ] that are not equal.

There are at least two loop entries, which is imposed by the syntax:

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++) { |  |

There are at least two values of po_sei_processing_order[ i ] that are not equal, which is imposed by the following constraints in bold:

po_sei_processing_order[ 0 ] shall be equal to 0, and for i greater than 0, po_sei_processing_order[ i ] shall be equal to po_sei_processing_order[ i − 1 ] or po_sei_processing_order[ i − 1 ] + 1.

The value of po_sei_processing_order[ po_num_sei_messages_minus2 + 1 ] shall not be equal to 0.

## Proposed changes

It is a requirement of bitstream conformance that, within an SEI processing order SEI message, there shall be at least two pairs of the syntax elements po_sei_payload_type[ i ] and po_sei_processing_order[ i ], and there shall be at least two values of po_sei_processing_order[ i ] that are not equal.

# Delta signaling of processing order value

## Introduction

JVET-AE2027 constrains the value of po_sei_processing_order[ i ] as follows:

po_sei_processing_order[ 0 ] shall be equal to 0, and for i greater than 0, po_sei_processing_order[ i ] shall be equal to po_sei_processing_order[ i − 1 ] or po_sei_processing_order[ i − 1 ] + 1.

Since the delta of the i-th processing order value compared to the (i − 1)-th processing order value can only be 0 or 1, it is asserted to be unnecessary to use 8 bits for the syntax element.

## Proposed changes

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++) { |  |
| if( i > 0 ) |  |
| po_sei_proceesing_order_incr_flag[ i ] | u(1) |
| else |  |
| po_reserved_zero_bit | u(1) |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| if( po_sei_wrapping_flag[ i ] ) { |  |
| reserved_alignment_56bits | u(56) |
| sei_message( ) |  |
| } else { |  |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(1213) |
| if( po_sei_prefix_flag[ i ]) { |  |
| po_num_prefix_bytes[ i ] | b(8) |
| for( j = 0; j < po_num_prefix_bytes[ i ]; j++ ) |  |
| po_prefix_byte[ i ][ j ] | b(8) |
| } |  |
| } |  |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| } |  |

po_sei_processing_order_incr_flag[ i ] is used to derive the value of PoSeiProcessingOrder[ i ], which po_sei_processing_order[ i ] indicates the preferred order of processing of the i-th SEI message type for which preferred processing order information is provided in the SEI processing order SEI message. PoSeiProcessingOrder[ 0 ] is set equal to 0. When i is greater than 0, PoSeiProcessingOrder[ i ] is set equal to PoSeiProcessingOrder[ i − 1 ] + po_sei_processing_order_incr_flag[ i ].

For any two different integer values of m and n that are greater than or equal to 0, po_sei_processing_orderPoSeiProcessingOrder[ m ] less than po_sei_processing_orderPoSeiProcessingOrder[ n ] indicates any SEI message type with payloadType equal to po_sei_payload_type[ m ] and, when present, bytes po_prefix_byte[ m ][ p ] for p ranging from 0 to po_num_prefix_bytes[ m ] − 1, inclusive, should be processed before any SEI message type with payloadType equal to po_sei_payload_type[ n ], and, when present, bytes po_prefix_byte[ n ][ q ] for q ranging from 0 to po_num_prefix_bytes[ n ] − 1, inclusive, and po_sei_processing_orderPoSeiProcessingOrder[ m ] equal to po_sei_processing_orderPoSeiProcessingOrder[ n ] indicates that there is no preferred order of processing between the SEI message types. When there are multiple SEI messages with the same values of po_sei_payload_type[ i ], po_num_prefix_bytes[ i ], and bytes po_prefix_byte[ i ][ j ] for j ranging from 0 to po_num_prefix_bytes[ i ] − 1, inclusive, they shall have the same value of po_sei_processing_orderPoSeiProcessingOrder[ i ].

po_sei_processing_order[ 0 ] shall be equal to 0, and for i greater than 0, po_sei_processing_order[ i ] shall be equal to po_sei_processing_order[ i − 1 ] or po_sei_processing_order[ i − 1 ] + 1.

The value of po_sei_processing_orderPoSeiProcessingOrder[ po_num_sei_messages_minus2 + 1 ] shall not be equal to 0.

# Prefix bits rather than prefix bytes

## Introduction

The following open question is documented as an editor's note:

[Ed per JVET-AD0386: Open issues -- Should the SEI prefix be required to consist of a number of complete syntax elements only? If yes, should the SEI prefix length be indicated in bytes or bits?]

When authoring the SEI processing order SEI message for a CVS, the encoder might not be able to determine the values of all syntax elements of those SEI messages for which a prefix is provided within the SEI processing order SEI message. Such an unconcluded syntax element may entirely fit into the last byte of the prefix. A decoder cannot know if an unconcluded syntax element is part of the given prefix or is just padding to fill in the last byte of the prefix. It is therefore proposed to indicate the number of prefix bits in the last prefix byte.

## Proposed changes

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| if( po_sei_wrapping_flag[ i ] ) { |  |
| reserved_alignment_6bits | u(6) |
| sei_message( ) |  |
| } else { |  |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| if( po_sei_prefix_flag[ i ]) { |  |
| po_num_prefix_bytes[ i ] | bu(8) |
| po_num_unused_prefix_bits[ i ] | u(3) |
| reserved_alignment_5bits | u(5) |
| for( j = 0; j < po_num_prefix_bytes[ i ]; j++ ) |  |
| po_prefix_byte[ i ][ j ] | b(8) |
| } |  |
| } |  |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| } |  |

po_num_prefix_bytes[ i ], when present, specifies the number of bytes associated with the i-th SEI message for which preferred processing order information is provided in the SEI processing order SEI message. When not present, the value of po_num_prefix_bytes[ i ] is inferred to be equal to 0.

po_num_unused_prefix_bits[ i ], when present, specifies the number of bits in po_prefix_byte[ i ][ po_num_prefix_bytes[ i ] − 1 ] that are not part of the prefix of the i-th SEI message for which preferred processing order information is provided in the SEI processing order SEI message. When not present, po_num_unused_prefix_bits[ i ] is inferred to be equal to 0.

reserved_alignment_5bits has no meaning and shall be equal to 0 in bitstreams conforming to this version of this Specification. Decoders shall allow this syntax element to have other values and shall ignore the value.

po_prefix_byte[ i ][ j ], when present, specifies the j-th byte value of the i-th SEI message.

When po_sei_prefix_flag[ i ] is equal to 1, PoPrefixBit[ i ][ k ], which specifies the k-th bit of the prefix of the i-th SEI message for which preferred processing order information is provided in the SEI processing order SEI message, is derived as follows:

for( j = 0, k = 0; j < po_num_prefix_bytes[ i ]; j++ ) {
	numUnusedBits = (j < po_num_prefix_bytes[ i ] − 1) ? 0 : po_num_unused_prefix_bits[ i ]
	for( bitIdx = 7; bitIdx >= numUnusedBits; bitIdx− −, k++ )	(xx)
		PoPrefixBit[ i ][ k ] = ( po_prefix_byte[ i ][ j ] & (1 << bitIdx ) ) ? 1 : 0
}

PoPrefixNumBits[ i ], which specifies the count of bits in the prefix of the i-th SEI message for which preferred processing order information is provided in the SEI processing order SEI message, is derived as follows:

if( po_sei_prefix_flag[ i ] )
	PoPrefixNumBits[ i ] = po_num_prefix_bytes[ i ] * 8 − po_num_unused_prefix_bits[ i ]	(xx)
else
	PoPrefixNumBits[ i ] = 0

po_sei_processing_order[ i ] indicates the preferred order of processing of the i-th SEI message type for which preferred processing order information is provided in the SEI processing order SEI message. For any two different integer values of m and n that are greater than or equal to 0, po_sei_processing_order[ m ] less than po_sei_processing_order[ n ] indicates any SEI message type with payloadType equal to po_sei_payload_type[ m ] and, when present po_sei_prefix_flag[ i ] is equal to 1, bytes po_prefix_byte[ m ][ p ] for p ranging from 0 to po_num_prefix_bytes[ m ] − 1, inclusive, bits PoPrefixBit[ m ][ p ] for p ranging from 0 to PoPrefixNumBits[ m ] − 1, inclusive, should be processed before any SEI message type with payloadType equal to po_sei_payload_type[ n ], and, when present po_sei_prefix_flag[ i ] is equal to 1, bytes po_prefix_byte[ n ][ q ] for q ranging from 0 to po_num_prefix_bytes[ n ] − 1, inclusive, bits PoPrefixBit[ n ][ q ] for q ranging from 0 to PoPrefixNumBits[ n ] − 1, inclusive, and po_sei_processing_order[ m ] equal to po_sei_processing_order[ n ] indicates that there is no preferred order of processing between the SEI message types. When there are multiple SEI messages with the same values of po_sei_payload_type[ i ], po_num_prefix_bytes [ i ], and bytes po_prefix_ byte[ i ][ j ] for j ranging from 0 to po_num_prefix_bytes [ i ] − 1, inclusive, po_sei_prefix_flag[ i ], and, when po_sei_prefix_flag[ i ] is equal to 1, both PoPrefixNumBits[ i ] and bits PoPrefixBit[ i ][ j ] ranging from 0 to PoPrefixNumBits[ i ] − 1, inclusive, they shall have the same value of po_sei_processing_order[ i ].

po_sei_processing_order[ 0 ] shall be equal to 0, and for i greater than 0, po_sei_processing_order[ i ] shall be equal to po_sei_processing_order[ i − 1 ] or po_sei_processing_order[ i − 1 ] + 1.

The value of po_sei_processing_order[ po_num_sei_messages_minus2 + 1 ] shall not be equal to 0.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).