| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 |  |  |  | Document: JVET-AF0053-v2 |
| --- | --- | --- | --- | --- |
| Title: | EE2-related: Test 2.5a and TIMD fusion improvement |  |  |  |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Pierre Andrivon, Médéric Blestel, Karam Naser, Fabrice Le Léannec, Philippe Bordes, Franck Galpin, Antoine Robert | Tel: Email: | pandrivon@ofinno.com karam.naser@interdigital.com |  |
| Source: | Ofinno LLC, InterDigital |  |  |  |

_____________________________

# Abstract

Contribution JVET-AE0184, studied in EE2-2.5a (JVET-AF0136), proposes to extend TIMD candidates list with IntraTMP and IBC intra prediction modes of neighboring blocks. Contribution JVET-AE0137 introduces a non-angular intra predictor (among DC or Planar) in the TIMD fusion process, additionally to the two selected TIMD modes. Besides, the location-dependent sample-based fusion process from DIMD is adjusted for TIMD. This contribution merges both EE2-2.5a and JVET-AE0137 and proposes to add the IntraTMP or IBC mode of neighboring blocks with the lowest template matching cost to the non-angular intra predictor candidates list. On top of ECM-10.0, the reported PSNR-Y, Cb, Cr, BD-rate and {EncT, DecT} results are as follows:

- AI: -0.08%, -0.05%, -0.04%  {100.3%, 99.7%}

- RA: -0.03%%, 0.00%, -0.10%  {98.8%, 100.1%}

# Introduction

## TIMD

The TIMD process [1] is based on a template cost analysis in which several intra modes -- from a candidates list comprising MPMs, wide-angular, DC, horizontal, vertical, and extended modes -- are tested. The two intra modes (IPM1 and IPM2) providing the smallest SATD costs are selected and blended in a fusion process according to their respective template cost under the condition that SATDIPM2<2.SATD(IPM1). In this contribution, the intra modes selection and the fusion process are modified.

## TIMD with IntraTMP and IBC candidates

JVET-AE0184 [2], studied in EE2-2.5a (JVET-AF0136 [4]), proposes to add available IntraTMP and IBC modes of the neighboring blocks to the TIMD candidate list to predict the current block template. Up to 50 modes can be appended to the list comprising MPM, DC mode, horizontal and vertical modes. Each prediction from IBC and IntraTMP is included in the template matching cost comparison to deduce the best two TIMD modes. If selected by the TIMD process, IntraTMP/IBC prediction is performed for the current block and the TIMD blending process is used.

## TIMD fusion process with a non-angular intra mode

JVET-AE0137 [3] introduces a third non-angular intra prediction mode IPMNA in the TIMD fusion process.

First, the non-angular intra mode IPMNA (among DC and Planar) with the lowest SATD cost is added to the TIMD fusion process if it is different from the two selected TIMD intra modes (IPM1 and IPM2) and SATDIPMNA<k.SATDIPM1 with k=1.5.

The weights wi used during the TIMD fusion process are computed from SATD cost:

wi=sumSATD-SATD(IPMi)2×sumSATD

with

sumSATD=j=13SATD(IPMj)

The logic behind the non-angular prediction activation condition is based on already computed SATD costs.

Additionally, location-dependent sample-based blending of the DIMD fusion process [1] is re-used but the location-dependent criterion applying to amplitudes of the selected predictors is replaced by a SATD cost-based criterion. The location-dependent criterion is determined from a ratio of the normalized SATD of the selected TIMD predictors computed in ABOVE and LEFT template area, as follows:

If norSATDA<18 .norSATDL, locDepi=1 (vertical)

Else if norSATDL<18 .norSATDA, locDepi=2 (horizontal)

Otherwise, locDepi=0 (diagonal)

where:

- locDepi is the location-dependent parameter value associated with the i-th selected TIMD mode (e.g. i belongs to [0;2]). A value of 0 indicates no location-dependency (diagonal sample-based blending), a value of 1 indicates vertical location-dependency of the i-th selected TIMD mode, a value of 2 indicates horizontal location-dependency of the i-th selected TIMD mode,

- norSATDA is the normalized SATD cost associated to a selected TIMD candidate and computed in ABOVE template:

norSATDA=1hT×wCU

- norSATDL is the normalized SATD cost associated to a selected TIMD candidate and computed in LEFT template:

norSATDL=1wT×hCU

with:

- hT, the height of the (ABOVE) template area,

- wT, the width of the (LEFT) template area,

- hCU, the height of the current CU,

- wCU, the width of the current CU.

Furthermore, CU size adaptations are undertaken. The maximum weight deviation of the sample-based blending (range ∆i) becomes CU-size dependant where bigger CUs (greater than 128 samples) have less deviations. Additionally, location-dependency is forced to 0 for small CU size (width or height less than 8 samples).

Eventually, if one of the two templates is not available, the location-dependency state is set accordingly to the present template. In case of LEFT template presence only, the location-dependency is set to 2 (horizontal), otherwise in case of ABOVE template presence only, the location-dependency is set to 1 (vertical).

# Proposal

It is proposed to merge JVET-AE0184 [1] with JVET-AE0137 [2]. Additionally, the best IntraTMP/IBC mode (i.e., with minimum SATD cost), is added to the non-angular candidate list used to determine the third intra prediction mode to be blended in the TIMD fusion process. As described in [2], the IPMNA candidate in the non-angular candidate list with the minimum SATD cost is selected and can be blended with the two selected TIMD modes.

# Experimental results

The proposed method is implemented on top of the ECM-10.0. Simulations are performed following the common test conditions of EE2 [5]. The simulation results are shown below.

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0,03% | -0,01% | 0,04% | 100,4% | 100,7% | 99,8% |
| Class A2 | -0,09% | -0,13% | -0,04% | 100,0% | 100,2% | 100,2% |
| Class B | -0,09% | -0,01% | -0,11% | 100,4% | 100,0% | 100,1% |
| Class C | -0,08% | -0,02% | -0,02% | 100,7% | 97,8% | 100,1% |
| Class E | -0,10% | -0,12% | -0,01% | 99,9% | 100,4% | 100,1% |
| Overall | -0,08% | -0,05% | -0,04% | 100,3% | 99,7% | 100,1% |
| Class D | -0,05% | 0,15% | -0,08% | 100,5% | 97,7% | 100,1% |
| Class F | -0,09% | 0,01% | 0,10% | 100,5% | 99,8% | 100,2% |
| Class TGM | -0,03% | 0,00% | 0,00% | 100,1% | 97,8% | 100,2% |

Table 1. Performance of the proposed method on top of ECM-10.0 (AI)

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.00% | -0.11% | -0.04% | 98.3% | 99.0% | 99.9% |
| Class A2 | -0.06% | -0.13% | -0.02% | 99.3% | 99.4% | 99.8% |
| Class B | -0.05% | 0.13% | -0.17% | 98.6% | 100.7% | 100.1% |
| Class C | 0.00% | 0.01% | -0.09% | 98.9% | 100.9% | 100.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.03% | 0.00% | -0.10% | 98.8% | 100.1% | 100.0% |
| Class D | 0.00% | -0.09% | -0.15% | 99.6% | 102.0% | 100.1% |
| Class F | -0.05% | 0.12% | 0.27% | 100.3% | 100.7% | 99.9% |
| Class TGM | -0.14% | -0.26% | -0.25% | 99.9% | 100.3% | 100.0% |

Table 2. Performance of the proposed method on top of ECM-10.0 (RA)

It is noted that numbers reported in italics denote missing QPs of the test replaced by reference QPs.

For convenience, performance of test EE2-2.5a ([4][5]) on top of ECM-10.0 is reported as follows:

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0,02% | 0,11% | 0,06% | 99,8% | 98,2% | 99,1% |
| Class A2 | -0,03% | -0,04% | -0,05% | 99,1% | 98,0% | 99,5% |
| Class B | -0,04% | 0,00% | -0,09% | 100,7% | 100,4% | 99,9% |
| Class C | -0,04% | 0,06% | 0,03% | 99,8% | 100,5% | 100,0% |
| Class E | -0,08% | 0,01% | -0,04% | 101,5% | 102,0% | 100,0% |
| Overall | -0,04% | 0,03% | -0,02% | 100,2% | 99,9% | 99,7% |
| Class D | -0,02% | 0,07% | -0,08% | 100,5% | 100,7% | 100,0% |
| Class F | -0,09% | -0,05% | -0,01% | 100,6% | 103,9% | 99,9% |
| Class TGM | -0,06% | 0,00% | -0,02% | 100,0% | 103,4% | 99,7% |

Table 3. Performance of EE2-2.5a on top of ECM-10.0 (AI)

# Conclusion

Results from this contribution demonstrate that the proposed method brings BD-Rate coding gains compared to ECM-10.0 and EE2-2.5a. Furthermore, it is observed that the encoding and decoding times are almost the same as the anchor’s. It is recommended to study this method in next round of ECM EE2.

# 5 References

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 10 (ECM 10),” JVET-AE2025, July 2023.

- K. Naser, F. Le Léannec, P. Bordes, F. Galpin, A. Robert, “AHG12 : TIMD with IntraTMP and IBC candidates”, JVET-AE0184, July 2023.

- P. Andrivon, M. Blestel, “Non-EE2 : On TIMD Fusion”, JVET-AE0137, July 2023.

- K. Naser, “EE2-2.5a: TIMD with IntraTMP and IBC”, JVET-AF0136, October 2023.

- P. Andrivon, “Cross-check of JVET-AF0136 (EE2-2.5a: TIMD with IntraTMP and IBC)”, JVET-AF0125, October 2023.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AE2017, July 2023.

# 6 Patent rights declaration(s)

Ofinno LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).