| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0058-v1 |
| --- | --- |

| Title: | AHG12: GOP-based RPR encoder control for ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, J. Ström, R. Yu, P. Wennersten, W. Ahmad | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

GOP-based RPR encoder control has been available for VTM since VTM-19.0. It selects coding in a reduced resolution when impact of re-scaling performance between source resolution and reduced resolution is expected to be limited. ECM does not have such functionality. This contribution proposes to add the GOP-based RPR encoder control from VTM to ECM to enable further experimentation of RPR functionality in ECM.

Objective performance (BDR) for RA 4K resolutions and QP 40, 43, 47 and 50 is reported as:

ECM-10.0 with GOP-based RPR versus ECM-10.0: Y: -3,25%, U  5,95%, V  5,15%

VTM-22.0 with GOP-based RPR versus VTM-22.0: Y: -8,92%, U -3,38%, V -5,87%

Objective performance (BDR) for RA all resolutions and QP 40, 43, 47 and 50 is reported as:

ECM-10.0 with GOP-based RPR versus ECM-10.0: Y: -1,48%, U  2,37%, V  1,54%

VTM-22.0 with GOP-based RPR versus VTM-22.0: Y: -4,08%, U -2,82%, V -3,92%

It is noted that the benefit of using the GOP-based RPR encoder control in ECM is less compared to its usage in VTM but that it still shows promising coding gains at challenging bitrates.

It is also asserted that subjective quality can be improved by the proposal.

# Introduction

In VTM [1] it has been shown beneficial to encode in lower resolution based on a GOP-based encoder control [2] especially for higher resolutions and challenging bitrates. The purpose with the contribution is to add such functionality also to ECM [3].

# Proposal

This contribution proposes the GOP-based RPR encoder control method from VTM to also be included in ECM. The method decides when it is likely better to encode pictures in reduced resolution with RPR and when it is likely better to encode them in the source resolution. The selection is based on QP and picture self-similarity after re-scaling. The self-similarity test is only conducted on the first source picture in display order within each GOP. If the re-scaled picture is determined to have sufficient similarity with the source picture, all pictures in the GOP are encoded at reduced resolution, otherwise they are encoded in the source resolution.

# Results

Encoding and decoding times are not accurate but there is a natural reduction in encoding and decoding time when encoding in reduced resolution.

In VTM the GOP-based RPR encoder control can be enabled by setting the config parameter “GOPBasedRPR” to 1. Same enabling and configuration parameters are also part of the proposal for ECM.

ECM-10.0 with GOP-based RPR versus ECM-10.0 [3]:

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 QP=40, 43, 47, 50 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -5,12% | -4,36% | -4,49% | 50,6% | 51,8% | 67,9% |
| Class A2 | -1,38% | 16,26% | 14,79% | 50,0% | 54,9% | 70,0% |
| Class B | -0,51% | 0,16% | -1,29% | 78,7% | 83,5% | 100,3% |
| Class C | -0,04% | -0,24% | -0,32% | 96,4% | 95,6% | 102,7% |
| Class E |  |  |  |  |  |  |
| Overall | -1,48% | 2,37% | 1,54% | 69,5% | 72,3% | 86,9% |

For comparison, VTM-22.0 with GOP-based RPR versus VTM-22.0 [1]:

|  | Over VTM-22.0 QP=40, 43, 47, 50 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -11,00% | -13,85% | -13,64% | 79% | 126% |
| Class A2 | -6,84% | 7,09% | 1,90% | 74% | 130% |
| Class B | -2,00% | -5,04% | -5,36% | 100% | 121% |
| Class C | 0,57% | 0,80% | 0,82% | 104% | 94% |
| Class E |  |  |  |  |  |
| Overall | -4,08% | -2,82% | -3,92% | 90% | 116% |

# Conclusion

It has been shown that the GOP-based RPR encoder control can provide objective benefits compared to ECM-10.0 especially for encoding 4K sequences at higher QPs. It is also asserted that subjective quality can be improved. It is suggested to include this functionality to ECM.

# References

[1] VTM-22.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-22.0

[2] K. Andersson, J. Ström, R. Yu, P. Wennersten, W. Ahmad,” AHG10: GOP-based RPR encoder control”, JVET-AB0080, Mainz, October 2022.

[3] ECM-10.0, available at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-10.0

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).