| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0062-v1 |
| --- | --- |

| Title: | AHG9: Some syntax and semantics changes for the SEI processing order SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 8910 University Center Lane San Diego, CA 92122, USA | Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following syntax and semantics changes for the SEI processing order SEI message:

- Move po_sei_prefix_flag[ i ] from immediately before po_sei_payload_type[ i ] to be immediately after po_sei_payload_type[ i ], because the SEI payload type is a higher-level information and this move can also avoid potential semantics dependency on a syntax element later in decoding order.

- To avoid potential syntax parsing problems, the SEI prefix indications, when present, are signalled in units of bits, and an SEI prefix indication for a particular SEI payloadType shall be a bit string that follows the SEI payload syntax of that value of payloadType and contains a number of complete syntax elements starting from the first syntax element in the SEI payload, same as in the SEI prefix indication SEI message.

- The signalling of the preferred order of processing for the 0-th SEI message type is skipped, and the value is derived to be equal to 0.

- The preferred order of processing for the i-th SEI message type for i greater than 0 is signalled using a flag instead of 8 bits, the flag indicating whether the preferred order of processing for the i-th SEI message type is equal to that of the ( i – 1 )-th SEI message type plus 1 or is the same as that of the ( i – 1 )-th SEI message type.

- Add a NOTE to clarify the following aspects: In the semantics of the SPO SEI message, two different types of SEI messages may have the same SEI payloadType value but are differentiated by some syntax elements in the SEI payload. For example, two NNPFC SEI messages with different nnpfc_id values are considered as having two different SEI message types. This change is asserted to be purely editorial.

- Remove the first constraint below as it is redundant to the second constraint below (it would also be OK to keep the first constraint and remove the second one, but the first one is more complicated and has some buggy issues that are asserted to need some effort to fix). This change is asserted to be purely editorial.

The first constraint: It is a requirement of bitstream conformance that, within an SEI processing order SEI message, there shall be at least two pairs of the syntax elements po_sei_payload_type[ i ] and po_sei_processing_order[ i ], and there shall be at least two values of po_sei_processing_order[ I ] that are not equal.

The second constraint: The value of po_sei_processing_order[ po_num_sei_messages_minus2 + 1 ] shall not be equal to 0.

- It is constrained that, an SEI prefix indication for a colour transform information (CTI) SEI message shall include at least all bits of the colour_transform_id syntax element of the CTI SEI message.

- It is constrained that, an SEI prefix indication for a neural-network post-filter characteristics (NNPFC) SEI message shall include at least all bits of the nnpfc_id syntax element of the NNPFC SEI message.

The proposed text changes, relative to JVET-AE2027, are included in an attachment of this contribution. There are also some changes included in the attachment that are not covered in the summary above, and those changes asserted to be purely editorial and very minor. An example of such changes is replacing of “if” without a corresponding “otherwise” with “when”.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).