| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0063-v2 |
| --- | --- |

| Title: | On MV-HEVC profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Hongbin Liu2, Li Zhang1, Shaohui Jiao2, Chao Hu2, Jianyong Cui2, Guangbao Xu2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Email: | {yekui.wang, liuhongbin.01, lizhang.idm, jiaoshaohui, huchao.jason, cuijianyong, xuguangbao} @bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

It is asserted that, although the syntax of MV-HEVC allows for an output layer set (OLS) in an MV-HEVC bitstream to have both texture layers and depth layers, the profile definitions do not allow this. Recently, applications have come up, e.g., virtual reality, that benefit from all the following three aspects simultaneously: 1) using inter-view prediction between different views, 2) having depths associated with the coded views to be available to the decoder and the renderer, and 3) using existing HEVC hardware encoder/decoder designs. It is also asserted that in such applications it is desirable that the bit depth of the depth component can be of up to 16 bits.

This contribution proposes the following:

- Update the definitions of the four new profiles, namely the Multiview Main 10, Multiview Monochrome, Multiview Monochrome 10, and Multiview Monochrome 12 profiles, to allow for an OLS containing a layer conforming to such a profile to have both texture layers and depth layers.

- Add a new profile, named the Multiview Monochrome 16 profile, where it is also allowed for an OLS containing a layer conforming to this profile to have both texture layers and depth layers.

The proposed changes, including some minor editorial changes not summarized above, are included in an attachment, with all the proposed changes marked relative to JVET-AE1006-v1.

During offline discussion after the presentation of JVET-AF0063-v1, the following aspects were newly observed:

- The multilayer extensions of HEVC allow two ways of representing depth:

  - Via DepthLayerFlag  = =  1 (depth), and/or

  - Via AuxId  = =  AUX_DEPTH (auxiliary depth)

And both of the following combinations are allowed:

- DepthLayerFlag  = =  1  &&  AuxId  = =  AUX_DEPTH

- DepthLayerFlag  = =  0  &&  AuxId  = =  AUX_DEPTH

- Although the current definitions of the existing Multiview Main profile and the being-developed Multiview Main 10 profile do not allow having both texture layers and depth layers in an OLS, they do allow having both auxiliary depth layers with chroma_format_idc equal to 1 and texture layers in an OLS but do not allow having both auxiliary depth layers with chroma_format_idc equal to 0 and texture layers in an OLS (referred to as “the wanted combination” below for simplicity).

- Identified another asserted bug for the profile definitions in the CDAM text, on the Multiview Main 10 profile: missing the constraint of colour_mapping_enabled_flag equal to 0 only.

In JVET-AF0063-v2, instead of proposing to allow having both texture layers and depth layers in an OLS, it is proposed to allow the wanted combination. Two options summarized as follows are proposed:

- Update the being-developed Multiview Main 10 profile to allow the wanted combination.

- Keep the being-developed Multiview Main 10 profile disallowing the wanted combination, but add a second Multiview Main 10 profile and also a second Multiview Main profile that both allow the wanted combination.

Besides, both options also include the addition of the Multiview Monochrome 16 profile and updates to the being-developed Multiview Monochrome, Multiview Monochrome 10, and Multiview Monochrome 12 profiles, and all these monochrome profiles allow the wanted combination.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).