| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0071-v1 |
| --- | --- |

| Title: | EE1-related: Further complexity reduction on the joint EE1-0 (LOP.2) unified filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tong Shao, Peng Yin, Sean McCarthy (Dolby) Jay N. Shingala, Ajay Shyam, Ajat Suneja, Siddarth P. Badya (Ittiam) | Email: | tong.shao@dolby.com pyin@dolby.com jay.shingala@ittiam.com ajayshyam@ittiam.com |
| Source: | Dolby Laboratories, Inc., Ittiam Systems |  |  |

_____________________________

# Abstract

A Low-complexity Operation Point (LOP) in-loop filter (LOP1.0) proposed in JVET-AD0156, with complexity of 16.2 kMAC/pixel (block-wise), was adopted in JVET-AD meeting to the NNVC-5.0 software. LOP1.0 served as an anchor for further LOP development. A unified filter architecture for LOP (LOP2.x), was subsequently proposed in JVET-AE0281 to meet LOP complexity constraints while following the HOP training process. The detailed training and inference results of the unified LOP filter are reported in JVET-AF0043.

To further reduce the complexity of the unified LOP2 filter, this contribution proposes to adjust the skip connections in the backbone which enables the fusion of adjacent 1x1 convolutions. This skip-fusion design could reduce the complexity of the unified LOP2.3 from 17.05 kMac/Pixel to 16.56 kMac/Pixel and reduce the number of convolutional layers by 13%. The fast stage 3 training results show that the BD-Rate for fp32 is {0.00%, 0.71%, 0.99%} under AI and {%, %, %} under RA, compared to the best LOP2.3 training results in JVET-AF0043. Compared to NNVC-6.0 VTM anchor, the BD-Rate for fp32 is {-4.77%, -9.14%, -9.13%} under AI and {%, %, %} under RA.

# Introduction

In JVET-AE0281 [2], a unified filter architecture for the LOP was proposed and agreed for a joint training targeting LOP2.x. As shown in Figure 1, the unified LOP model uses the simplified head block, simplified backbone and the split luma-chroma architecture which filter luma and chroma in separate branches using the simplified backbone and simplified tail blocks. As described in JVET-AF0043 [1], the LOP2.3 architecture and parameters are shown in Figure 1.

Figure 1 . LOP2.3 filter architecture of JVET-AF0043.

Figure 2 depicts the architecture of the LOP [3] in-loop filter which was adopted as the NNVC-5.0 reference software anchor. One key feature of this design is the fusion of the two adjacent 1x1 convolutions after CP decomposition, which has the benefit of reducing the NN complexity as well as the number of the NN layers. Since there is no skip connection at the backbone/basic block level, fusion is possible as long as there are adjacent 1x1 layers. However, in LOP2.3 in Figure 1, each backbone block has a skip connection, and because the end point of the skip connection is between two 1x1 convolution layers, the fusion of these two layers is not possible anymore. Therefore, this contribution proposes to adjust the skip connections, i.e., the start and end point of the skip connection, so that there is no start/end point between the two 1x1 convolutions and the two layers could be fused into one single 1x1 convolutional layer to further reduce the complexity.

Figure 2. Architecture of LOP in NNVC-5.0.

# Proposed method

In this contribution, further complexity reduction is proposed based on the LOP2.3 model architecture in Figure 1 The new skip-fusion model architecture is derived from this version.

Figure 3. Proposed skip-fusion LOP model architecture.

As shown in Figure 3, the proposed skip-fusion LOP model architecture is derived from the unified LOP2.3 in Figure 1. The major difference is that the backbone block is redesigned, i.e., the order of the layers is adjusted. Compared to the backbone block in Figure 1, this new backbone block moves the first 1x1 convolutional layer to the end of the block layers. Thus, the skip connection’s start point is before the PReLU, while the end point is right after the moved 1x1 convolutional layer. After this adjustment, it’s obvious that the last two 1x1 convolutional layers could be fused into one single 1x1 convolution. Fusion of adjacent 1x1 convolution layers is performed to further reduce the complexity and reduce the number of  layers. This procedure could also be interpreted as shift all backbone blocks’ skip connections to the right by one layer. All key parameter settings other than the backbone’s layer orders remain the same as the LOP2.3. And an extra 1x1 convolution should be added before and after backbone blocks in Luma and Chroma paths to ensure the input and output channel numbers are consistent after the adjustment. The training dataset and scheduler are also the same as the unified LOP2.3. In the configurations of Figure 3, the complexity of the unified LOP is further reduced from 17.05 kMac/Pixel to 16.56 kMac/Pixel.

# Training and inference results

Fast Stage 3 training is performed using the LOP2.3 stage 3 training dataset and inference results in float point are reported. The training in general follows the stage 3 training described in JVET-AF0043 [1] with the batch size modified to 64.

## Fast Stage 3 dataset

The dataset used in Fast Stage 3 training is generated using the trained LOP2.3 SADL int16 model in stage 2, including DIV2K, BVI-DVC and TVD. Detailed description is reported in JVET-AF0043.

## Training procedure

Stage 3 training strategy follows the JVET-AF0043 [1] with a change in batch size to 64.

Training strategy:

## "stage3" : {
        "training" : {
            "mse_epoch" : 58,
            "max_epochs" : 60,
            "component_loss_weightings" : [12, 2],
            "dataloader": {"batch_size": 64 },
            "optimizer": {"lr": 0.0004, },
            "lr_scheduler" : {"milestones" : [55, 57, 58]
            }
        }
    }

## Inference results

Simulations are performed based on the common test conditions for neural-network-based video coding technology for AI and RA configurations [4]. BD-Rate results for SADL float32 compared to the unified LOP2.3 in JVET-AF0043 with our training results (trained with batch size 64), the unified LOP2.3 in JVET-AF0043 with the best training results (QC) and the NNVC-6.0 VTM anchor are reported. It is noted that the best LOP-2.3 results reported in JVET-AF0043 show more than 0.1% RA and AI gain compared to our training. For fair comparison of the modified skip fusion architecture with LOP-2.3 reference architecture, we have also compared the results with our LOP-2.3 model trained with batch size 64 (bs64).

Table 1, Table 2, and Table 3 show the AI and RA performances of the proposed skip-fusion model. Compared to the unified LOP2.3 with our training results, the BD-Rate for fp32 is {-0.11%, -0.31%, -0.33%} under AI and {%, %, %} under RA. Compared to the unified LOP2.3 best training results, the BD-Rate for fp32 is {0.00%, 0.71%, 0.99%} under AI and {%, %, %} under RA. And compared to NNVC-6.0 VTM anchor, the BD-Rate for fp32 is {-4.77%, -9.14%, -9.13%} under AI and {%, %, %} under RA. The current results show that the proposed skip-fusion model have the following performance:

- 1) for AI, comparable luma performance as the best LOP2.3 training result, and about 1% worse chroma performance.

- 2) for RA,

Table 1. Comparative results of proposed skip-fusion model (bs64) vs. our unified LOP2.3 (bs64), SADL float point

|  | Proposed skip-fusion (bs64) vs. unified LOP2.3 (bs64) SADL float |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.10% | 0.82% | 0.65% |  |  |  |
| Class A2 | -0.10% | -0.05% | -0.51% |  |  |  |
| Class B | -0.08% | -0.45% | -0.13% | -0.16% | 0.73% | -1.00% |
| Class C | -0.15% | -1.15% | -0.69% | -0.19% | -0.57% | -2.67% |
| Class E | -0.13% | -0.37% | -0.99% |  |  |  |
| Overall | -0.11% | -0.31% | -0.33% |  |  |  |
| Class D | -0.10% | 0.13% | -0.89% | -0.13% | 3.66% | -2.80% |

Table 2. Comparative results of proposed skip-fusion model (bs64) vs. best unified LOP2.3 (bs128), SADL float point

|  | Proposed skip-fusion (bs64) vs. best unified LOP2.3 (bs128) SADL float |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.02% | 1.38% | 0.61% |  |  |  |
| Class A2 | 0.07% | 0.90% | 0.62% |  |  |  |
| Class B | -0.01% | 0.49% | 0.81% | -0.04% | 1.60% | 2.07% |
| Class C | -0.03% | 0.29% | 1.09% | -0.14% | 1.09% | 2.01% |
| Class E | 0.02% | 0.80% | 1.90% |  |  |  |
| Overall | 0.00% | 0.71% | 0.99% |  |  |  |
| Class D | 0.03% | 0.52% | 1.55% | -0.09% | 3.17% | 2.46% |

Table 3. Comparative results of proposed skip-fusion model (bs64) vs. NNVC-6.0 VTM anchor, SADL float point

|  | Proposed skip-fusion (bs64) vs. NNVC-6.0 vtm anchor SADL float |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -4.46% | -7.31% | -7.20% |  |  |  |
| Class A2 | -4.35% | -9.31% | -8.05% |  |  |  |
| Class B | -4.35% | -9.40% | -9.39% | -5.06% | -10.18% | -9.63% |
| Class C | -4.63% | -10.24% | -10.08% | -5.33% | -10.99% | -10.56% |
| Class E | -6.41% | -8.88% | -10.46% |  |  |  |
| Overall | -4.77% | -9.14% | -9.13% |  |  |  |
| Class D | -4.93% | -8.25% | -10.39% | -6.28% | -6.49% | -10.25% |

# Conclusions:

This contribution proposes a skip-fusion model based on the unified LOP architecture to further reduce the complexity. By adjusting the skip connections in the backbone blocks, the fusion of adjacent 1x1 convolutions is enabled, which reduces the complexity of the unified LOP model from 17.05 kMac/Pixel to 16.56 kMac/Pixel and the number of convolution layers are also reduced by 20% in the backbone structure and by 13% in the entire LOP2.3 architecture. Fast stage 3 training was performed, and the results compared to the LOP2.3 best training results show that the BD-Rate for fp32 is {0.00%, 0.71%, 0.99%} under AI and {%, %, %} under RA. Compared to NNVC-6.0 VTM anchor, the BD-Rate gain fp32 is {-4.77%, -9.14%, -9.13%} under AI and {%, %, %} under RA. These results show the good trade-off between the BD-Rate and the complexity of the proposed model.

The contribution proposes to further study the skip-fusion model in the next EE.

# References

- AHG11: Status of the joint EE1-0 (LOP.2) unified filter training, JVET-AF0043, Dmytro Rusanovskyy, Yun Li, Marta Karczewicz (Qualcomm), Jay N. Shingala, Ajay Shyam (Ittiam), Tong Shao, Peng Yin (Dolby).

- Unified LOP filter design, training procedure and filter usage, JVET-AE0281, Dmytro Rusanovskyy, Yun Li, Marta Karczewicz (Qualcomm), Tong Shao, Peng Yin, Sean McCarthy (Dolby), Jay N. Shingala, Ajay Shyam (Ittiam), Junru Li, Yue Li, Chaoyi Lin, Kai Zhang, Li Zhang (Bytedance).

- J. N. Shingala, A. Shyam, A. Suneja, S. Badya, T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy, “EE1: Complexity reduction on neural-network loop filter”, JVET-AD0156, Antalya, April 2023

- S. Liu, A. Segall, E. Alshina, R.-L. Liao, “Common Test Conditions and evaluation procedures for neural network-based video coding technology”, JVET-AE2016, July 2023

# Acknowledgement

We would like to thank Qualcomm for crosschecking the inference results.

# Patent rights declaration(s)

Dolby may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).