| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0080-v2 |
| --- | --- |

| Title: | EE2-2.7: An extrapolation filter-based intra prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu, Yue Yu, Haoping Yu, Jonathan Gan, Dong Wang (OPPO) | Tel: Email: | xuluhang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the EE2-2.7 test results for the extrapolation filter-based intra prediction mode (EIP). In the EIP mode, the extrapolation filters are derived from the neighboring reconstructed pixels or inherited from the previous decoded blocks. The extrapolation filter generates the prediction signals for the current block from the top-left position to the bottom-right position in a parallel manner. The tests are conducted on the ECM-10.0 reference software with AI and RA configurations. The test results are reported as follows,

AI(Y/U/V): -0.24%/-0.19%/-0.19%, EncT 103.0%, DecT 100.3%

RA(Y/U/V): -0.10%/-0.12%/-0.19%, EncT 100.7%, DecT 99.9%

Proposed method

The proposed extrapolation filter-based intra prediction is processed in three steps. First, the extrapolation filter coefficients are derived from a neighboring reconstructed area of the current block or inherited from a previous EIP block. Second, the extrapolation process generates predicted signals from the top-left to bottom-right corner within the current block. Third, an intra prediction angle is derived by analyzing the gradient of the predicted block, and then the corresponding intra-mode is used to select the MTS, NSPT and LFNST kernel for transformation.

The application of EIP is restricted to blocks with sizes not greater than 32x32 and the luma component only.

Obtaining the EIP filter

The following three EIP filter shapes are proposed in this method.

Figure-1. The defined three filter shapes with fifteen inputs and one output.

There are two ways to obtain the filter coefficients for the current CU in this proposal. First, the coefficients can be derived from the neighboring reconstructed pixels, and second, they can also be inherited from the previously decoded blocks.

Derivation of EIP coefficients

The decoder decodes the relevant syntax elements to determine the selected type of reconstructed area and the filter shape for the current block. The selected filter moves in the selected reconstructed area either horizontally or vertically with a one-pixel step to construct the auto-correlation matrix and the cross-correlation vector. The calculation of coefficients from the auto-correlation matrix and the cross-correlation vector is the same as that in CCCM.

Figure-2. The defined three types of the reconstructed area. The size of the reconstructed area depends on the min(blockWidth, blockHeight) and the selected filter shape. For example, when the current block is an 8x16 block and the selected filter shape is 4x4. The aboveSize of the reconstructed area is equal to min(8, 16) + 4 – 1 = 11, and the leftSize of the reconstructed area is equal to min(8, 16) + 4 – 1 = 11.

Inheritance of the EIP filters

The EIP merge mode is also proposed in this contribution. The filter shape and the filter coefficients can be inherited from the previous decoded blocks with EIP or EIP merge mode. The decoder decodes an EIP merge flag to decide whether the proposed merge mode is used when the current block uses the EIP mode. A merge index is further decoded when the EIP merge flag is true. The EIP merge list includes spatial adjacent and non-adjacent candidates, temporal candidates, and history candidates. The constructed EIP merge list can include up to 12 candidates and the list will be reduced to up to 6 candidates by the reordering process based on the SAD cost measured on an L-shape template with column width and row height equal to 1. In the SAD calculation, predictions of the template area by EIP filters are generated only from reconstructed (neighbouring and template) samples, allowing the EIP filters to be applied in parallel rather than sequentially.

The positions and inclusion order of the spatial adjacent, temporal, non-adjacent, shifted temporal and history candidates are the same as those defined in ECM-10.0 for the CCP merge prediction candidates.

Prediction of the current block

The EIP mode generates prediction values for the current block from the top-left position to the bottom-right position by a diagonal prediction order, as shown in Figure-3.

Figure-3. Example of generating predictions for different positions in the current block by a diagonal order

The calculation for the prediction values in this contribution is shown as follows,

predx,y=i=014(ci×tx-offsetXi, y-offsetYi)

where predx,y is the predicted value at (x, y) in the current block, ci is the ith coefficient of the selected EIP filter, the index of the coefficients is from 0 to 14, tx-offsetXi, y-offsetYiis a reconstructed or a predicted value used for the current position’s prediction. offsetXi and offsetYi are the position offsets to the current position along x and y directions, respectively.

Mapping to the LFNST/NSPT/MTS set

The proposed method uses the DIMD process to derive an intra prediction mode of the current block based on the EIP predicted samples. Specifically, a horizontal gradient and a vertical gradient are calculated for each predicted sample to build a histogram of gradient (HoG). Then the intra prediction mode corresponding to the largest histogram count is used to determine the LFNST, NSPT or MTS transform set.

Proposed CU level syntax

The EIP related syntax is signaled at CU level. An example of EIP related syntax is shown in the following table.

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| …… |  |
| cu_eip_flag[x0][y0] | ae(v) |
| if(cu_eip_flag[x0][y0]){ |  |
| eip_merge_flag[x0][y0] | ae(v) |
| if(eip_merge_flag[x0][y0]){ |  |
| eip_merge_idx[x0][y0] | ae(v) |
| } |  |
| else{ |  |
| eip_mode_idx[x0][y0] // eip_mode_idx represents the combination of the reconstruction area and the filter shape used to derive the EIP coefficients | ae(v) |
| } |  |
| } |  |
| …… |  |
| } |  |

Simulation results

The proposed method is implemented on top of the ECM-10.0 software [1], and tested based on the common test condition [2]. The test results of the proposed method are summarized in Table 1 and Table 2.

Table 1. The proposed method over ECM-10.0 (AI configuration)

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.19% | -0.13% | -0.17% | 103.3% | 101.8% | 100.3% |
| Class A2 | -0.24% | -0.17% | -0.11% | 103.1% | 101.0% | 100.3% |
| Class B | -0.22% | -0.18% | -0.20% | 103.3% | 100.5% | 100.3% |
| Class C | -0.26% | -0.15% | -0.11% | 102.1% | 98.9% | 100.4% |
| Class E | -0.30% | -0.35% | -0.37% | 103.3% | 99.5% | 100.3% |
| Overall | -0.24% | -0.19% | -0.19% | 103.0% | 100.3% | 100.3% |
| Class D | -0.04% | -0.13% | -0.25% | 103.1% | 100.2% | 100.4% |
| Class F | -0.21% | -0.09% | 0.00% | 101.3% | 100.0% | 100.3% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |

Table 2. The proposed method over ECM-10.0 (RA configuration)

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.09% | -0.27% | -0.09% | 100.6% | 99.8% | 100.7% |
| Class A2 | -0.07% | -0.32% | -0.09% | 100.0% | 99.8% | 100.3% |
| Class B | -0.11% | 0.07% | -0.20% | 100.7% | 100.1% | 100.1% |
| Class C | -0.12% | -0.10% | -0.33% | 101.1% | 99.8% | 100.4% |
| Class E |  |  |  |  |  |  |
| Overall | -0.10% | -0.12% | -0.19% | 100.7% | 99.9% | 100.4% |
| Class D | -0.07% | -0.25% | -0.28% | 100.3% | 101.3% | 100.4% |
| Class F | -0.11% | 0.14% | 0.16% | 100.6% | 100.1% | 100.2% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |

Conclusions

This contribution reports the results of EE2-2.7. The proposed method brings 0.24% coding gain for AI and 0.10% coding gain for RA with reasonable encoding and decoding time changes over the ECM-10.0 reference software. It is suggested to adopt the proposed method in the next version of ECM reference software.

References

- ECM-10.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-10.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, April, 2023.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).