| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0082-v2 |
| --- | --- |

| Title: | Non-EE2: on LFNST/NSPT for inter coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Jonathan Gan Yue Yu Haoping Yu Dong Wang | Tel: Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes to utilize LFNST/NSPT for inter coding. For inter coded block, an intra prediction mode is first derived according to the inter prediction block. Then the derived intra prediction mode is used to select an LFNST/NSPT transform set and the transform can be processed with the selected LFNST/NSPT kernel, like in the intra coding process. On top of ECM-10.0, the performance of the proposed inter LFNST/NSPT is reported as follows:

RA {-0.37% Y, 0.14% U, 0.09% V, 114.0% EncT, 101.3% DecT, 100.0% VmPeak}

Introduction

LFNST (low frequency non-separable transform) is a transform coding tool in VVC. In ECM, LFNST is further enhanced with more transform sets for a finer granularity of directions, i.e., from 4 to 35, and more transform kernels in each set, i.e., from 2 to 3. In addition, larger kernels are introduced for large blocks.

NSPT (non separable primary transform) is introduced to replace the separable DCT-II plus LFNST transform combinations for the block shapes of 4x4, 4x8/8x4, 4x16/16x4, 8x8, 8x16/16x8, 4x32/32x4, and 8x32/32x8 in ECM. The kernel mapping method for intra prediction mode is the same as LFNST.

Inter MTS is enabled in ECM CTC, with DST-VII and DCT-VIII replaced by separable KLTs for small blocks. In the current ECM, LFNST/NSPT are applied to intra coded blocks only. In other words, LFNST/NSPT can’t be used for inter coded blocks. According to our observation, some inter prediction residuals still exhibit directional features. As LFNST/NSPT are more efficient in dealing with directional features, it is desirable to adapt LFNST/NSPT for inter coded blocks.

Proposed method

To utilize LFNST/NSPT for inter coding, a virtual intra prediction mode (VIPM) is employed. Each VIPM represents a kind of feature, for example, a specific direction. The VIPM is derived by applying a DIMD-like process, which is also used in MIP and IntraTMP, to the inter prediction block. The horizontal and vertical gradients of pixels inside the prediction block are calculated with Sobel operator and the horizontal and vertical gradients of each pixel are used to derive a direction. The amplitudes of the gradient of each pixel are accumulated for the corresponding direction. The intra prediction mode corresponding to the strongest accumulation is selected as the VIPM. With the VIPM, the transform set of LFNST/NSPT can be derived. For inter coding, the kernel mapping method is the same as that for intra LFNST/NSPT.

The distribution of inter LFNST/NSPT index is different from that of intra LFNST/NSPT index. Therefore the signaling of inter LFNST/NSPT index is proposed to be different from that of intra LFNST/NSPT index. The intra LFNST/NSPT index binarization employs two context coded bins for each symbol, while it is proposed to use truncated unary code with different context models for inter LFNST/NSPT index coding.

Test results

The proposed methods were implemented on top of the ECM-10.0 software [1] and tested based on the common test condition [2]. The test results are summarized below.

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.30% | 0.08% | -0.12% | 114.7% | 101.0% | 100.2% |
| Class A2 | -0.24% | 0.10% | 0.03% | 114.0% | 101.2% | 99.8% |
| Class B | -0.39% | 0.20% | 0.15% | 113.1% | 101.8% | 99.9% |
| Class C | -0.48% | 0.14% | 0.22% | 114.7% | 100.9% | 100.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.37% | 0.14% | 0.09% | 114.0% | 101.3% | 100.0% |
| Class D | -0.34% | 0.24% | 0.05% | 113.7% | 101.3% | 100.1% |

Conclusions

This contribution proposes to utilize LFNST/NSPT for inter coding. The proposed method brings -0.37% coding gain in RA. In the current implementation the encoding algorithm is not optimized and there is room to significantly reduce the encoding time. It is suggested to investigate the proposed method in EE2.

References

- ECM10.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-10.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AE2017, July, 2023.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).