| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0083-v2 |
| --- | --- |

| Title: | Non-EE2: Enhancements on CCP merge for chroma intra coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hang Huang, Yue Yu, Haoping Yu, Dong Wang (OPPO) Zhipin Deng, Kai Zhang, Li Zhang (Bytedance Inc.) | Tel: Email: | huanghang@oppo.com {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com |
| Source: | OPPO Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a fusion mode for the CCP merge mode, in which the final prediction is generated by a weighted sum between the CCP-merge prediction and the MM-CCCM prediction or the DIMD prediction. A fusion type flag is signalled to indicate whether the MM-CCCM or DIMD is selected. Additionally, CCP merge mode is harmonized with LB-CCP by including the LB-CCP flag in the CCP merge candidate. On top of ECM-10.0, the performance of the proposed contribution is reported as follows:

AI {-0.03%, -0.65%, -0.74%, 100.6%, 100.8%}; RA { }.

Introduction

Cross-component merge mode (CCP merge) and local-boosting cross-component prediction (LB-CCP) have been adopted in ECM-9.0. In the CCP merge mode, a candidate list is created, which includes CCP models collected from the previous blocks coded by CCLM, MMLM, CCCM, GLM, chroma fusion, and CCP merge modes. Then one candidate in the list will be selected as the CCP model for the current block. With LB-CCP, the CCP prediction can be filtered, indicated by a flag.

Proposed method

In the proposed CCP-merge fusion method, a fused prediction is generated by a weighted sum of the CCP-merge prediction and either the MM-CCCM prediction or the DIMD prediction. A CCP-merge fusion flag is signalled conditioned on the CCP-merge flag, to indicate whether the fusion mode is applied. A CCP-merge fusion type flag is further signalled if the CCP-merge fusion flag is true, to indicate whether the MM-CCCM prediction or the DIMD prediction is selected and fused with the CCP-merge prediction.

Besides, it is proposed to harmonize CCP merge mode and LB-CCP. The LB-CCP flag could be inherited from a CCP candidate in the CCP merge candidate list.

Test results

The proposed methods were implemented on top of the ECM-10.0 software and tested based on the common test condition [2]. The test results are summarized below.

Table 1. Simulation results of the proposed method

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.01% | -1.12% | -1.26% | 100.6% | 100.9% | 100.0% |
| Class A2 | -0.10% | -0.67% | -0.64% | 100.6% | 100.3% | 99.9% |
| Class B | -0.01% | -0.67% | -0.83% | 100.6% | 101.0% | 100.0% |
| Class C | -0.02% | -0.38% | -0.29% | 100.7% | 101.4% | 100.0% |
| Class E | -0.02% | -0.49% | -0.81% | 100.6% | 100.3% | 100.0% |
| Overall | -0.03% | -0.65% | -0.74% | 100.6% | 100.8% | 100.0% |
| Class D | -0.02% | -0.14% | -0.28% | 101.1% | 98.9% | 100.0% |
| Class F | -0.02% | -0.20% | -0.14% | 100.5% | 101.2% | 100.0% |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | -0.01% | -0.12% | -0.19% | 100.7% | 100.6% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D | -0.05% | 0.21% | -0.66% | 100.8% | 100.3% | 100.0% |
| Class F |  |  |  |  |  |  |

Conclusions

This contribution proposes improvements to the cross-component prediction merge mode. The results show that the gain of the CCP merge mode can be further improved. It is suggested to further study this proposal in EE2.

References

- ECM10.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-10.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, April, 2023.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).