| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0086-v1 |
| --- | --- |

| Title: | EE1-1.2.6: On flipping of input and output of model in NNVC HOP filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the results of EE1-1.2.6. In EE1-1.2.6, flipping operations applied to the inputs and output of the NNVC HOP filter neural network are proposed to further improve the coding performance. The simulation results are summarized as follows:

Test 1.2.6: The proposed method,

when compared with NNVC-6.0 HOP filter anchor,

RA: {-0.08%, -0.48%, -0.69%} EncT 98%, DecT 99%;

LDB: {-0.39%, -2.09%, -1.43%} EncT 98%, DecT 101%.

when compared with NNVC VTM anchor,

RA: {-11.71%	, -26.02%	, -26.51%} EncT 184%, DecT 51546%;

LDB: {-11.05%, -23.09%, -16.79%} EncT 197%, DecT 42268%.

Introduction

In the current NNVC HOP filter[2], the rec_yuv, pred_yuv, bs_yuv, IBP_yuv, and QP information are injected into the network. The network outputs the filtered samples. Besides, some tools are applied to the usage of the HOP filter to further improve the coding performance, such as QP offset and residual scaling.

Proposed

This EE studies the proposed flipping method on top of the NNVC HOP filter. To reduce the encoding time, an early termination method on the encoder side, as shown in Fig. 1, is used to skip some CTUs’ inferences during the determination of QP offset[3] and flipping type.

The encoder selects the best QP offset and flipping type by applying them to the inputs and executing the inference, then comparing the distortion between the filtered pictures and original pictures. Finally, the candidate (QP offset or flipping type) with the smallest distortion will be selected and applied to all CTUs in the current slice. Slice-level flags are signalled in the bitstream to indicate the usage of flipping and the flipping type that is applied to the current slice.

CTU

CTU

Fig.1 Encoder-side method, only shadowed CTUs will be checked.

Inference

The inference information is identical to NNVC-6.0 HOP filter, as shown in Table 1.

Table. 1 Inference Information

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.45M |
|  | Total Number of Parameters (All Models) | 1.45M |
|  | Parameter Precision (Bits) | 16 |
|  | Memory Parameter (MB) | 2.8MB/model, 1 model in all |
|  | Multiply Accumulate (kMAC/pixel) | 477K |
|  | Calculation Method | block basis |
| Optional |  |  |
|  | Total Conv. Layers | 154 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

No training is conducted in this contribution.

Experimental results

The proposed method is implemented on top of the NNVC-6.0 HOP filter and all of the tests are conducted under the common test condition[4].

NNVC-6.0 HOP anchor

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.08% | -0.42% | 98% | 96% |
| Class A2 | -0.07% | -0.41% | -0.86% | 98% | 97% |
| Class B | -0.08% | -0.53% | -0.68% | 97% | 101% |
| Class C | -0.10% | -0.77% | -0.77% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.48% | -0.69% | 98% | 99% |
| Class D | -0.18% | -0.83% | -1.51% | 99% | 101% |
| Class F |  |  |  |  |  |

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.28% | -2.20% | -1.48% | 96% | 101% |
| Class C | -0.35% | -2.82% | -2.90% | 98% | 100% |
| Class E | -0.63% | -0.94% | 0.62% | 102% | 104% |
| Overall | -0.39% | -2.09% | -1.43% | 98% | 101% |
| Class D | -0.43% | -2.72% | -3.40% | 100% | 96% |
| Class F | -0.42% | -1.83% | -2.26% | 100% | 106% |

NNVC-6.0 VTM anchor

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -12.27% | -21.97% | -27.28% | 175% | 48203% |
| Class A2 | -11.93% | -24.50% | -24.09% | 167% | 45725% |
| Class B | -11.10% | -27.03% | -25.60% | 203% | 56074% |
| Class C | -11.87% | -28.94% | -28.89% | 184% | 53378% |
| Class E |  |  |  |  |  |
| Overall | -11.71% | -26.02% | -26.51% | 184% | 51546% |
| Class D | -13.14% | -28.16% | -29.25% | 269% | 89424% |
| Class F |  |  |  |  |  |

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -10.13% | -22.71% | -16.52% | 174% | 38011% |
| Class C | -11.15% | -27.77% | -25.05% | 166% | 43296% |
| Class E | -12.46% | -17.50% | -6.24% | 307% | 48857% |
| Overall | -11.05% | -23.09% | -16.79% | 197% | 42268% |
| Class D | -12.34% | -25.90% | -19.10% | 128% | 30794% |
| Class F | -7.48% | -20.84% | -16.31% | 211% | 17699% |

Conclusion

This contribution reports the results of EE1-1.2.6, which proposed applying flipping operations to both the input and output of the NNVC HOP filter to improve the coding performance. This proposed method can provide -0.08% coding gain for RA and -0.39% coding gain for LDB without running time impact. Considering its good trade-off performance, it is suggested to adopt this method into the NNVC common software.

Reference

[1] Z. Xie, Y. Yu, H. Yu, D. Wang. Non-EE1: On flipping of input and output of model in NNVC filter set 0. JVET-AC0065, 29th Meeting, by teleconference, 11-20 Jan.

[2] F. Galpin, et. al. AhG11 - EE1-0 High Operation Point model. JVET-AE0191, 31st Meeting, Geneva, 11–19 July 2023.

[3] Z. Xie, Y. Yu, H. Yu, D. Wang. EE1-1.3: On BaseQP adjustment in CNNLF. JVET-AA0122, 27th Meeting, by teleconference, 13-22 July 2022.

[4] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AE2016, 31st Meeting, 11–19 July 2023.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).