| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0087-v1 |
| --- | --- |

| Title: | AHG9: Experimental results of object mask auxiliary picture coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zixiang Zhang, Jie Chen, Yan Ye, Shurun Wang | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

The object mask information (OMI) SEI message was included in the first draft of technology under considerations for future extensions of VSEI at the last meeting. With OMI SEI message, the object masks are sent as auxiliary pictures. This contribution reports the experimental results of coding object masks as auxiliary pictures with VTM layered coding. According to the coding results, it is reported that it is practical to code the mask pictures as auxiliary pictures.

Introduction

Annotated region (AR) SEI message was adopted to VSEI and HEVC. It allows results of video analysis performed by the encoder, such as information about object bounding boxes and object labels, to be sent to the decoder. This could reduce the workload and power consumption of the decoder. However, AR SEI message does not support sending detailed shape information of objects which could be essential to some applications. Thus, in JVET-AD0175 [1] and JVET-AE0095 [2], it was proposed to send the object masks as the auxiliary pictures which are coded in a separate layer and signal the related information in a new type of SEI message named object mask information (OMI) SEI message.

The proposed OMI SEI message was included in the first draft of technology under considerations for future extensions of VSEI [3] in the AE meeting, and it was further recommended to bring experimental results with real-world examples rather than hand-crafted examples.

In this meeting, the experimental results of coding object masks as auxiliary pictures are reported in this contribution and software implementation of OMI SEI message are provided in JVET-AF0088 [4].

Experimental settings

Experiments are conducted to code the object mask auxiliary pictures using VTM-21.0 layered coding. The flow-chart of the experiments is shown in Figure 1.

Figure 1. The flow-chart of the object mask auxiliary picture coding

- Mask sequence generation

The JVET CTC [5] sequences were used in the experiment. The object masks were generated by invoking Mask R-CNN model in Detectron2 [6] and MaskTrack R-CNN model in MMTracking [7]. For simplicity, the experiments only consider objects that are labelled as “person”.

Two types of mask sequences were generated: 1) single-mask sequences and 2) multiple-mask sequences. For single-mask sequences, all the detected persons in the picture are treated as one mask (with the same mask ID), and mask ID (i.e., the sample value in the mask area) is set to 255. For multiple-mask sequences, at most four persons in a picture are kept, and the mask IDs (i.e., the sample value in the mask area) are set to 64, 128, 192 and 255. The background samples are set to 0.

Examples of single-mask and multiple-mask sequences are given in Figure 2 and Figure 3, respectively. The bit-depth of the mask sequences is 8-bit and the color format is YUV400.

Figure 2. An example of single-mask sequence (all persons with the same mask IDs)

Figure 3. An example of multiple-mask sequence (different persons with different mask IDs)

- Coding of object mask auxiliary pictures

Coding is performed with VTM21.0. Multi-layer coding is used. The normal sequence was coded as primary pictures in layer 0 and the accompanying mask sequence was coded as auxiliary pictures in layer 1.

For layer 0 coding, the encoding configurations follow JVET CTC. And for layer 1 coding, two experiments were conducted. In the first experiment, lossless coding is used and in the second experiment, lossy coding is used. For lossy coding of layer 1, the loop-filters are disabled and other configurations follow JVET CTC, including QP settings.

- Mask recovery

As the mask IDs are signaled in OMI SEI message to the decoder, the decoder can make a further decision for each decoded sample whether it belongs to background area or to mask area based on the mask IDs and the decoded sample value. Thus, a “mask recovery” process is added after the mask sequences are decoded. It outputs the recovered mask sequences.

For single-mask sequences, the samples with decoded value less than or equal to 128 are decided as background area and the sample value is set to 0; the samples with decoded value larger than 128 are decided as mask area and the sample value is set to 255.

For multiple-mask sequences, the samples with decoded value less than or equal to 32 are decided as background area and the sample value is set to 0; the samples with decoded value larger than 32 and less than or equal to 96 are decided as mask0 area and the sample value is set to 64; the samples with decoded value larger than 96 and less than or equal to 160 are decided as mask1 area and the sample value is set to 128; the samples with decoded value larger than 160 and less than or equal to 223 are decided as mask2 area and the sample value is set to 192; the samples with decoded value larger than 223 are decided as mask3 area and the sample value is set to 255.

- Distortion calculation

After the mask recovery, the recovered mask sequences are compared with the original mask sequences. The number of correct samples and incorrect sample are counted. If the recovered value of a sample is the same as its original value, it is counted as a correct sample; otherwise, it is counted as an incorrect sample. Then incorrect ratio is calculated as follows

incorrect ratio=incorrect sample numbertotal sample number

Experimental results

Single-mask case

The ratio of mask layer bitrate to the total bitrate is calculated as follows and the results are summarized in Table 1.

bitrate ratio=bitrate of layer 1bitrate of layer 0+bitrate of layer 1

Table 1. The bitrate ratio of mask sequence in single-mask case

| class | QP | Lossy coding for mask pictures |  |  | Lossless coding for mask pictures |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AI | RA | LD | AI | RA | LD |
| class A1 | 22 | 3.97% | 12.40% | 9.68% | 5.28% | 13.61% | 10.57% |
|  | 27 | 10.65% | 22.90% | 21.87% | 14.39% | 25.57% | 24.37% |
|  | 32 | 16.96% | 33.64% | 33.83% | 23.55% | 37.18% | 36.75% |
|  | 37 | 23.83% | 45.89% | 45.74% | 33.73% | 49.82% | 49.14% |
| class A2 | 22 | 0.23% | 0.60% | 0.45% | 0.29% | 0.67% | 0.50% |
|  | 27 | 0.52% | 1.39% | 1.31% | 0.72% | 1.64% | 1.53% |
|  | 32 | 0.90% | 2.77% | 2.81% | 1.34% | 3.34% | 3.30% |
|  | 37 | 1.50% | 5.45% | 5.46% | 2.42% | 6.55% | 6.44% |
| class B | 22 | 2.61% | 6.53% | 5.39% | 3.50% | 7.19% | 5.89% |
|  | 27 | 4.74% | 13.29% | 12.77% | 6.75% | 15.18% | 14.44% |
|  | 32 | 7.75% | 22.34% | 23.79% | 11.54% | 25.45% | 26.35% |
|  | 37 | 11.99% | 34.18% | 36.94% | 18.49% | 38.18% | 40.29% |
| class C | 22 | 3.18% | 8.96% | 7.98% | 4.48% | 10.04% | 8.84% |
|  | 27 | 4.99% | 15.90% | 16.03% | 7.52% | 18.27% | 18.13% |
|  | 32 | 7.81% | 25.73% | 27.61% | 12.34% | 29.41% | 30.54% |
|  | 37 | 12.05% | 38.40% | 41.46% | 19.84% | 42.97% | 45.20% |
| class D | 22 | 4.73% | 11.04% | 9.06% | 6.92% | 12.68% | 10.33% |
|  | 27 | 6.99% | 19.89% | 19.03% | 10.82% | 23.48% | 22.22% |
|  | 32 | 10.69% | 32.07% | 34.42% | 17.06% | 37.43% | 39.05% |
|  | 37 | 16.36% | 46.68% | 51.79% | 26.69% | 53.04% | 57.19% |
| class E | 22 | 6.88% | 26.24% | 27.50% | 9.29% | 27.71% | 28.61% |
|  | 27 | 10.54% | 39.37% | 50.90% | 15.02% | 42.44% | 53.34% |
|  | 32 | 15.49% | 52.35% | 68.04% | 22.94% | 56.20% | 70.31% |
|  | 37 | 21.72% | 64.70% | 79.64% | 33.15% | 68.49% | 81.38% |
| overall | 22 | 3.54% | 10.47% | 9.45% | 4.90% | 11.49% | 10.23% |
|  | 27 | 6.21% | 18.21% | 19.38% | 8.98% | 20.54% | 21.42% |
|  | 32 | 9.67% | 27.69% | 30.96% | 14.49% | 31.12% | 33.69% |
|  | 37 | 14.31% | 39.06% | 43.19% | 22.11% | 43.16% | 46.45% |

The incorrect ratio is given in Table 2.

Table 2 incorrect sample ratio for lossy coding in single-mask case

|  |  | AI | RA | LD |
| --- | --- | --- | --- | --- |
| class A1 | 22 | 0 | 0 | 0 |
|  | 27 | 0 | 0 | 0 |
|  | 32 | 0 | 5.36×10-10 | 8.09×10-10 |
|  | 37 | 0 | 3.15×10-7 | 4.32×10-7 |
| class A2 | 22 | 0 | 0 | 0 |
|  | 27 | 0 | 0 | 0 |
|  | 32 | 0 | 1.34×10-10 | 4.02×10-10 |
|  | 37 | 0 | 3.34×10-8 | 3.76×10-8 |
| class B | 22 | 0 | 0 | 0 |
|  | 27 | 0 | 0 | 0 |
|  | 32 | 0 | 3.21×10-10 | 4.82×10-10 |
|  | 37 | 0 | 2.87×10-7 | 4.31×10-7 |
| class C | 22 | 0 | 0 | 0 |
|  | 27 | 0 | 0 | 0 |
|  | 32 | 0 | 2.30×10-9 | 0 |
|  | 37 | 0 | 6.51×10-7 | 9.54×10-7 |
| class D | 22 | 0 | 0 | 0 |
|  | 27 | 0 | 0 | 0 |
|  | 32 | 0 | 0 | 0 |
|  | 37 | 0 | 2.28×10-6 | 2.85×10-6 |
| class E | 22 | 0 | 0 | 0 |
|  | 27 | 0 | 0 | 0 |
|  | 32 | 0 | 6.03×10-10 | 0 |
|  | 37 | 0 | 1.18×10-6 | 1.21×10-6 |
| overall | 22 | 0 | 0 | 0 |
|  | 27 | 0 | 0 | 0 |
|  | 32 | 0 | 6.64×10-10 | 2.75×10-10 |
|  | 37 | 0 | 8.06×10-7 | 1.02×10-6 |

Multiple-mask case

The ratio of mask layer bitrate to the total bitrate is calculated and summarized in Table 3.

Table 3. The bitrate ratio of mask sequence in multiple-mask case

| class | QP | Lossy coding for mask pictures |  |  | Lossless coding for mask pictures |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AI | RA | LD | AI | RA | LD |
| class A1 | 22 | 0.97% | 3.82% | 2.93% | 1.29% | 4.37% | 3.37% |
|  | 27 | 3.12% | 7.67% | 7.45% | 4.01% | 9.13% | 8.71% |
|  | 32 | 4.84% | 11.91% | 11.85% | 6.84% | 15.25% | 15.11% |
|  | 37 | 6.45% | 15.43% | 15.37% | 10.79% | 23.23% | 23.02% |
| class A2 | 22 | 0.16% | 0.43% | 0.35% | 0.20% | 0.48% | 0.39% |
|  | 27 | 0.37% | 0.94% | 0.92% | 0.46% | 1.09% | 1.07% |
|  | 32 | 0.60% | 1.62% | 1.64% | 0.84% | 2.25% | 2.29% |
|  | 37 | 0.83% | 2.44% | 2.42% | 1.51% | 4.48% | 4.49% |
| class B | 22 | 0.97% | 2.70% | 2.23% | 1.31% | 3.02% | 2.48% |
|  | 27 | 1.99% | 5.49% | 5.22% | 2.63% | 6.36% | 5.96% |
|  | 32 | 3.27% | 9.07% | 9.07% | 4.71% | 11.42% | 11.41% |
|  | 37 | 4.67% | 12.79% | 12.92% | 8.00% | 19.01% | 19.45% |
| class C | 22 | 1.58% | 5.01% | 4.45% | 2.26% | 5.84% | 5.15% |
|  | 27 | 2.64% | 9.46% | 9.73% | 3.88% | 11.42% | 11.48% |
|  | 32 | 4.23% | 16.26% | 17.81% | 6.62% | 20.11% | 21.44% |
|  | 37 | 6.59% | 23.17% | 26.12% | 11.31% | 32.42% | 35.21% |
| class D | 22 | 2.90% | 8.07% | 6.68% | 4.32% | 9.65% | 8.01% |
|  | 27 | 4.83% | 14.73% | 14.26% | 6.92% | 18.18% | 17.41% |
|  | 32 | 7.37% | 22.34% | 23.98% | 11.34% | 30.29% | 31.88% |
|  | 37 | 10.48% | 27.69% | 31.65% | 18.77% | 45.09% | 49.31% |
| class E | 22 | 4.77% | 22.06% | 23.89% | 6.68% | 23.52% | 24.90% |
|  | 27 | 8.34% | 33.95% | 45.88% | 11.04% | 37.03% | 48.20% |
|  | 32 | 11.82% | 42.71% | 59.71% | 17.33% | 50.41% | 65.47% |
|  | 37 | 14.57% | 47.35% | 66.30% | 25.87% | 63.13% | 77.61% |
| overall | 22 | 1.84% | 6.58% | 6.24% | 2.61% | 7.37% | 6.87% |
|  | 27 | 3.42% | 11.45% | 12.95% | 4.68% | 13.27% | 14.52% |
|  | 32 | 5.21% | 16.75% | 19.64% | 7.75% | 21.02% | 23.59% |
|  | 37 | 7.15% | 21.05% | 24.91% | 12.49% | 30.80% | 34.12% |

The incorrect ratio is given in Table 4.

Table 4 incorrect sample ratio for lossy coding in multiple-mask case

|  |  | AI | RA | LD |
| --- | --- | --- | --- | --- |
| class A1 | 22 | 0 | 4.1×10-10 | 1.34×10-10 |
|  | 27 | 0 | 7.21×10-7 | 3.73×10-7 |
|  | 32 | 3.19×10-7 | 1.47×10-5 | 1.52×10-5 |
|  | 37 | 1.71×10-5 | 8.03×10-5 | 8.5×10-5 |
| class A2 | 22 | 0 | 1.34×10-10 | 0 |
|  | 27 | 0 | 1.78×10-7 | 5.89×10-8 |
|  | 32 | 2.35×10-7 | 7.96×10-6 | 6.88×10-6 |
|  | 37 | 1.02×10-5 | 3.61×10-5 | 3.46×10-5 |
| class B | 22 | 0 | 1.61×10-10 | 0 |
|  | 27 | 0 | 7.15×10-7 | 2.97×10-7 |
|  | 32 | 5.72×10-7 | 2.37×10-5 | 2.27×10-5 |
|  | 37 | 2.98×10-5 | 1.26×10-4 | 1.31×10-4 |
| class C | 22 | 0 | 0 | 0 |
|  | 27 | 0 | 7.82×10-7 | 4.17×10-7 |
|  | 32 | 6.51×10-7 | 4.75×10-5 | 5.27×10-5 |
|  | 37 | 5.73×10-5 | 3.4×10-4 | 3.96×10-4 |
| class D | 22 | 0 | 0 | 0 |
|  | 27 | 0 | 1.01×10-5 | 3.18×10-6 |
|  | 32 | 1.71×10-6 | 3.55×10-4 | 3.55×10-4 |
|  | 37 | 2.88×10-4 | 1.591×10-4 | 1.74×10-3 |
| class E | 22 | 0 | 1.21×10-9 | 0 |
|  | 27 | 0 | 4.86×10-6 | 1.37×10-6 |
|  | 32 | 2.65×10-6 | 1.24×10-4 | 1.11×10-4 |
|  | 37 | 1.38×10-4 | 4.49×10-4 | 4.87×10-4 |
| overall | 22 | 0 | 2.75×10-10 | 1.83×10-11 |
|  | 27 | 0 | 2.93×10-6 | 9.68×10-7 |
|  | 32 | 9.97×10-7 | 9.86×10-5 | 9.74×10-5 |
|  | 37 | 9.21×10-5 | 4.57×10-4 | 5.0×10-4 |

According to the results above, the percentage of the mask layer bitrate is sequence dependent and configuration dependent, which is as expected. On average, the mask layer bitrate comprises a small percentage of the total bitrate, especially when the primary pictures are coded with low QPs. The bitrate ratio difference between lossy coding and lossless coding of the mask pictures is not very significant, as these mask pictures are simple and relatively easy to encode. And even in the lossy coding case, the percentage of incorrect mask samples after mask reconstruction is extremely small.

Conclusion

This contribution reports results of coding mask pictures as auxiliary pictures with VTM-21.0 layered coding. Both lossless and lossy coding of mask pictures are performed. Results show that it is practical to code the mask pictures as auxiliary pictures. It is suggested to adopt the OMI SEI message into next version of VSEI to support coding the object masks as auxiliary pictures.

Reference

J. Chen, S. Wang, Y. Ye, “AHG8/AHG9: On object mask auxiliary picture and object mask information SEI message for VSEI and HEVC”, JVET-AD0175, Apri. 2023.

P. de Lagrange, E. François, D. Doyen, J. Chen, S. Wang, Y. Ye, “AHG8/AHG9: Proposed changes to the candidate new object mask information SEI message”, JVET-AE0095, July 2023.

M. M. Hannuksela, J. Chen, S. Deshpande, S. McCarthy, “Technologies under consideration for future extensions of VSEI (draft 1)”, JVET-AE2032, Sept. 2023.

J. Chen, Z. Zhang, Y. Ye, S. Wang, “AHG9: Software implementation and further fixes of the object mask information SEI message”, JVET-AF0088, Oct. 2023

M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AE2017, July 2023.

https://github.com/facebookresearch/detectron2.git

https://github.com/open-mmlab/mmtracking.git

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).