| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0088-v1 |
| --- | --- |

| Title: | AHG9: Software implementation and further fixes of the object mask information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Zixiang Zhang, Yan Ye, Shurun Wang | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

This contribution provides the software implementation of the object mask information (OMI) SEI message and also proposes fixes and further changes to the OMI SEI message in the draft 1 of technologies under consideration for future extensions of VSEI.

The proposed fixes and changes include the following four aspects:

- Aspect 1: The text related to bounding box parameters was fixed and refined.

- Aspect 2: The binarization of bounding box parameters was changed from ue(v) to u(16)

- Aspect 3: A gating flag for bounding box parameters was added to give signaling flexibility to the encoder

- Aspect 4: The parsing dependency among different OMI SEI messages was removed by always signaling omi_mask_cancel[ i ][ j ][ k ]

Introduction

With the increase of intelligent applications such as object detection, segmentation and tracking, machine tasks become a common workload for end-to-end video systems. To achieve more accurate machine task results and reduce the decoder side workload, JVET-AD0175 [1] and JVET-AE0095 [2] proposed to code the object masks as auxiliary pictures and signal the related information in a new type of SEI message named OMI SEI message so that the object masks extracted by the encoder side from original videos can be sent in the bitstream. In the AE meeting, the proposed OMI SEI message was included in the first draft of technology under considerations for future extensions of VSEI [3].

In this meeting, JVET-AF0087 [4] reports the coding results of the proposed object mask auxiliary pictures and shows that it is practical to code object masks as auxiliary pictures. This contribution provides the software implementation of the OMI SEI message as in the attachment. And some fixes and further improvements to the current OMI SEI message are also proposed in this contribution.

Software implementation of OMI SEI message

The OMI SEI message was implemented on top of VTM-21.2. The software package is attached to the contribution. And an example of encoder configuration is also provided in the package. To enable OMI SEI message, the multi-layer coding configuration of VTM should be used. And as the auxiliary picture is specified in scalability dimension information (SDI) SEI message, the SDI SEI message should also be enabled.

The following is an example of encoder and decoder command to launch the software. “-omi” specifies configuration files for object mask information in encoder side and “----SEIObjectMaskInfosFilename” specifies the output file name for OMI SEI message in decoder side.

EncoderApp.exe -c .\cfg\multi-layer\two_layers_independent.cfg -c .\cfg\sei_vui\scalability_dimension_info.cfg -c .\cfg\encoder_lowdelay_vtm.cfg -l0 -c .\cfg\per-sequence\BasketballPass.cfg -l1 -c .\cfg\per-sequence\BasketballPass.cfg -omi .\cfg\sei_vui\object_mask_infos\lowdelay\mask_info -b test.bin -f 5 -q 63

DecoderApp.exe -p 0 -b test.bin -o dec.yuv --SEIObjectMaskInfosFilename=dec_omi.txt

Proposals

Proposal 1: Text fix for bounding box parameters

It is proposed to clearly specify that the coordinates of the bounding box of the object mask are defined on the cropped part of the decoded picture, relative to the conformance cropping window specified by the active SPS to eliminate the ambiguity. This definition aligns with bounding box coordinates definition of AR SEI message in VSEI.

Additionally, it also proposed to add the definition of variables CroppedWidth , CroppedHeight  and  SubWidthC which were used but not defined in the draft yet.

The text changes of proposal 1 is highlighted in magenta.

Proposal 2: Change the binarization of bounding box parameters

In the current TuC draft, the bounding box parameters, i.e., top-left coordinates and width and height of the bounding box, are coded with ue(v). Considering that the objects are usually uniformly distributed in the picture, it is proposed to use fix length code for these bounding box parameters. This also aligns with the AR SEI message design in VSEI.

The text changes of proposal 2 is highlighted in cyan.

Proposal 3: Add a gating flag for bounding box parameters

As the object mask itself is already coded as auxiliary picture, the bounding box signaled in SEI message is just to help delimit the object mask in the auxiliary picture. Thus, it is proposed to add a gating flag to make the signaling of bounding box parameters optional, which gives the flexibility to the encoder to decide whether to signal the bounding box to delimit the mask or not.

The text changes of proposal 3 is highlighted in yellow.

Proposal 4: Remove parsing dependency among different OMI SEI messages

In the current TuC, an object mask cancel flag omi_mask_cancel[ i ][ j ][ k ] is signalled to indicate whether to cancel the object mask with identifier equal to omi_mask_id[ i ][ j ][ k ]. This flag is only signalled when it is not the first time that an object mask with identifier equal to omi_mask_id[ i ][ j ][ k ] is present in the SEI message in the current CLVS, as a mask which didn’t exist before cannot be cancelled. Thus, the decoder has to maintain an object mask identifier list and update it whenever an object mask identifier is parsed. When parsing the SEI message, the decoder needs to check the list to decide whether to parse omi_mask_cancel[ i ][ j ][ k ] flag or not, which introduces parsing dependency on the previous SEI messages.

To simplify the design, it is proposed to always signal the omi_mask_cancel[ i ][ j ][ k ] flag so that there is no dependency among the different OMI SEI messages in parsing process, and add a constraint that if an object mask with identifier equal to omi_mask_id[ i ][ j ][ k ] is present in the SEI message for the first time in the current CLVS, the value of omi_mask_cancel[ i ][ j ][ k ] shall be 0.

This issue was discovered when the OMI SEI message software was being implemented, and it is reported that removing this parsing dependency streamlines the software implementation.

The text changes of proposal 4 is highlighted in green.

Text changes of the proposed aspects

    - Object mask information SEI message syntax

| object_mask_info( payloadSize ) { | Descriptor |
| --- | --- |
| omi_cancel_flag | u(1) |
| if( !om_cancel_flag ) { |  |
| omi_aux_id_minus128 | ue(v) |
| omi_num_primary_pic_layer_minus1 | ue(v) |
| for(i = 0; i <= omi_num_primary_pic_layer_minus1; i++ ) |  |
| omi_primary_pic_layer_id[ i ] | ue(v) |
| omi_mask_id_length_minus8 | ue(v) |
| omi_mask_confidence_info_present_flag | u(1) |
| if( omi_mask_confidence_info_present_flag ) |  |
| omi_mask_confidence_length_minus1 | u(4) |
| omi_mask_depth_info_present_flag | u(1) |
| if( omi_mask_depth_info_present_flag ) |  |
| omi_mask_depth_length_minus1 | u(4) |
| omi_mask_label_info_present_flag | u(1) |
| if( omi_mask_label_info_present_flag ) { |  |
| omi_mask_label_language_present_flag | u(1) |
| if( omi_mask_label_language_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| omi_bit_equal_to_zero | f(1) |
| omi_mask_lable_language | st(v) |
| } |  |
| } |  |
| for( i = 0; i <= omi_num_primary_pic_layer_minus1; i++ ) { |  |
| for( j = 0; j < numAuxLayer[ omi_primary_pic_layer_id[ i ] ]; j++ ){ |  |
| omi_mask_pic_update_flag[ i ][ j ] | f(1) |
| if( omi_mask_pic_update_flag[ i ][ j ]) { |  |
| omi_num_mask_in_pic_update[ i ][ j ] | ue(v) |
| for( k = 0; k < omi_num_mask_in_pic_update[ i ][ j ]; k++ ) { |  |
| omi_mask_id[ i ][ j ][ k ] | u(v) |
| omi_mask_bounding_box_present_flag[ i ][ j ][ k ] | u(1) |
| if (omi_mask_bounding_box_present_flag[ i ][ j ][ k ]) { |  |
| omi_mask_top[ i ][ j ][ k ] | ue(v) u(16) |
| omi_mask_left[ i ][ j ][ k ] | ue(v) u(16) |
| omi_mask_width[ i ][ j ][ k ] | ue(v) u(16) |
| omi_mask_height[ i ][ j ][ k ] | ue(v) u(16) |
| } |  |
| if( maskIdExist[ i ][ j ][ omi_mask_id[ i ][ j ][ k ] ] ) { |  |
| omi_mask_cancel[ i ][ j ][ k ] | u(1) |
| maskIdExist[ i ][ j ][ omi_mask_id[ i ][ j ][ k ] ] = !omi_mask_cancel[ i ][ j ][ k ] |  |
| } |  |
| else |  |
| maskIdExist[ i ][ j ][ omi_mask_id[ i ][ j ][ k ] ] = 1 |  |
| if( maskIdExist[ i ][ j ][ omi_mask_id[ i ][ j ][ k ] ] !omi_mask_cancel[ i ][ j ][ k ] ) { |  |
| if( omi_mask_confidence_info_present_flag ) |  |
| omi_mask_confidence[ i ][ j ][ k ] | u(v) |
| if( omi_mask_depth_info_present_flag ) |  |
| omi_mask_depth[ i ][ j ][ k ] | u(v) |
| while( !byte_aligned( ) ) |  |
| omi_bit_equal_to_zero | f(1) |
| if( omi_mask_label_info_present_flag ) |  |
| omi_mask_label[ i ][ j ][ k ] | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

    - Object mask information information SEI message semantics

The object mask information (OMI) SEI message provides information about object mask pictures coded as auxiliary pictures. Object mask auxiliary pictures have nuh_layer_id equal to sdi_layer_id[ i ] and sdi_aux_id [ i ] in the range of 128 to 159, inclusive, for any value of i in range of 0 to sid_max_layers_minus1, inclusive.

NOTE 1 – Each object mask auxiliary picture layer is associated with one primary picture layer and one primary picture layer may be associated with one or more object mask auxiliary picture layers.

Use of this SEI message requires the definition of the following variables:

– A cropped picture width and picture height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

– A conformance cropping window left offset, ConfWinLeftOffset

– A conformance cropping window top offset, ConfWinTopOffset

– A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.3.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

When an access unit contains an auxiliary picture picA in a layer, with nuh_layer_id equal to nuhLayerIdA, that is indicated as an object mask auxiliary layer by an OMI SEI message, and a primary picture picB in a layer, with nuh_layer_id equal to nuhLayerIdB, that is indicated as a primary layer by the OMI SEI message, OMI SEI message persists in output order until one or more of the following conditions are true:

– A CLVS containing the auxiliary picture picA ends.

– A CLVS containing the primary picture picB ends.

– A CVS ends.

– The bitstream ends.

…

omi_num_mask_in_pic_update[ i ][ j ] indicates the number of object masks of which the information to be signalled in the j-th auxiliary picture associated with i-th primary picture. omi_num_mask_in_pic_update [ i ][ j ] shall be in the range of 0 to (1<<BitDepthY) – 1, inclusive, where BitDepthY is the bit depth for the samples of the luma component. The variable omiNumMaskInPic[ i ][ j ] indicating the number of object masks in the j-th auxiliary picture associated with i-th primary picture is set to omi_num_mask_in_pic_update[ i ][ j ] when the current SEI message is the first OMI SEI message in the current CLVS.

The variable numAuxLayer[ primaryLayerId ] indicates the number of the auxiliary picture layers associated with primary picture layer with nuh_layer_id equal to primaryLayerId. The variable associatedAuxLayerId[ primaryLayerId ][ i ] indicates the value of nuh_layer_id of the i-th auxiliary picture layer associated with primary picture layer with nuh_layer_id equal to primaryLayerId. numAuxLayer[ primaryLayerId ] and associatedAuxLayerId[ primaryLayerId ][ i ] are derived as follows.

for( i = 0; i <= sdi_max_max_layers_minus1; i++ )
	numAuxLayer[ sdi_layer_id[ i ] ] = 0;
for( i = 0; i <= sdi_max_layers_minus1; i++ ){
	if( sdi_aux_id[ i ] == omi_aux_id_minus128 + 128 ){
		for( j = 0; j <= sdi_num_associated_primary_layers_minus1[ i ]; j++ ) {	
			primaryLayerId = sdi_layer_id[ sdi_associated_primary_layer_idx[ i ][ j ] ];
			associatedAuxLayerId[ primaryLayerId ][ numAuxLayer[ primaryLayerId ] ] = sdi_layer_id[ i ];
			numAuxLayer[ primaryLayerId ]++;
		}
	}
}

omi_mask_bounding_box_present_flag[ i ][ j ][ k ] equal to1 indicates the syntax elements omi_mask_top[ i ][ j ][ k ], omi_mask_left[ i ][ j ][ k ], omi_mask_width[ i ][ j ][ k ], and omi_mask_height[ i ][ j ][ k ], are present. omi_num_mask_in_pic_update[ i ][ j ][k] equal to 0 indicates syntax elements, omi_mask_top[ i ][ j ][ k ], omi_mask_left[ i ][ j ][ k ], omi_mask_width[ i ][ j ][ k ], and omi_mask_height[ i ][ j ][ k ], are not present.

omi_mask_id[ i ][ j ][ k ] indicates the identifier of k-th object mask in the j-th object mask auxiliary picture associated with the i-th primary picture.

The variable maskId[ i ][ j ][ k ] specifying the object mask identifier of k-th object mask in the j-th object mask auxiliary picture associated with i-th primary picture in the SEI message is derived as follows:

for( i = 0; i <= omi_num_primary_pic_layer_minus1; i++ ) {
	for( j = 0; j < numAuxLayer[omi_primary_pic_layer_id[ i ]]; j++ ) { 
		for( k = 0; k < omiNumMaskInPic[ i ][ j ]; k++ ) { 
			maskId[ i ][ j ][ k ] = omi_mask_id[ i ][ j ][ k ] + (1<<BitDepthY)*j	
		}
	}
}

omi_mask_top[ i ][ j ][ k ], omi_mask_left[ i ][ j ][ k ], omi_mask_width[ i ][ j ][ k ], and omi_mask_height[ i ][ j ][ k ] indicate the coordinates of the top-left corner and the width and height, respectively, of the bounding box of the object mask with identifier equal to omi_mask_id[ i ][ j ][ k ] in the cropped decoded picture, relative to the conformance cropping window specified by the active SPS.

The value of omi_mask_left[ i ][ j ][ k ] shall be in the range of 0 to ( CroppedWidth / SubWidthC – 1 ), inclusive, CroppedWidth and SubWidthC being associated to the the j-th object mask auxiliary picture associated with i-th primary picture. When it is not present, the value of omi_mask_left[ i ][ j ][ k ] is inferred to be 0.

The value of omi_mask_top[ i ][ j ][ k ] shall be in the range of 0 to ( CroppedHeight / SubHeightC – 1 ), inclusive, CroppedHeight  and SubHeightC  being associated to the the j-th object mask auxiliary picture associated with i-th primary picture. When it is not present, the value of omi_mask_top[ i ][ j ][ k ] is inferred to be 0.

The value of omi_mask_width[ i ][ j ][ k ] shall be in the range of 0 to ( CroppedWidth / SubWidthC − omi_mask_left[ i ][ j ][ k ] ), inclusive. When it is not present, the value of omi_mask_width [ i ][ j ][ k ] is inferred to be ( CroppedWidth / SubWidthC − omi_mask_left[ i ][ j ][ k ] ).

The value of omi_mask_height[ i ][ j ][ k ] shall be in the range of 0 to ( CroppedHeight / SubHeightC − omi_mask_top[ i ][ j ][ k ] ), inclusive. When it is not present, the value of omi_mask_height [ i ][ j ][ k ] is inferred to be ( CroppedHeight / SubWidthC − omi_mask_top[ i ][ j ][ k ] ).

The identified object mask is within a bounding box containing luma samples with horizontal coordinates from SubWidthC * ( ConfWinLeftOffset + omi_mask_left[ i ][ j ][ k ] ) to SubWidthC * ( ConfWinLeftOffset + omi_mask_left[ i ][ j ][ k ] + omi_mask_width[ i ][ j ][ k ] ) − 1, inclusive, and vertical coordinates from SubHeightC * ( ConfWinTopOffset + omi_mask_top[ i ][ j ][ k ] ) to SubHeightC * ( ConfWinTopOffset + omi_mask_top[ i ][ j ][ k ] + omi_mask_height[ i ][ j ][ k ] ) − 1, inclusive.

Variable I[ i ][ j ][ x ][ y ] is the decoded value of the sample at the relative sample location (x, y) in the j-th object mask auxiliary picture associated with the i-th primary picture. The following process is to determine each mask region in each auxiliary picture.

for( i = 0; i <= omi_num_primary_pic_layer_minus1; i++ ) {
	for( j = 0; j < numAuxLayer[omi_primary_pic_layer_id[ i ]]; j++ ) { 
		for( k = 0; k < omiNumMaskInPic[ i ][ j ]; k++ ) { 	
			if( pI[ i ][ j ][ x ][ y ] == omi_mask_id[ i ][ j ][ k ]
					&& x >= omi_mask_left[ i ][ j ][ k ] 
					&& x < omi_mask_left[ i ][ j ][ k ] + omi_mask_width[ i ][ j ][ k ]
					&& y >= omi_mask_top[ i ][ j ][ k ] 
					&& y < omi_mask_top[ i ][ j ][ k ] + omi_mask_height[ i ][ j ][ k ] )
				The sample at location (x, y) in the j-th object mask auxiliary picture associated with 
				the i-th primary picture is associated with the object mask with the identifier of
				maskId[ i ][ j ][ k ]
		}
	}
}

omi_mask_cancel[ i ][ j ][ k ] equal to 1 cancels the persistence scop of object mask with identifier equal to om_mask_id[ i ][ j ][ k ]. omi_mask_cancel[ i ][ j ][ k ] equal to 0 indicates the information of object mask with identifier equal to omi_mask_id[ i ][ j ] is signalled.

It is a requirement of bitstream conformance that when omi_mask_id[ i ][ j ][ k ] with value equal to omiMaskId is parsed for the first time in the current CLVS, the value of the corresponding omi_mask_cancel[ i ][ j ][ k ] shall be equal to 0.

The variable maskIdExist[ i ][ j ][ id ] equal to 1 indicates the object mask with identifier equal to id in the j-th object mask auxiliary picture associated with i-th primary picture exists. The variable maskIdExist[ i ][ j ][ id ] equal to 0 indicates the object mask with identifier equal to id in the j-th object mask auxiliary picture associate with i-th primary picture does not exist. maskIdExist[ i ][ j ][ id ] is initialized with 0 before decoding the current CVS.

omi_mask_confidence[ i ][ j ][  ] indicates the degree of confidence associated with the k-th object mask in the j-th object mask auxiliary picture associated with i-th primary picture, in units of 2-( omi_mask_confidence_length_minus1 + 1 ), such that a higher value of omi_mask_confidence[ i ][ j ][ k ] indicates a higher degree of confidence. The length of the omi_mask_confidence[ i ][ j ][ k ] syntax element is omi_mask_confidence_length_minus1 + 1 bits.

…

Conclusion

The contribution provides the software implementation of OMI SEI message in the current TuC draft, and on top of it, some minor fixes and changes to the OMI SEI message are also proposed. It is suggested to adopt the OMI SEI message with proposed changes into the version of VSEI draft and the software implementation into the next release of VTM.

Reference

J. Chen, S. Wang, Y. Ye, “AHG8/AHG9: On object mask auxiliary picture and object mask information SEI message for VSEI and HEVC”, JVET-AD0175, April 2023.

P. de Lagrange, E. François, D. Doyen, J. Chen, S. Wang, Y. Ye, “AHG8/AHG9: Proposed changes to the candidate new object mask information SEI message”, JVET-AE0095, July 2023

M. M. Hannuksela, J. Chen, S. Deshpande, S. McCarthy, “Technologies under consideration for future extensions of VSEI (draft 1)”, JVET-AE2032, Sept. 2023

Z. Zhang, J. Chen, Y. Ye, S. Wang, “AHG9: Experimental results of object mask auxiliary picture coding”, JVET-AF0087, Oct. 2023

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).