| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0093-v1 |
| --- | --- |

| Title: | [AHG9] On intermediary output picture(s) from activation of an NNPFGC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is asserted that the current text for neural-network post-filter group activation (NNPFGA) SEI message lacks of support for the case when there is a need to have intermediate output pictures (i.e., pictures that are output by previous filter(s) in the same NNPFGC but are not needed for final output). This contribution proposes modification with aims at enabling the NNPFGA SEI message to support such case.

The proposed modifications are as follows:

- At the end of the current syntax table of NNFPGA, signal a new syntax element called nnpfga_num_output_pic_update that specifies the number of output picture output flag in the output picture list to be updated. The value of nnpfga_num_output_pic_update shall be less than or equal to the number of pictures in NnpfgaOutputPicList.

- If nnpfga_num_output_pic_update is greater than 0, signal a loop of nnpfga_output_pic_update_flag[ i ] for i ranges from 1 to nnpfga_num_output_pic_update, inclusive.

  - nnpfga_output_pic_update_flag[ i ] is 1 specifies that the i-th picture in NnpfgaOutputPicList is included in the final output picture.

  - nnpfga_output_pic_update_flag[ i ] is 0 specifies that the i-th picture in NnpfgaOutputPicList is not included in the final output picture

- When the value of nnpfga_num_output_pic_update is less than the number of pictures in NnpfgaOutputPicList, the value of nnpfga_output_pic_update_flag[ i ] for i in the range from nnpfga_num_output_pic_update to the number of pictures in NnpfgaOutputPicList − 1, inclusive, is inferred to be equal to 1.

# Problem Statement

In the 31st JVET meeting, the first design of neural-network post-filter group characteristic (NNPFGC) and neural-network post-filter group activation (NNPFGA) SEI messages were agreed to be included in the Technologies under consideration for future extensions of VSEI (JVET-AE2032).

It is asserted that the current design lacks the capability of handling the case where there are intermediary pictures (i.e., pictures that are output by previous filter(s) in the same NNPFGC but are not needed for final output).

The following excerpt from JVET-AE0298 describes how the list of output pictures are created and populated. In summary, the list of output pictures of an NNPFG contains all pictures outputted by each NNPF member of the NNPFGC that is activated by an NNPFGA in which output pictures from each NNPF member is added into the list as long as there is no same picture already in the list.

“Let NnpfgaOutputPicList, which is the list of pictures output by NNPF process of the NNPFG in output order, be initially empty and formed in decreasing order of n in the range of 0 to nnpfga_num_filters_minus2 + 1, inclusive, by including each picture that is output by the NNPF process of the n-th loop entry that has no corresponding picture already present in NnpfgaOutputPicList.”

The above design cannot support the case when output picture(s) from an NNPF is needed only as input picture(s) for later NNPF but not for the final output pictures as illustrated in the figure below. The picture outputted by NNPF#1 is needed as input for NNPF#2 and NNPF#3 but it is not needed for the final output of the NNPFG. This is not possible in the current signaling design.

Figure 1 – Illustration of intermediary output picture that is not needed for final output.

# Proposal

We propose the following modification to provide support for the described case above:

- At the end of the current syntax table of NNFPGA, signal a new syntax element called nnpfga_num_output_pic_update that specifies the number of output picture output flag in the output picture list to be updated. The value of nnpfga_num_output_pic_update shall be less than or equal to the number of pictures in NnpfgaOutputPicList.

- If nnpfga_num_output_pic_update is greater than 0, signal a loop of nnpfga_output_pic_update_flag[ i ] for i ranges from 1 to nnpfga_num_output_pic_update, inclusive.

  - nnpfga_output_pic_update_flag[ i ] is 1 specifies that the i-th picture in NnpfgaOutputPicList is included in the final output picture.

  - nnpfga_output_pic_update_flag[ i ] is 0 specifies that the i-th picture in NnpfgaOutputPicList is not included in the final output picture

- When the value of nnpfga_num_output_pic_update is less than the number of pictures in NnpfgaOutputPicList, the value of nnpfga_output_pic_update_flag[ i ] for i in the range from nnpfga_num_output_pic_update to the number of pictures in NnpfgaOutputPicList − 1, inclusive, is inferred to be equal to 1.

# Text implementation

…

| nn_post_filter_group_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfga_target_id | ue(v) |
| nnpfga_cancel_flag | u(1) |
| if( !nnpfga_cancel_flag ) { |  |
| nnpfga_persistence_flag | u(1) |
| nnpfga_num_filters_minus2 | ue(v) |
| for( i = 0; i <= nnpfga_num_filters_minus2 + 1; i++ ) { |  |
| nnpfga_target_base_flag[ i ] | u(1) |
| nnpfga_input_all_pics_flag[ i ] | u(1) |
| if( !nnpfga_input_all_pics_flag[ i ] ) { |  |
| nnpfga_num_input_pics_minus1[ i ] | ue(v) |
| for( j = 0; j <= nnpfga_num_input_pics_minus1[ i ]; j++ ) |  |
| nnpfga_input_pic_skip_count[ i ][ j ] | ue(v) |
| } |  |
| nnpfga_num_output_entries[ i ] | ue(v) |
| for( j = 0; j < nnpfga_num_output_entries[ i ]; j++ ) |  |
| nnpfga_output_flag[ i ][ j ] | u(1) |
| } |  |
| nnpfga_num_output_pic_update | ue(v) |
| for( i = 0; i < nnpfga_num_output_pic_update; i++ ) |  |
| nnpfga_output_pic_update_flag[ i ] | u(1) |
| } |  |
| } |  |

…

nnpfga_output_flag[ i ][ j ] equal to 1 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ j ] derived for the i-th NNPF of the target NNPFG is output by the NNPF process activated by this loop entry, where the NNPF process is specified in the semantics of the NNPFC SEI message. nnpfga_output_flag[ i ][ j ] equal to 0 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ j ] derived for the i-th NNPF of the target NNPFG is not output by the NNPF process activated by this loop entry. When nnpfga_num_output_entries[ i ] is less than NumInpPicsInOutputTensor derived for the i-th NNPF of the target NNPFG, nnpfga_output_flag[ i ][ j ] is inferred to be equal to 1 for each value of i in the range of nnpfga_num_output_entries[ i ] to NumInpPicsInOutputTensor − 1, inclusive.

Let NnpfgaOutputPicList, which is the list of pictures output by NNPF process of the NNPFG in output order, be initially empty and formed in decreasing order of n in the range of 0 to nnpfga_num_filters_minus2 + 1, inclusive, by including each picture that is output by the NNPF process of the n-th loop entry that has no corresponding picture already present in NnpfgaOutputPicList.

nnpfga_num_output_pic_update specifies the number of output picture update flag that is present. When nnpfga_num_output_pic_update is greater than 0, nnpfga_output_pic_update_flag[ i ] for i in the range from 0 to nnpfga_num_output_pic_update − 1, inclusive, are present. The value of nnpfga_num_output_pic_update shall be in the range from 0 to number of pictures in NnpfgaOutputPicList, inclusive.

nnpfga_output_pic_update_flag[ i ] equal to 1 specifies that the i-th picture NnpfgaOutputPicList is included in the final output picture list. nnpfga_output_pic_update_flag[ i ] equal to 0 specifies that the i-th picture NnpfgaOutputPicList is not included in the final output picture list. When the value of nnpfga_num_output_pic_update is less than the number of pictures in NnpfgaOutputPicList, the value of nnpfga_output_pic_update_flag [ i ] for i in the range from nnpfga_num_output_pic_update to the number of pictures in NnpfgaOutputPicList − 1, inclusive, is inferred to be equal to 1.

Let numCandidateOutputPics be the number of pictures in NnpfgaOutputPicList. The list of output picture flag candOutputPicFlag[ i ] for i in the range from 0 to numCandidateOutputPics − 1, inclusive is derived as follows:

for( i = 0; i < numCandidateOutputPics; i++ )
	candOutputPicFlag[ i ] = 1
for( i = 0; i < nnpfga_num_output_pic_update; i++ ) {
	candOutputPicFlag[ i ] = nnpfga_output_pic_update_flag[ i ]

Let FinalNnpfgaOutputPicList, which is the list of pictures output by NNPF process of the NNPFG in output order, be derived as follows:

for( i = 0, j = 0; i < numCandidateOutputPics; i++ ) {
	if(candOutputPicFlag[ i ] ) {
		FinalNnpfgaOutputPicList[ j++ ] = NnpfgaOutputPicList[ i ]
	}
}

…

When applying an NNPF group to the current picture, the following applies:

–	The filtered and/or interpolated pictures are generated by applying the NNPF process specified in the semantics of the NNPFC SEI message, in a patch-wise manner, as specified in the semantics of the NNPFGA SEI message activating the NNPF group.

–	The pictures in FinalNnpfgaOutputPicList NnpfgaOutputPicList are included into ListNnpfOutputPics, in the same order as the pictures are stored in FinalNnpfgaOutputPicList NnpfgaOutputPicList.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).